This is the documentation about the GamePak reader

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

IMPORTANT NOTICE

~~~~~~~~~~~~~~~~

Please note that I've built one and it worked, I checked the plan twice to

find any errors, but I can't be held responsible for anything that might

happen:

- burning your parallel port(s)

- burning your computer(s)

- burning your GamePak cartridge(s)

- burning your cash for the ICs

- some japanese cie going bankrupt

- whatever...

-----------------------------------------------------------------------------

For any comments or suggestions, contact me via e-mail at:

yvan.rivard@inno.com

-----------------------------------------------------------------------------

Comments

~~~~~~~~

This is the 2nd release of this package. The GIF has been retouched so it's

now even easier to read. The BACKUP.EXE program included in the 1st package

was saving in chunks of 16kBytes and was waiting for [Enter]. It's now fixed

and now saves games to GAMEPAK.GB instead.

-----------------------------------------------------------------------------

Notes

~~~~~

Follow the plan of BACKUP.GIF and you shouldn't have any problems. Connect

all +5V pins and GND pins on the ICs and the GamePak connector to your power

supply pins.

In *my* backup unit, all chips are of type 74LSxxx. I don't know if it'll

work if you use anything else than 74LSxxx (especially the 74LS157 since I

grounded the Strobe pin directly).

There's a little 0.01uF capacitor between the pins +5V and GND of all my

ICs. There's also a 100uF capacitor at the power supply input pins.

input    |\         output

_________|  \/\___________

|  /\/

|/

(It's a triangle with a little round at the top) :-)

Thoses are inverters. There's 6 inverters in a 7404 chip.

-----------------------------------------------------------------------------

Parts List

~~~~~~~~~~

A lot of wires :-)

1 x GameBoy GamePak connector

1 x male DB25 connector + plastic cap

1 x male DB15 connector + plastic cap

1 x 74LS138

1 x 74LS157

1 x 74LS04

1 x 74LS244

3 x 74LS393

1 x 100uF capacitor

7 x 0.01uF capacitor

1 x board to built this thing on it

-----------------------------------------------------------------------------

How to do a +5V power supply

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Use the analog game port!

Pin  Description

1    +5V

4    GND.

-----------------------------------------------------------------------------

Description of ICs

~~~~~~~~~~~~~~~~~~

For thoses of you without any reference manual about ICs, here's the pinout

of the chips used in this backup unit.

74LS04 - Hex inverter (x6)

�����Ŀ

�1  14�         1 - input #1        14 - Vcc (+5V)

�2  13�         2 - output #1       13 - input #6

�3  12�         3 - input #2        12 - output #6

�4  11�         4 - output #2       11 - input #5

�5  10�         5 - input #3        10 - output #5

�6   9�         6 - output #3        9 - input #4

�7   8�         7 - GND (0V)         8 - output #4

�������

74LS138 - 3 lines to 8 lines decoder/multiplexer

�����Ŀ

�1  16�         1 - A               16 - Vcc (+5V)

�2  15�         2 - B               15 - Y0

�3  14�         3 - C               14 - Y1

�4  13�         4 - G2A             13 - Y2

�5  12�         5 - G2B             12 - Y3

�6  11�         6 - G1              11 - Y4

�7  10�         7 - Y7              10 - Y5

�8   9�         8 - GND (0V)         9 - Y6

�������

74LS157 - Quad 2 lines to 1 line data selector/multiplexer

with non-inverting output

�����Ŀ

�1  16�         1 - S (Select)      16 - Vcc (+5V)

�2  15�         2 - 1A              15 - G (Strobe)

�3  14�         3 - 1B              14 - 4A

�4  13�         4 - 1Y              13 - 4B

�5  12�         5 - 2A              12 - 4Y

�6  11�         6 - 2B              11 - 3A

�7  10�         7 - 2Y              10 - 3B

�8   9�         8 - GND (0V)         9 - 3Y

�������

74LS244 - Use the pinout from the GIF.

74LS393 - Dual 4-bits binary counter

�����Ŀ

�1  14�         1 - A1              14 - Vcc (+5V)

�2  13�         2 - Clear1          13 - A2

�3  12�         3 - QA1             12 - Clear2

�4  11�         4 - QB1             11 - QA2

�5  10�         5 - QC1             10 - QB2

�6   9�         6 - QD1              9 - QC2

�7   8�         7 - GND (0V)         8 - QD2

�������

-----------------------------------------------------------------------------

That's all. Once this thing is built, use BACKUP.EXE to read the games from

the gamepak to your PC. If all goes well, the backup should say "CRC ok!".

If the CRC isn't right, there's something wrong with the backup unit, your PC

or the gamepak.

BTW, I built my backup unit inside the casing of a defective GameBoy. Looks

really cool, but was hard to squeeze all theses wires inside the case. Even the

power led lights up when the unit is plugged! Try to do better! :-)

Have a nice backup!