# APPENDIX G.  Additional OPERATIONS

CONTENTS

HST Cellular

Voice/Data (External Modems Only)

Hewlett Packard 3000 Installations

MI/MIC Operations (External Modems Only)

# Cellular Operations

This function applies to Dual Standard HSTs only.  HST cellular modems can be used to answer or originate calls, but are proprietary.  They can only connect with other U.S. Robotics Dual Standard modems with HST cellular capabilities.

To enable cellular mode, load the &F3 template settings by typing the following command:

AT &F3 <Enter>

To save this as your power-on/default setting, type this:

AT &F3 &W <Enter>

The &F3 command sets the serial port rate to 19,200 bps, and formats the data for 8 bits, no parity and one stop bit.  The ini-tialization string sent to the modem by loading this template is shown below.

AT B1 X7 S10 = 30 &B1 &L2 &H1 &I0 &R2

•	B1¾enables HST modulation

•	X7—displays extended result codes including NO DIAL TONE, RINGING, NO ANSWER, and BUSY messages

•	&B1¾locks the serial port rate so that it remains higher than the connection rate.

•	S10=30¾sets the modem to wait 3 seconds after loss of carrier before hanging up.

•	&L2¾enables cellular mode

•	&H1¾enables hardware flow control

•	&I0¾disables software flow control

•	&R2¾sets the modem so it sends received data to the computer or terminal when the Ready To Send (RTS) signal is high, only if the computer or terminal supports RTS.

The modem uses the U.S. Robotics proprietary HST cellular protocol to make connections.  The call starts at 1200 bps, a V.42 error control connection is negotiated, then the modem switches to 4800 bps and begins measuring the line conditions.  Depending on line conditions, the modem decreases or increases its connection rate, to a minimum speed of 300 bps and a maximum of 12K bps.  If a connection terminates before data transfer is completed, the system tries to re-establish the link and complete the transfer.

If a large number of blocks are being resent (shown by the ARQ LED flashing randomly; some software displays the blocks retransmitted in the file transfer window), you may want to reduce the packet size by doing the following:

1.	Be sure DIP switch 9 is ON so that the modem remains online if it receives +++.

2.	If DIP switch 9 is OFF, switch it to ON and reset the modem by issuing the ATZ command.

3.	Send the modem the escape sequence by typing +++ so that the transmission is interrupted, and wait 1 second.

4.	Type AT S33=1 to reduce the packet size.

5.	Then type ATO so that the modem returns online and continues to transmit.

NOTE:  HST cellular connections can only be made with other U.S. Robotics modems in HST cellular mode.  The modem cannot be used to place a voice or data call to another phone or modem when it is in this mode.

Voice/Data (external Modems Only)

The voice/data switch is located on the modem's front panel.  While the switch's primary function is to toggle back and forth between voice and data communications, this feature also allows you to perform other functions at the touch of a switch.  Use Register S32 to select these functions, as shown in Table G.1.

Additionally, the Courier modem allows you to use the voice/data switch to set the modem for remote access if attached to certain synchronous devices.  This is described in Enabling Remote Access in Appendix F.

NOTE:  Use the voice/data switch when the modem is in Command mode (offline).  If you press the switch while the modem is online, the modem hangs up and returns a NO CARRIER result code.

## Voice/Data Operations¾S32=1, S32=2 or at commands

When you install the Courier, you have the option of plugging your phone into the second modular jack of the modem so it's available for voice calls.  You can also switch between the phone and modem during a call.

If the remote modem has handset exclusion, you can change from voice to data and back again, with or without issuing a command.

Handset exclusion means that if the modem is using the phone line, your phone (handset) is automatically disconnected.  However, if you keep the handset off hook, once you hang up the modem the handset takes over the phone line and you can use voice communications again.  If the remote user does not have handset exclusion, you may not be able to switch back to voice mode successfully once you have switched to Data mode.

The following instructions describe how to toggle voice/data communications with the switch.  This is followed by instructions for doing the same thing with AT commands.  If the remote user does not have a switch, he or she can follow the instructions for using the appropriate AT command.

Voice/Data Switch¾S32=1, S32=2

1.	Phone the other user to establish the bit rate, parity, word length and number of Stop bits the other person's modem accepts.  (Both modems should be offline, in Command mode.)

You and the other user must also agree on which modem will go off hook in Originate mode and which in Answer mode.  The Courier is factory set to go off hook in Originate mode, S32=1.  The Answer modem should have S32 set to 2.

NOTE:  If the stored command string command has been used (&ZC=s), the modem will default to S32=9, which executes the stored command string when the voice/data switch is pushed.

Either party's device can be the originate or answer modem;  it doesn't matter who made the phone call.  But one modem must first enter Originate mode and the other then enter Answer mode.

2.	Without hanging up the phone, press the voice/data switch.

If S32 is set to 2, Answer mode, press the voice/data switch immediately after the remote user forces the remote modem off hook in Originate mode.

3.	The other party should force the remote modem off hook in Answer mode by pressing the switch.

If yours is the Answer modem (S32=2), the other party should press the voice/data switch immediately before you do so.

4.	If the remote modem has handset exclusion, the remote user should also keep the phone off hook in order to switch back to voice later.  If the remote modem doesn't have handset exclusion, you can try to switch back to voice later.  Or, you both can hang up your phones as soon as the modems go off hook.

Software Commands¾ATD, ATA

If your voice/data switch is set for a function other than voice/data operations and you don't wish to change it, use AT commands.

1.	Call the other user to establish the bit rate, parity, word length and number of Stop bits the other person's modem accepts.  Determine which modem will originate and which will answer.

2.	If you are to originate the connection, type the following command:

ATD <Enter>

NOTE:  Be sure the modem is not set to X2, X4, X6 or X7, or it will return the NO DIAL TONE result code and hang up.

3.	The other party must then have the remote modem go off hook in Answer mode.  The following command is used to do this:

ATA <Enter>

4.	If the remote modem also has handset exclusion, leave both phones off hook in case you wish to switch back again to voice after your data transfer.  If the remote modem doesn't have handset exclusion, switching back to voice may not be successful.  If you don't want to switch back to voice later, hang up both phones as soon as the modems go off hook.

Allowable Voice/Data Switch Functions

Use Register S32 to set the voice/data switch for the function you desire.  The default is S32=1¾pressing the switch forces the modem off hook in Originate mode.  Pressing the switch when you power on the modem causes it to perform a power-on self-test.  See Table G.1¾S32 Functions below.

NOTE:  Some of the applications listed below require some sort of preconfiguration.  Review the Chapter/Appendix referenced before using the Voice/Data switch function.

S32 Value	Voice/Data Switch Function	Related Command

0		Disabled	_
	1		Voice/Data¾Originate Mode (Default*)	ATD (Appendix G)
	2		Voice/Data¾Answer Mode 	ATA (Appendix G)
	3		Redial Last Number	ATDL (Chapter 4)
	4		Dial Number Stored at position 0	ATDS0 (Chapter 4)
	5		Auto Answer On/Off Toggle	ATS0=0 or 1 (Chapter 4)
	6		Reset Modem	ATZ (Chapter 4)
	7		Initiate Remote Digital Loopback	AT&T6, S16=8 
						(Appendix H)
	8		Busy Out Phone Line Toggle		_
	9		Execute Stored Command (Default*)	AT&ZC=s 	(Chapter 4,
						Appendix F, G)

*	The modem is shipped from the factory set at S32=9.  If a command string has been stored (&ZC=s), the modem executes the stored command string when the voice/data switch is pushed.  If no command string is stored, the modem still reports that S32=9, but actually behaves as if it is set to S32=1, Originate mode.

Table G.1¾S32 Functions

HEWLETT PACKARD INSTALLATIONS

During error control connections, the Courier recognizes the ASCII ENQ/ACK characters exchanged between many Hewlett Packard host computers and their terminals.  The HP host sends the terminal an ENQ character at predefined intervals, and sends no more data until the terminal responds with an ACK character.

Courier modems manage this ENQ/ACK protocol so that communication is speeded up, thereby enabling HP terminals to achieve high speeds on dial-up lines.  Special flow control settings, using the &I command, are required for HP users.  These settings apply to ARQ connections only and to Courier modems set to either B0 or B1.  Disregard other Courier flow control commands.

Set the Courier to Host mode if it is attached to the host computer, or to Terminal mode if it is attached to a terminal, as follows:

Host mode	AT&I3 <Enter>

Terminal mode	AT&I4 <Enter>

MI/MIC operations (external Modems Only)

description

Mode Indicate/Mode Indicate Common (MI/MIC) closure is required by some installations whose existing hardware does the dialing.  The modems do not Auto Dial.

In these situations, the modem must be forced off hook in Originate mode.  This is done by shorting (closing) two of the pins (not Tip and Ring) in the phone connector.  The modem is then ready to go online and accept data when it connects with the number dialed by the system equipment.

Courier modems are shipped with MI/MIC disabled, that is, for normal use.  To set the modems for MI/MIC closure, you must do two things:  physically set jumpers on the modem’s printed circuit board, and enable bit 5 of Register S34 (ATS34=32 or ATS34.5=1).  We recommend you then write that setting to NVRAM as a power-on default.

Once you've set Register S34, have the system force the modem off hook by closing the MI/MIC leads in the phone line connector.  The modem's OH (Off Hook) status light, or LED, goes on when the modem goes off hook.

## Setting the MI/MIC Jumpers

You'll have to dismantle the modem case to set the two jumper switches on the printed circuit board, as follows:

1.	Power off the modem and disconnect all of its cables.

2.	Turn the modem upside down.  Remove the two square vinyl feet near the back of the case, on either side of the bottom label's DIP switch diagram.  Be careful to put the vinyl feet aside, upside down, where they won't become stuck to another object.

3.	Remove the two Phillips screws located in the wells beneath the vinyl feet.

4.	Gently pry off the plastic volume slide-switch cover.

5.	Raise the back end of the case bottom until it is at about a 60° angle; lift it away from inside the front of the modem.  Put the case bottom aside.

6.	Locate the voice/data switch at the front of the modem.  Lift up and remove the modem (printed circuit board), carefully easing the voice/data switch out of its opening in the front panel.

7.	Turn the modem rightside up and locate jumper switches J4 and J6, near the power-adapter socket.  Use Figure G-1 as a guide.

§

Figure G-1¾Courier Modem Board

8.	The jumpers are black shunts that cover two out of three upright metal contacts.  As shown in the figure, set the jumpers so they cover the two contacts on each switch that are closest to the front panel.  The third contact on each switch is exposed.

9.	Replace the modem in the case top:  ease the voice/data switch into the opening in the front panel and make sure the back of the board rests on the locator pins at the rear (from which you removed the screws).

10.	Connect the modem's RS-232, power and phone cables, in that order (see chapter 2).  Be sure the phone cable is plugged into the jack closest to the center of the modem¾the jack represented by the wall jack icon on the case bottom label.

CAUTION:  When you power on the modem there will be potentially hazardous voltage, particularly near the phone jacks.  Do not touch the board when the power is on.

11.	Power on the modem.  Try MI/MIC closure.  Check to see that the Off Hook (OH) status light goes on.  If you dialed a number, listen for an answer tone from the remote modem.  Then drop the DTR signal.  The modem should go on hook and the OH status light should go off.

12.	When you are sure the equipment is working correctly, disconnect the modem’s RS-232, power and phone cables, and replace the bottom of the modem case.  Ease the two nibs near the front corners into their openings in the front of the case top, guide the rectangular slot over the volume switch, and ease the back of the case bottom into place.  Replace the two screws, the two vinyl feet, and the volume slide-switch cover.

## troubleshooting

You may find that the modem does not respond to MI/MIC closure, which you can monitor by observing the LED.  Or the modem may fail to go back on hook when the computer or terminal drops the Data Terminal Ready (DTR) signal.  The probable reason for either of these conditions is that your phone equipment reverses MI/MIC polarity.

It's possible to solve this problem by reversing the modem's MI/MIC wiring.  You'll have to dismantle the modem case again and reset the two jumper switches on the printed circuit board, as follows:

1.	Lift off the jumpers from J4 and J6.  Reverse the positions shown in Figure G-1.  That is, on each switch, cover the center and rightmost contacts.  Make sure the jumpers cover two contacts on each switch, or you'll disable the MI/MIC function.

2.	Replace the modem in the case top:  ease the voice/data switch into the opening in the front panel and make sure the back of the board rests on the locator pins at the rear (from which you removed the screws).

3.	Reconnect the modem's RS-232, power and phone cables, in that order.  Be sure the phone cable is plugged into the jack closest to the center of the modem¾the jack represented by the wall jack icon on the case bottom label.

CAUTION:  When you power on the modem there will be potentially hazardous voltage, particularly near the phone jacks.  Do not touch the board when the power is on.

4.	Power on the modem.  Try MI/MIC closure again.  Check to see that the Off Hook (OH) status light goes on.  If you dialed a number, listen for an answer tone from the remote modem.  Then drop the DTR signal.  The modem should go on hook and the OH status light should go off.

5.	If closure is not working properly, review the steps in this section.  Be sure the jumper switches are in the reverse position of those in Figure G-1.  If you still have problems, there may be a problem with the phone cable.  Or there may be a problem with your hardware.

6.	When the equipment is working correctly, disconnect the modem’s RS-232, power and phone cables, and replace the bottom of the modem case.  Ease the two nibs near the front corners into their openings in the front of the case top, guide the rectangular slot over the volume switch, and ease the back of the case bottom into place.  Replace the two screws, the two vinyl feet, and the volume slide-switch cover.