# APPENDIX D.  Dial Security and 
remote access operations

Dial Security is designed to protect networks and data centers from unauthorized access.  The kind of security provided by the Courier modem is different from other kinds of dial security where software is used by the computer to control user access.  The Courier’s Dial Security is set up in the modem’s firmware, so access is controlled from modem to modem.

Setting up this kind of security requires action at both the host and remote modem sites.  This appendix contains instructions for both operations.

WARNINGS:

If you want to set up Security on your Courier modem, you must complete the steps below in the order listed:

1.	Set up your own security account:
	·	Set up your local access password
	·	Enable local access password protection
2.	Set up remote user accounts.

3.	Enable Dial Security.

You must set up your local access security information before enabling Dial Security and allowing remote calls, as outlined under Set Up Your Security Account in what follows.

Be sure to remember your local access password; if you enable security and forget your local access password, you risk being locked out of your system.

## Overview

The Courier’s Dial Security allows you to configure up to 10 Security accounts.  Each security account is stored in nonvolatile memory (NVRAM) and may be set up in one of three ways:

·	Automatic password access

·	Prompted password access

·	Password with Dialback protection

### Autopass

Autopass is the term we use for the basic implementation of Dial Security in the Courier modem.  It is in effect whenever Dial Security is enabled.

To use autopass alone, the following requirements must be met:

•	Both host and remote modems must be U.S. Robotics modems with Dial Security enabled.

•	The connection must be under V.42 error control (both modems must be set to &M4 or &M5).

When the remote modem is set for Dial Security, it automa-
tically includes an autopass password (configured by the remote user to be compatible with the host modem’s security) in its V.42 error control request.  The host modem checks all the enabled passwords in its security accounts for a match.

If the remote user has set up the modem with an invalid password, the host modem returns an INVALID PASSWORD message and hangs up.

If the remote user failed to enable Dial Security on the remote Courier, the host Courier will not accept the call unless prompting (below) is enabled on the host modem.

### Prompting

While the use of autopass (above) is restricted to U.S. Robotics Dial Security modems, prompting allows secured connections with any remote modem whose user has been assigned an allowed password.  When prompting is enabled, and the host modem doesn’t receive an autopass password, it prompts the remote user for a password.  The host modem checks the received password against each of its active Security accounts.

If the password is valid, a secure connection is completed.

If the password is invalid, the host modem prompts twice more before disconnecting.

If there are no password attempts at the remote end, the host modem times out after 60 seconds and disconnects.

NOTE:  There is no prompting capability in synchronous mode.

Prompting is more flexible than autopass because it:

•	doesn't require a remote modem to have U.S. Robotics Dial Security.

•	doesn't require a V.42 connection, so remote modems without V.42 error control capabilities can connect.

### Optional Dialback

Dialback offers an additional layer of security.  When the host modem receives a valid password that matches an account and Dialback is enabled, the host first disconnects.  The remote user receives a NO CARRIER message.  Then the host modem dials back the remote modem.

Typically, the Dialback number is stored in the Security account’s phone number field.  However, if the security account has been set up to allow a new number, the host prompts the remote user for the new number before hanging up, and then dials back.

In order to use dialback, Dial security and prompting must be enabled.

WHAT THE HOST MODEM OPERATOR NEEDS TO DO

Complete the steps below in the order listed:

1.	Set up your own Security account.

2.	Set up Security accounts for your remote users.

3.	Enable Dial Security.

4.	Maintain Security accounts.

## Set Up YOUR SECURITY Account

Security accounts are set up via an AT command line structure, rather than a screen format.

1.	Set up your Security account using one of 10 accounts (numbered 0-9).  Each account has five possible fields, as shown in the following table.

Password	Account	Dialback	Allow New		Dialback
Enabled	Enabled	Enabled	Number		Number

8 chars. max.	YES/NO	YES/NO	YES/NO		up to 37
ASCII 32-127						characters 
case sensitive*						(0-9)

BILL	Y	Y	N	1-419-555-5555

*  If “BILL” is the password, “Bill” is an invalid entry.

Use the %An= command and format (below) to set up accounts, where n is the account number, 0-9.  The fields in the above table are entered after the equal sign, each separated by a comma, as in the following example for account 0 with the password BILL.

WARNING:  Do not insert spaces between commas or between fields and commas.  Spaces will invalidate the command.

AT%A0=BILL,Y,Y,N,1-419-555-5555 <Enter>

This example instructs the modem to store the configuration for  account 0:  password (BILL), account enabled (Y), dialback enabled (Y), allow a new number (N), and the dialback phone number.

NOTE:  The %An= command is automatically written to NVRAM and does not require an &W.

Some accounts may have fewer security options.  The following example sets the password (Judy) and enables account 1 without any dialback options.

AT%A1=Judy,Y,,, <Enter>

Each security function can be configured or modified independently.  If a field is to remain as is, just insert a comma, as shown in the following command.  It allows the remote user to supply a new Dialback number different from the one stored in the original account record.

AT%A0=,,,Y, <Enter>

2.	Set up your local access password.  This password protects the Security accounts from unauthorized users.  When this protection is enabled, you cannot view, modify, or tamper with the Security accounts unless you enter the correct local access password.  The local access password must be chosen and protection must be enabled before Dial Security is enabled.

In the following example, the local access password is the same as the password in account 0.

AT%L=PW0 <Enter>

## Set up Accounts for Remote users

Once your account is configured and password protection has been enabled, you can set up the remote user accounts.  Use the %An= command (using the same guidelines you used to set up the system administrator’s account in Step 1 of the previous section) to set up remote user accounts.

Once security accounts have been enabled, you are responsible for communicating valid password information to your remote users.

## Enable Dial Security

Once you have completed the previous steps, you are ready to enable Dial Security.

When you enable Dial Security, you must choose either autopass or prompting as the security method.  You must know what types of modems remote users are using and set the Dial Security parameters accordingly.  Autopass is limited to U.S. Robotics modems with Dial Security, but prompting is not.

1.	Register S53 is the bit-mapped register used to enable Dial Security.  The &W command must be used to save Register S53 settings to NVRAM.  Otherwise, when the computer is powered off and on again, or the modem is reset using ATZ, it will default to S53=0, Dial Security disabled.

To enable Dial Security with autopass and local access password protection, but without prompting, type the following command:

AT S53.0=1 .2=1 &W <Enter>

The following command enables Dial Security with autopass, prompting, and local access password protection.

AT S53.0=1 .1=1 .2=1 &W <Enter>

NOTE:  Enabling local access password protection disables the &Zn=s command that stores up to ten phone numbers.  Stored phone numbers occupy the same space in NVRAM as the dialback numbers for Dial Security accounts, and cannot be used when Dial Security is enabled.

If the local access password is not protected, the &Zn=s command overwrites the corresponding Dial Security dialback number.  For example, the following command overwrites the dialback number for account 5:

AT &Z5 = 555-8976 <Enter>

However, if the local access password is protected, and a user tries to use the &Zn=s command, an ACCESS DENIED message is displayed.

WARNING:  Be sure DIP switch 10 is OFF so the modem will load the settings stored in NVRAM.  If someone sets DIP switch 10 ON, the low performance template settings (&F0) are loaded, and Dial Security is disabled.  If this should happen, reset DIP switch 10 to OFF, power off the modem and power it back on, or reset the modem by typing ATZ <Enter> so that the proper settings take effect.

## Maintain security ACCOUNTs

Once the local access password is set and protected, the system administrator is the only one who can access account information.  To modify or change account information, use the %S= and %E= commands described next.

### Account Access (%S)

Once Dial Security is enabled, you can access accounts by entering the local access password using the %S= command, which allows access to the accounts by disabling local security.

AT%S=(your local access password) <Enter>

NOTE:  The modem echoes the local access password, which is case sensitive.  The system will accept an invalid password entry, but will lock you out from the modem’s security commands.  For example, if the password is Bob, but you enter BOB, an OK is displayed.  However, if you try to type a security command (for example, ATI10 <Enter> to view accounts), an ACCESS DENIED message is displayed.

### Account Status

Once access has been granted, you can view account information by typing the I10 option of the Inquiry (I) command:

ATI10 <Enter>

Remote users may only use this command during a remote access session if local access security is disabled.

### Erasing Account Information (%E)

Use the %E=n command to make system edits.

%E=1	Erase local access password.

%E=2	Erase autopass password.

%E=3	Erase passwords in accounts 0–9.

%E=4	Erase phone numbers in accounts 0-9.

%E=5	Disable Account, Dialback, and New Number
		fields in accounts 0-9.

To edit or overwrite an individual account, or an individual account field, use the %An= command described in Set Up Your Security Account, earlier in this appendix.

### Remote Configuration

Dial Security accounts may be configured remotely.  See Configuring Dial Security Remotely at the end of this appendix.

What the Remote Caller Needs to do

When remote users want to call in to your Courier (assuming you have enabled Dial Security), they must contact you to obtain a valid password.  They must also find out if they must set the remote modem for auto answer (necessary if your modem uses Dialback as a security method).

1.	If the host modem security is set up, get a password from the host modem operator.  Passwords are case-sensitive, so be sure to copy it down correctly.

If the host modem security is set up for prompting and the host operator enables dialback for your account, skip to Step 3.

2.	For remote users with COURIER modems only.  Set up security on your modem, including an account that uses the password the host asked you to use.  Refer to Set up Your Security Account, earlier in this appendix, for instructions.

Then assign the password as your autopass password.  Enter the following command, where n is the number of the account you set up):

AT %V=PWn <Enter>

You can check to see that you've correctly set up your autopass password by typing the following command:

AT I10 <Enter>

Your autopass password appears in the right-hand column below FORCED AUTOPASS.

Once the autopass password is set, enable your Courier modem’s Dial Security with the following command:

ATS53.0=1 &W <Enter>

3.	If Dialback is enabled at the host Courier site, set your modem to answer the host modem when it disconnects and dials back.

To set the modem to answer the Dialback call, set DIP switch 5 OFF and reset the modem (ATZ <Enter>).

Alternatively, type the following command:

AT S0=1 <Enter>

4.	Call the host modem.

5.	When the call is completed, if you want to disable auto answer, do one of the following:

·	Set DIP switch 5 ON and reset the modem (ATZ<Enter>).

·	Type the following command:

AT S0=0 <Enter>

Remote Access Operations

You can set a Courier modem so that someone at a remote location can configure your modem.

This might be helpful if you have problems making a connection with another modem.  For example, if you have trouble connecting with a bulletin board, you can allow the bulletin board operator to dial in to your modem and view its configuration settings.  If necessary, the bulletin board operator can send the Courier a configuration string that will make it compatible with the bulletin board.

Another use might be for Dial Security administration when the system administrator is unable to be at the host modem site.  It might be urgent, for example, to disable an account.

## At the Host Courier Site

There are two tasks required to set up remote access:

1.	Set up password security.

2.	Enable remote access.

Set Up Password Security

You can designate two passwords for remote access security, each allowing a different level of access to the remote user.  You can assign one password that allows viewing privileges only, whereby the host Courier's configuration can be remotely viewed but not changed.  You can assign another password that allows both remote viewing and configuration privileges.

Remote access passwords can be up to eight alphanumeric characters long, and are not case-sensitive.

%Pn	Use the %Pn command to assign remote privileges.

%P0	Viewing privileges only

%P1	Viewing and configuration privileges

To assign a password that allows viewing privileges only, use the command format below:

AT%P0=[password] <Enter>

To assign a password that allows viewing and configuration privileges, use the command format below:

AT%P1=[password] <Enter>

### Disabling Password Security

If you want to disable an assigned password (and thereby disable remote access security), use the following command format:

AT%P0= <Enter>

or

AT%P1= <Enter>

WARNING:  If you disable the %P1 password, a remote user does not need to enter a password for configuration access.

Enable Remote Access

Set Register S41 for a value of 1 or greater.  S41 is used to set the number of allowable login attempts, as explained later.  A setting of zero allows no login attempts, and thus disables remote access.

AT S41=1 <Enter>

NOTE:  This method will not work if your modem is attached to certain synchronous devices.  Refer to Appendix F for more information.

### LED Indicator for Remote Access

The Courier's SYN status light indicator (LED) flashes to indicate when it is in a remote access session.

## At the Remote Modem Site

Remote configuration can be performed at any time during an asynchronous connection.  The user performing remote configuration can use any modem; it does not have to be a U.S. Robotics model.

1.	Make sure the host Courier has been set for remote configuration, as described earlier.  Then establish a connection.  It does not matter which modem originates the call.

2.	After a connection has been established, send the following escape sequence:

Pause four seconds,

type four tildes: ~~~~

and

pause another four seconds.

NOTE:  You can change the escape sequence character with Register S42.  The pause duration (guard time) can be modified with Register S43.  (These values are set at the host Courier modem.)

3.	When the Courier begins its login sequence, the caller will see a display similar to the following:

U.S Robotics Courier HST Dual Standard V.34 Fax Remote Session

Serial Number 000000A000000001

4 .	At this point, if password security is active, the caller is prompted for the password.

Password (Ctrl-C to cancel)?.....

As described earlier in Password Security, entering the password assigned by the %P0 command allows viewing privileges only.  Entering the password assigned by the %P1 command allows viewing and configuration privileges.  Note that there is a 3-minute time limit for entering the password.

As mentioned earlier, if the number of unsuccessful login attempts exceeds the set limit, the modem returns online and refuses any further login attempts during the remainder of that connection.

When a password is accepted, the Courier indicates that it has entered Remote Access mode and the remote prompt appears on the remote caller's screen.

Access Granted

Remote->

5.	If password security is not active (no passwords have been set or both passwords are disabled), the Courier automatically enters Remote Access mode and the remote prompt appears on the remote caller's screen.

Remote->

6.	Once the remote access session has been established, keep in mind that there is a 3 minute inactivity timer.  If the modem detects no activity for 3 minutes, it aborts the remote access session and resumes a normal online connection.

### Aborting the Request for Remote Access

If you want to abort the remote access login before you have entered the password, return online by pressing <Ctrl>-C or typing ATO <Enter>.

Remote Viewing and Configuration

Once you've gained remote access, you can communicate with the host Courier as if you are entering commands at its attached computer.  Depending on your access privileges, you can use the regular Courier AT commands.

If you have view privileges only (with %P0), you can use any of the view (Inquiry) commands described in Chapter 6.

If you have view and configure privileges (set with %P1), you can use any of the modem commands, except those commands that cannot be used while online, such as the Dial command.  You can also use the remote configuration commands explained later.

When you make remote configuration changes, the remote prompt is altered to indicate that changes have been made.  The prompt will change from:

Remote->

to

Remote+>

If you restore the original configuration (with %C1, explained next), the original prompt is also restored (back to Remote->), indicating that the original configuration is intact.

By default, configuration changes do not take effect until the connection is terminated (see %Cn).  However, the new configuration is immediately reflected on the information screens (ATIn).

Remote Configuration Commands

There are some additional commands that are only executable during a remote access session.  These commands are as follows.

%Bn	Configure the Courier's serial port rate.

%B0	110 bps			%B6	9600 bps

%B1	300 bps			%B7	19,200 bps

%B2	600 bps			%B8	38,400 bps

%B3	1200 bps		%B9	57,600 bps

%B4	2400 bps		%B10	115,200 bps

%B5	4800 bps

%Cn	Configuration control.

%C0	Defer configuration.   This is the default.  Configu-
	ration changes are deferred until the call is ended; 
	they take effect for ensuing connections.  You do 
	not need to enter this command; it is the default 
	unless you enter %C1 or %C2.

%C1	Restore configuration.  Use this command to 
	cancel any configuration changes made during 
	remote access, and restore the original 
	configuration.  However, commands that have 
	been written to NVRAM (with &W) will not be 
	restored to their previous settings.  Additionally, if 
	you have forced immediate configuration changes 
	(with %C2), those changes cannot be reversed with 
	%C1.

%C2	Execute configuration.  Use this command to force 
	configuration changes to take effect immediately, 
	during the current connection.  We recommend 
	against forcing immediate configuration changes 
	unless absolutely necessary, as this can result in an 
	unreliable connection or even a loss of connection.

%Fn	Configure data format.

%F0	No parity, 8 data bits.

%F1	Mark parity, 7 data bits.

%F2	Odd parity, 7 data bits.

%F3	Even parity, 7 data bits.

### Password Commands

%Pn=		Disables password security (n=0 or 1) when no 	
	character follows the equal sign.

%Pn=s	Specify the following password string (s) for viewing 
	privileges only (n = 0) or view and configuration
	privileges (n= 1).

%Pn?		Display password n.

### Command Format

When typing commands during the remote access session, no delay between command strings is necessary.  For example, you can type the following commands without pausing after each one:

•	a password:  ABCDEF <Enter>

•	a configuration string: AT&H1&R2&W <Enter>

•	and a request for an information screen: ATI5 <Enter>

The maximum number of characters between carriage returns is 40.

Ending a Remote Access Session

One of four commands ends a remote access session.

•	<Ctrl>-C aborts the login procedure.

•	ATZ resets the modem and terminates the connection.

•	ATH terminates the connection.

•	ATO or ends the remote access session, but the modems remain online.

## Configuring Dial Security remotely

The system administrator can configure host Courier security account information with a Courier modem at a remote site using the procedures described below.

NOTE:  Before remote configuration is possible, the local modem's remote access must be enabled, and a remote access password that allows viewing and configuration privileges must be assigned.  For convenience, you may want to use your local access password as your remote access password.

### Dialing In From the Remote Site

1.	From the remote site, connect to the host modem using Dial Security.  Once a connection is made, follow the instructions for beginning a remote access session as described earlier in this appendix.

2.	When remote access has been granted, use the %S= command to access the Dial Security accounts.

3.	If you wish, you can view account information by typing the following Inquiry (I) command:

AT I10 <Enter>

4.	Make any configuration changes and execute them immediately by typing the following command:

AT %C2 <Enter>

5.	To end the remote session and reactivate local access security on the host modem, reset the modem by typing:

ATZ <Enter>

WARNING:  If you do not use the ATZ command to end a remote access session, local access security will remain disabled at the host Courier site and anyone dialing in to your modem for remote access will have access to the I10 screen and all Security accounts.