# Chapter 4.  Data Mode Operations

The information in this chapter applies to asynchronous calls only.  For synchronous operations, refer to Appendix F.

Detailed command descriptions are in this chapter.  Additional command summaries are in Appendix C, on the bottom panel of the modem, and in the Quick-Reference Card.

## COMMAND SET USAGE

The Courier command set enables you to send the modem two kinds of instructions:

•	operations, such as dialing or hanging up

•	configurations, such as enabling error control or data compression

Follow these guidelines:

1.	Your software must be loaded and, if you are using a computer, it must be in Terminal mode.

Some communications programs put the computer in terminal mode automatically when they are loaded.  Others require you to display a communications terminal screen, press a Function key, or perform some other operation.  Refer to your communications software documentation for instructions.

In Terminal mode the computer acts as if it were a standard terminal such as a teletypewriter, rather than a data processor.  Keyboard entries go directly to the modem, whether the entry is a modem command or data to be transmitted over the phone lines.  Received data is output directly to the screen.

2.	Type commands in either upper or lower case, not a combination (AT or at—not At).

3.	All commands except A/, A> and +++ are preceded by the AT (attention) prefix and are executed with the Enter/Carriage Return key (<Enter>).

4.	Command length = 60 characters maximum.  The modem doesn't count the AT prefix, Carriage Return character, or spaces.  It counts (but doesn't act on) punctuation such as hyphens and parentheses.

5.	A missing numeric parameter is assumed to be zero, as in the command to hang up:  ATH <Enter> is the equivalent of ATH0 <Enter>.

Example (spaces are not required, but are added here for readability):

AT &K3 X2 DT 071 312 1234 <Enter>

AT	Attention; a command follows.

&K3	Disable MNP5 data compression; use only V.42 bis compression.

X2	Use the X2 result code subset.

DT	Dial the following number using tone dialing.

<Enter>	Execute the commands.

This chapter groups related commands into the following categories.

•	Basic Commands

•	Dialing/Answering

Dialing

Dial Options

Cancel Dialing

Store Phone Numbers

Redialing

Answer Mode

Auto Answer

Hanging Up

•	Setting/Using Defaults

Customizing NVRAM

Resetting the Modem

•	Configuration:

Echo/Speaker

Result Codes

Modulation

Error Control/Data Compression

Data Rates

RS-232 Signal Operations

Flow Control

•	S-Registers

•	Inquiry and Help

•	Testing

•	International Calls

•	Miscellaneous Commands

For an alphabetical listing of commands, check the first page of the index.

NOTE:  The defaults listed are based on the modem's shipping configuration:  load from nonvolatile random access memory (NVRAM), DIP switch 10 OFF, which is the same as the &F1 configuration template).  For a complete listing of default configuration templates, see Appendix B.

BASIC COMMANDS

AT	Attention command prefix.  Use AT alone to test for the OK result code.  AT must prefix all commands except A/, A> and +++.

Any	Terminate the current dialing operation resulting from an

key	issued Dial command; terminate Repeat mode (> or A>).

DIALING/ANSWERING

Dialing

Dn	Dial the specified phone number; also execute Dial options.

The maximum number of characters allowed is 60, including the AT prefix, punctuation and spaces.  The Carriage Return (Enter key) isn't counted as a character.

NOTE:  With the exception of the following Dial options, the modem ignores any commands issued after D in the same command string.

Dial Options

D	Dial the number that follows and enter Originate mode.  Optional parameters:

P 	Pulse dial.  Default.

T	Tone dial.

,	(Comma) Pause for 2 seconds before continuing to dial.

;	Return to Command mode after dialing.  If your phone is plugged into the modem, you can use this option to have the modem Auto Dial a telephone rather than a modem.  The Courier dials, remains off hook and returns the OK message, indicating it is in Command mode.

For example, to have the modem place a voice call, enter the Dial command with a semicolon:

ATDT5551234; <Enter>

When the modem returns the OK result, pick up your phone receiver so you can talk to the other party, and send the command that hangs up the modem:

ATH <Enter>

" 	Dial the letters that follow (in an alphabetical phone number).  NOTE:  If you are including another command after the dial string, use closing quotation marks before the additional command.

!	Transfer a call (flash the switch-hook).  This command applies to modems in installations where other modems share the phone line.  The modem flashes the switch-hook (goes off hook 0.5 seconds, on hook for 0.5 seconds and off hook again) to dial the specified extension.

W	This command is useful in situations where you must wait for a second dial tone before continuing dialing.  For example, if you need to dial for an outside line, the Courier continues dialing as soon as it detects the next dial tone.

AT DT 9 W 5551234 <Enter>

NOTE:  This command executes only if result code option X3 or greater has been issued.  If the modem is set to X2 or lower, the modem interprets the W as a comma (two-second pause).

@	Wait for an answer (with X3 or higher).  Some online services answer the phone and return a tape-recorded request for information before processing transactions.  In such instances, the @ command can be used in the Dial string to tell the modem to detect at least one ring, wait for five seconds of silence at the other end of the call, and then continue to execute the Dial string.

To use the @ command, set the modem to X3, X4 or X7.  If the modem is set to X2 or lower, the modem returns an ERROR message when encountering the @ character in a command string.  If set to X5 or X6, the modem hangs up when it detects a voice answer and sends the VOICE result code.

/	A slash (/) causes a pause of only 125 milliseconds.

R	Reverse frequencies.  This command allows calls to an originate-only modem.  It reverses the modem's originate/answer frequencies, forcing the Courier to dial out at the answer frequency.  The command follows the Dial command, before or after the phone number:

AT D1234567R <Enter>

X2-X7	Adaptive dialing.  When any of the X2 through X7 options is in effect and you do not issue a dialing type in the Dial string, the Courier uses tone dialing, which is faster than the default pulse type.  However, if the phone company's central office does not have tone detection equipment, the modem cannot break dial and continues to detect the dial tone.  If this occurs, the modem automatically reverts to pulse dialing.

DL	Dial the last-dialed number.  The modem stores each Dial command until it receives the next Dial command.  Use DL instead of A/, described on the next page, if you wish to send the modem non-Dial commands before dialing again.

DSn	Dial the number stored in nonvolatile random access memory at position n, where n = 0-9.

Cancel Dialing

To cancel Dial-command execution, press any key. If you inadvertently hit a key on the keyboard while the modem is dialing, the call is canceled.  If this occurs, type the A/ command explained under Automated Redialing below.

When the modem receives a command, it stores the instruction in its command buffer until it receives the next AT command.  Note that if you've sent the modem an additional command since the Dial command, A/ re-executes that command instead of redialing.

Store Phone Numbers

&Zn=s	This command stores up to ten numbers, where n is the position 0-9 in nonvolatile memory, and s is the phone number string.  The number-string may be up to 40 characters long, including any Dial command options.

AT &Z2=555-6789 <Enter>

Do not include modem settings in the &Zn string.  If the call requires a special setting, insert it in the command string before the DSn command.  In the following example, &M0 (no error control) is inserted before the Dial command:

AT&M0 DS2 <Enter>

NOTE:  The &Zn=s command functions differently when Dial Security is enabled.  See Appendix D for more information.

&Zn?	Display the phone number stored in NVRAM at position n 
(n = 0-9).

Redialing

A/	Re-execute the last issued command.  A/ doesn't take the AT prefix or a Carriage Return, and can be used to redial.

A/

Automated Redialing (>, A>)

While > and A> can be used to continuously repeat any command, they are designed for automated redialing.

### Enter Repeat Mode

>	If you know the modem you are calling is frequently busy, include the Repeat command in the Dial string, as follows:

AT > DT 1234567 <Enter>  or
AT DT 1234567 > <Enter>

The modem enters Repeat mode, dials the number, waits 60 seconds for a carrier (default), and hangs up.  Then after a two-second pause, it redials.

The cycle continues until the modems connect or the modem reaches a maximum of 10 attempts.  The 10-try limit is mandated by Industry Canada (IC) to prevent tying up local telephone company exchanges with unconnected calls.

A>	This command combines the features of both the A/ and > commands.  The modem enters Repeat mode as described above, and redials the Dial string in the command buffer.  Like the A/ command, A> does not take the AT prefix or a Carriage Return.

### Exit Repeat Mode

Should you use > or A> with a command other than a Dial string, abort the cycle by pressing any key.

To abort automated redialing, be sure to press any key when the result code appears, during the pause before the modem begins dialing again.  If you press any key while the modem is dialing, that dial attempt is canceled but the cycle continues.

Answer Mode

Force Answer Mode

A	Force Answer mode when the modem hasn't received an incoming call.

Auto Answer

The Courier is shipped with DIP switch 5 ON, Auto Answer suppressed.  To set the modem to automatically answer incoming calls, do one of the following:

1.	Before powering on the modem, set DIP switch 5 OFF.  When you turn the computer on, the modem answers incoming calls on the first ring.

2.	When the modem is on, set your communications software to enable auto answer.  The following command instructs the modem to answer on the first ring.  (You can substitute a higher value.  See the S-Register summary in Appendix B.)

AT S0 = 1 <Enter>

When the modem senses a call coming in, it sends the result code RING to your screen, goes off hook, and sends the remote modem a high-pitched answer tone.  If there is no Carrier Detect within 60 seconds, the modem hangs up.  If the connection is made, the modem sends a CONNECT result code.  When the call is disconnected by you or the remote user, the modem hangs up and returns the NO CARRIER code.

NOTE:  If DIP switch 5 is OFF and S0=0, the Auto Answer will be disabled.  Be sure that S0=1-256.

Suppressing Auto Answer

To disable Auto Answer, reverse Steps 1 or 2 above.  Set DIP switch 5 ON before powering on the modem, or set the modem to answer on zero rings with the following command.

AT S0 = 0 <Enter>

Points to Remember

1.	If the modem is attached to a computer, you can set the modem to receive calls when you're not at your computer.  Load your communications software as you normally do, and set the modem to Auto Answer.  Also set your software's host mode function to save incoming messages and/or files.

2.	If you've attached your phone so it can be used for conventional calls, disable Auto Answer when you are not expecting incoming data calls.  Otherwise, your modem may answer the phone before you do, greeting a voice caller with a high-pitched answer tone.

Hanging Up

Hn	On/off hook control.

H0	Hang up (go on hook).

H1	Go off hook.

+++	Escape code operations.  Once the modem is online to another system, the only command it recognizes is an escape code of three typed pluses, which forces the modem back to Command mode.  Do the following when issuing the command:

•	Wait one second after sending the last item of data

•	Type: +++

•	Wait one second before typing any data

Do not type the AT prefix or a Carriage Return.  The guard time of one second before and after the code prevents the modem from misinterpreting the occurrence of +++ in the transmitted data stream.

If necessary, the character used in the escape code or the duration of the guard time can be changed by resetting Register S2 or S12.  See the S-Register Summary in Appendix B.

In response to +++, the modem returns to Command mode.  However, it keeps the line open or hangs up, depending on the setting of DIP switch 9:

DIP Switch 9	Response to +++

OFF	Modem goes on hook (hangs up), sends NO CARRIER result code (factory setting)

ON	Modem maintains connection (Online-Command mode), sends OK result code

The factory setting (OFF) forces an automatic disconnect when you issue +++.   One advantage of this is that you are not likely to inadvertently run up an all-night phone bill.

Set DIP switch 9 ON if you want the modem to respond to +++ by entering Online-Command mode, enabling it to execute commands and return online.  (See the O command, next.)

WARNING:  For unattended modem operations:  in rare instances, the modem may fail to recognize the +++ escape code sequence.  If you are running the modem under software control for unattended operations, we suggest you use the sure fire method of dropping the DTR signal from the computer or terminal for at least 50 milliseconds, to avoid costly phone charges.  Methods of turning the DTR signal off¾for example, closing the communications port¾differ from one computer to another.

Returning Online

On	If DIP switch 9 is ON (on detection of the escape code the modem maintains the connection), you can issue commands and then toggle the modem back online with the On command, as in this example:

AT Q1 O <Enter>

There are two ways to return online.

ATO0	Return online (normal).  (Used in the example above.)

ATO1	Return online and retrain.  Use to have the modem re-synchronize if there were errors in a non-ARQ data transfer.

Hanging Up

If DIP switch 9 is ON, the escape code forces the modem back to Command mode but leaves the line open.  If you want the modem to hang up, issue the following command once the modem sends the OK result code:

ATH <Enter>

If DIP switch 9 is OFF, the modem automatically hangs up on receipt of the escape code.

SETTING/USING DEFAULTS

The modem's read-only memory (ROM) permanently stores the modem's four factory template settings.  Nonvolatile random access memory (NVRAM) allows you to save one of these four templates, or add your own modifications, and write all the settings to NVRAM as your power-on defaults.

&Fn	The modem is shipped with four configurations (templates), 
&F0-&F3, stored in permanent nonprogrammable memory (ROM).  Appendix B includes configuration listings for each template.  Any one of the templates may be loaded into current memory (AT &Fn) or written to nonvolatile memory to serve as the reset default (AT &Fn &W).  Note, however, that &F0 is always loaded into memory if DIP switch 10 is ON.

When you power on the Courier, it loads the settings stored in NVRAM if DIP switch 10 is OFF.  Until you write your own settings to NVRAM, the defaults stored there are the same as the permanent ROM factory settings stored in position 1, &F1.

To view the &F1 settings, select option 5 of the I (inquiry) command:

AT I5 <Enter>

Customizing NVRAM

&W	To substitute a template other than &F1, write the desired template to NVRAM, using the &W command.

AT &F2 &W <Enter>

To modify the &Fn configuration in NVRAM, type your changes and then save them to NVRAM, as in the following example.  The original factory template remains intact.

AT M2 S10=40 &A2 &W <Enter>

NOTE:  When writing a different default configuration to NVRAM, insert any additions after the &Fn command but before &W.  Otherwise they will be overwritten by &Fn.

After sending a configuration to NVRAM, you can change any setting just for the current session, as in the following example.  The NVRAM configuration remains intact.

ATX6 <Enter>

But if you want the new setting to be a default, write it to NVRAM at the same time, as in the following example.  X7 is substituted for the Xn value stored earlier.  Any other setting that was changed and can be saved to NVRAM will also be saved.

AT X7 &W <Enter>

Resetting the Modem

Z	Software reset to NVRAM settings when DIP switch 10 is OFF (factory setting).  If DIP switch 10 is OFF, the modem resets to the &F0 configuration template, with no flow control.

NOTE:  Use the ATZ command also if you've changed the position of DIP switches 1-7 or 9 while the modem is on, so that the modem can read the new setting.  The only other way to initiate a new setting for switches 1-7 and 9 is to turn the modem off and on again.

CONFIGURATION

Echo/Speaker

En	Command mode local echo.  Enables/disables the display of your typed commands.  If double characters appear on the screen, both the modem's local echo and your software's local echo are on.

The Courier is shipped with DIP switch 4 OFF, enabling local echo.  The En command controls the local echo for a current session, independently of the switch setting.  At power-on and reset, the modem operates according to the DIP switch setting.  The En command is not stored in nonvolatile memory as a power-on/reset default.

E0	Command mode echo OFF.  The modem does not display keyboard commands.

E1	Command mode echo ON.

Fn	Online local echo.  This command causes the modem to display a copy of the data it is transmitting to another system.  Many systems, however, return a copy of received data, which is called a remote echo.  If the modem's online echo is ON and there is also remote echoing, double characters appear on the screen.

In some microcomputer documentation, the term duplex is applied to local online echoing, although the term is not technically accurate.

F0	Online echo ON.  Sometimes called half duplex.  As the modem transmits data to a remote system, it also sends a copy of the data to the screen.

F1	Online echo OFF.  Sometimes called full duplex.  Default.

Mn	Speaker (audio monitor).

M0	The speaker is always OFF.

M1	The speaker is ON until carrier is established.  Default.

M2	The speaker is always ON, including during data transfer.

M3	The speaker is ON after the last digit is dialed and remains ON until carrier is established.

Result Codes

Qn 	Enable/suppress the display of result codes.  The Courier is shipped with DIP switch 3 ON, to display result codes.  Use the Qn command to control the display for a current session, independently of the switch setting.

At power-on and reset, the modem operates according to the DIP switch setting.  The Qn command is not stored in nonvolatile random access memory.

Q0	Result codes displayed.

Q1	Result codes suppressed (quiet).

Q2	Result codes suppressed in Answer mode.

Vn	Return result codes in words or numbers (Verbal/Numeric mode). At power-on and reset, the modem operates according to the DIP switch setting.  The Vn command is not stored in nonvolatile memory as a power-on/reset default.

V0	Numeric mode.

V1	Verbal mode.

Xn	Result code set options.  Use the following table (Default = X7, all codes except 12/VOICE).  For result codes for synchronous operations, see Appendix F.

Setting

Result Codes	X0	X1	X2	X3	X4	X5	X6	X7

0/OK	·	·	·	·	·	·	·	·

1/CONNECT	·	·	·	·	·	·	·	·

2/RING	·	·	·	·	·	·	·	·

3/NO CARRIER	·	·	·	·	·	·	·	·

4/ERROR	·	·	·	·	·	·	·	·

5/CONNECT 1200		·	·	·	·	·	·	·

6/NO DIAL TONE			·		·		·	·

7/BUSY				·	·	·	·	·

8/NO ANSWER				·	·	·	·	·

9/RESERVED

10/CONNECT 2400		·	·	·	·	·	·	·

11/RINGING						·	·	·

12/VOICE						·	·

13/CONNECT 9600		·	·	·	·	·	·	·

18/CONNECT 4800		·	·	·	·	·	·	·

20/CONNECT 7200 		·	·	·	·	·	·	·

21/CONNECT 12000		·	·	·	·	·	·	·

25/CONNECT 14400		·	·	·	·	·	·	·

47/CONNECT 16800 		·	·	·	·	·	·	·

85/CONNECT 19200		·	·	·	·	·	·	·

91/CONNECT 21600		·	·	·	·	·	·	·

99/CONNECT 24000		·	·	·	·	·	·	·

103/CONNECT 26400		·	·	·	·	·	·	·

107/CONNECT 28800		·	·	·	·	·	·	·

Functions

Adaptive Dialing			·	·	·	·	·	·

Wait for 2nd Dial Tone (W)				·	·	·	·	·

Wait for Answer (@)				·	·	·	·	·

Fast Dial			·		·		·	·

Table 4.1¾Result Code Options

NOTE:  Additional messages indicate an error control connection and the modulation for a call.  See the next section, Additional Result Code Subsets.

Result Code	Meaning

0/OK	Command has been executed.

1/CONNECT	Connection with another modem; if set to X0, connection may be between 300 and 28.8 bps; if X1 or higher, connection is at 300 bps.

2/RING	Incoming ring detected.

3/NO CARRIER 	Carrier detect has failed or carrier has been dropped due to disconnect.

4/ERROR	Command is invalid.

5/CONNECT 1200	Connection with another modem at 1200 bps.

6/NO DIAL TONE	Dial tone not detected during the normal 2 seconds, set in Register S6.

7/BUSY	Busy signal detect; modem hangs up.

8/NO ANSWER	After waiting 5 seconds for an answer, modem hangs up; returned instead of NO CARRIER when the @ option is used.

10/CONNECT 2400	Connection with another modem at 2400 bps.

11/RINGING	The modem has dialed; remote phone line is ringing.

12/VOICE	Voice answer at remote site; modem hangs up.

13/CONNECT 9600	Connection at reported rate.  Same meaning for results of 4800 (18), 7200 (20), 12K (21), 14.4K (25), 16.8K (47), 19.2K (85), 21.6K (91), 24K (99), 26.4K (103), or 28.8K (107)

Adaptive Dialing	The modem attempts to use tone dialing and, if that doesn't work, reverts to rotary dialing.

Wait for Another	The modem continues dialing as soon as it detects

Dial Tone (W)	another dial tone.  See the dial options earlier in this chapter.

Wait for an 	The modem continues dialing when it detects 5 seconds of

Answer (@)	silence on the line.  See the dial options earlier in this chapter.

Fast Dial	The modem dials immediately on dial-tone detect, instead of waiting the normal 2 seconds set in Register S6.

Table 4.2¾Result Code Definitions

Additional Result Code Subsets

NOTE:  ARQ (Automatic Repeat Request) is used in this manual to denote calls under error control.

&An	Enable/disable additional result code subsets.  See the Xn command earlier in this chapter.

&A0	ARQ result codes are disabled.  This setting does not affect an error control connection; the modem returns the standard CONNECT messages if result codes are enabled.

&A1	ARQ result codes are enabled, indicating that a connection is under error control.  Message 14 is displayed if the modem is set to X0 and the connection is at any rate from 1200 to 28.8K bps.  The remaining results indicate the connection rate and require a setting of X1 or higher.

14/CONNECT/ARQ	57/CONNECT 16800/ARQ

15/CONNECT 1200/ARQ	88/CONNECT 19200/ARQ

16/CONNECT 2400/ARQ	94/CONNECT 21600/ARQ

17/CONNECT 9600/ARQ	100/CONNECT 24000/ARQ

19/CONNECT 4800/ARQ	104/CONNECT 26400/ARQ

22/CONNECT 12000/ARQ	108/CONNECT 28800/ARQ

24/CONNECT 7200/ARQ

26/CONNECT 14400/ARQ

&A2	Additional VFC, V34, HST, or V32 modulation indicator.  Included for users of HST Dual Standard modems.  If your software cannot handle the added modulation information, select &A1 or &A0.

23/CONNECT 9600/HST		27/CONNECT 9600/ARQ/HST

28/CONNECT 4800/HST		29/CONNECT 4800/ARQ/HST

30/CONNECT 7200/HST		34/CONNECT 7200/ARQ/HST	31/CONNECT 12000/HST		32/CONNECT 12000/ARQ/HST	35/CONNECT 14400/HST		36/CONNECT 14400/ARQ/HST	53/CONNECT 16800/HST		57/CONNECT 16800/ARQ/HST	33/CONNECT 9600/V32 		37/CONNECT 9600/ARQ/V32

38/CONNECT 4800/V32		39/CONNECT 4800/ARQ/V32

40/CONNECT 7200/V32		44/CONNECT 7200/ARQ/V32

41/CONNECT 12000/V32		42/CONNECT 12000/ARQ/V32

45/CONNECT 14400/V32		46/CONNECT 14400/ARQ/V32

83/CONNECT 16800/V32		84/CONNECT 16800/ARQ/V32

87/CONNECT 19200/V32		90/CONNECT 19200/ARQ/V32

93/CONNECT 21600/V32		96/CONNECT 21600/ARQ/V32

97/CONNECT 21600/VFC		98/CONNECT 21600/ARQ/VFC

101/CONNECT 24000/VFC		102/CONNECT 24000/ARQ/VFC

105/CONNECT 26400/VFC		106/CONNECT 26400/ARQ/VFC

109/CONNECT 28800/VFC		110/CONNECT 28800/ARQ/VFC

139/CONNECT 14400/VFC		141/CONNECT 14400/ARQ/VFC

143/CONNECT 16800/VFC		145/CONNECT 16800/ARQ/VFC

147/CONNECT 19200/VFC		149/CONNECT 19200/ARQ/VFC

111/CONNECT 21600/V34		112/CONNECT 21600/ARQ/V34

113/CONNECT 24000/V34		114/CONNECT 24000/ARQ/V34

115/CONNECT 26400/V34		116/CONNECT 26400/ARQ/V34

117/CONNECT 28800/V34		118/CONNECT 28800/ARQ/V34

120/CONNECT 2400/V34		122/CONNECT 2400/ARQ/V34

124/CONNECT 4800/V34		126/CONNECT 4800/ARQ/V34

128/CONNECT 7200/V34		130/CONNECT 7200/ARQ/V34

132/CONNECT 9600/V34		134/CONNECT 9600/ARQ/V34

136/CONNECT 12000/V34		138/CONNECT 12000/ARQ/V34

140/CONNECT 14400/V34		142/CONNECT 14400/ARQ/V34

144/CONNECT 16800/V34		146/CONNECT 16800/ARQ/V34

148/CONNECT 19200/V34		150/CONNECT 19200/ARQ/V34

&A3	Additional error control indicator (LAPM, HST, MNP, SYNC, or NONE) and data compression type (V42BIS or MNP5).  Default.  When the call is not under one of those protocols (and ARQ is not included in the result code), the modem reports either SYNC, indicating a synchronous connection, or NONE, for no protocol.

If the modems are using data compression, the type of compression, V42BIS or MNP5, is added to the result code.  In the first of the following examples, the modems negotiated error control for the call (ARQ), used VFC modulation, are using the LAPM error control protocol, and are using V.42 bis compression.

CONNECT 28800/ARQ/VFC/LAPM/V42BIS [or MNP/MNP5]

CONNECT 19200/ARQ/V32/LAPM/V42BIS  [or MNP/MNP5]

CONNECT 16800/ARQ/HST/HST/V42BIS  [or MNP/MNP5]

CONNECT 14400/ARQ/V32/LAPM/V42BIS [or MNP/MNP5]

CONNECT 9600/ARQ/HST/CELLULAR/HST/V42BIS  [or MNP5]

CONNECT 9600/SYNC

CONNECT 2400/NONE

NOTE:  Although these codes will return numeric identifiers, they are the same numeric identifiers used for &A2 result codes.  If the modem is in Numeric mode (V0) and set to &A3, you will not be able to differentiate between &A2 and &A3 result codes.  &A3 result codes may not be compatible with some software.

Modulation

Bn	Handshake options.  There are three commands that apply to international calls above 1200 bps¾Bn, &Gn, &Pn.  See International Calls later in this chapter for information on the other two settings.

B0	ITU-T (formerly CCITT) answer sequence.  Default.  This is required to answer all V.32-type calls, as well as calls from overseas.

B1	Bell answer tone.  This setting selects HST modulation in Dual Standard modems, but should only be used if the modem is not required to answer V.32-type calls.  
NOTE:  This setting is required for HST cellular calls.

Error Control/Data Compression

&Mn	Enable ARQ (error control) or synchronous protocols.  Both your modem and the remote modem must use the same protocol.

&M0	Normal mode, no error control.  Due to the nature of phone line channels, this is never recommended for calls above 2400 bps.

&M1	This setting is exclusive of the modems' error control and is used only for online synchronous mode without V.25 bis.  See Appendix F for more information .

&M2	Reserved.

&M3	Reserved.

&M4	Normal/ARQ mode.  Default.  If an ARQ connection isn't made, the modem operates in Normal mode, as though it were set to &M0.

NOTE:  When V.32-type modems revert to Normal mode, they transfer data at high speeds without the reliability of error control.  To avoid this, both local and remote modems should always be set for error control.  Modems in HST mode, if unable to establish an error control connection, drop to 2400 bps.

&M5	The modem enters ARQ asynchronous mode.  The modem hangs up if an ARQ connection cannot be made.

&M6	The modem enters V.25 bis synchronous mode, using a character-oriented link protocol similar to BISYNC.  See Appendix F for more information

&M7	The modem enters V.25 bis synchronous mode, using the HDLC link protocol.

&Kn	Enable/disable data compression.

&K0	Data compression disabled.

&K1	Auto enable/disable.  Default.  The modem enables compression if the serial port rate is fixed, &B1.  It disables compression if the serial port rate follows the connection rate, &B0, because compression offers no throughput advantage when the serial port and connection rates are equal.  Compression may even degrade throughput.

&K2	Data compression enabled.  Use this setting to keep the modem from disabling compression.

&K3	Selective data compression.  The modem negotiates only for V.42 bis compression, and disables MNP Level 5 (MNP5) compression.  Use this setting to transfer 8-bit binary files, .ZIP files, and other files that are already compressed.  See the note below.

NOTE:  MNP5 compression is not useful when transferring files that are already compressed, such as the .ZIP files downloaded from many Bulletin Boards and 8-bit binary files, which appear to the modem to be compressed.  MNP5 tends to add data to the transmission so that throughput over the link degrades.  V.42 bis compression dynamically detects when data is already compressed and turns off until it detects that compression will work to advantage.  The special &K3 setting enables the best throughput for already-compressed files.

See Throughput Guidelines and Data Compression in Appendix A for more information, including throughput to expect for different kinds of files.

Data Rates

The modem can be set to a fixed or variable serial port rate.  A fixed rate sets the modem for the highest possible throughput and provides the best performance.  A variable rate allows the modem to switch to match the more limited rate on the phone connection.

Your software must support fixed or variable serial port rates, and must be set to either of the two settings.  NOTE:  Your software may refer to these options with terms like locked serial port (fixed rate) or autobaud (variable rate).

Most communications programs support variable rates, but not all software supports fixed rates.

&Bn	Serial port rate variable or fixed.

&B0	Variable rates.  When the modem switches its connection rate to connect with a modem operating at a different rate, it also switches its serial port rate.  The software or terminal also switches serial port rates to match the connection rate.

&B1	Fixed rate.  Default.  The modem always communicates with the terminal or computer at the rate at which you have set the terminal or software, regardless of the connection rate.  For the greatest throughput, set the serial port to 115.2K, 57.6K, 38.4K bps for high speed calls and to at least 9600 bps for 2400-bps calls.

This setting is not affected by the &N setting.  However, the serial port rate must be equal to or higher than the &Nn rate.

&B2	Fixed for ARQ calls/Variable for non-ARQ calls.  Answer mode only.  When the modem goes off hook and connects in ARQ mode, it shifts its serial port rate up to a user-specified rate, for example, 38.4K bps.  If the connection is not under error control, the modem behaves as if it were set to &B0 and switches its serial port rate to match the connection rate of each call.

To implement this feature, first set your software to the desired rate.  Then send the modem the AT &B2 [other settings] &W command.

The modem stores the rate of the command in NVRAM along with the settings.  Each time it makes an ARQ connection, the modem checks NVRAM for the specified serial port rate.

When sending subsequent configurations to NVRAM, be sure your software is set to your selected serial port rate, so that the correct rate is maintained.

&Nn	Connection rate variable or fixed.

&N0	Variable rates.  Default.  The Courier negotiates with the remote modem for the highest possible connection rate, depending on the capabilities of the remote modem.

&N1-	Fixed rate.  The modem only connects if the remote

&N14	modem is operating at the same rate.  If not, the modem hangs up.  If you wish, you can filter out calls at other than a specific rate, for security or other reasons, by fixing the connection rate.

The connection rate must always be lower than, or equal to, the serial port rate, never higher.

The options are as follows.

&N1	300 bps	&N8	14.4K bps
&N2	1200 bps	&N9	16.8K bps 
&N3	2400 bps	&N10	19.2K bps
&N4	4800 bps	&N11	21.6K bps
&N5	7200 bps	&N12	24K bps 
&N6	9600 bps	&N13	26.4K bps

&N7	12K bps	&N14	28.8K bps

RS-232 Signal Operations

&Cn	Carrier Detect operations.  At power-on and reset, the modem operates according to the setting of DIP switch 6.  This command is not stored in nonvolatile memory as a power-on/reset default.

&C0	CD override, CD always ON.

&C1	Normal CD operations.  The Courier sends a CD signal when it connects with another modem and drops the CD when it disconnects.

&Dn	Data Terminal Ready (DTR) operations.  At power-on and reset, the modem operates according to the setting of DIP switch 1.  This command is not stored in nonvolatile memory as a power-on/reset default.

&D0	DTR override.  The modem operates as though the DTR is always ON.

&D1	Advance usage:  If issued before connecting with another modem, the modem can enter online command mode during a call by toggling DTR.  (Most communications software packages have a method for toggling DTR.)  &D1 functions similarly to the escape code (+++), except that this setting is independent of DIP switch 9.

If DIP Switch 1 is ON (DTR override) when you issue the &D1 command, the DTR override is automatically turned off.  However, if you change the setting of DIP switch 1 after issuing &D1, the DIP switch setting takes precedence.

Return online with the On command, or hang up with the Hn command.

&D2	Normal DTR operations.  The terminal or computer must send a DTR signal for the modem to accept commands.  Dropping DTR terminates a call.

&Sn	The modem sends the computer or terminal a Data Set Ready (DSR) signal via the RS-232 interface.  (Data Set is industry jargon for modem.)  Few, if any, commercial communications programs require the modem to control DSR, &S1.  Leave the modem set for DSR overridden, &S0, unless you know that your installation requires a different setting.

&S0	DSR is always ON (override).  Default.

&S1	In Originate mode, the modem sends the DSR after it dials, when it detects the remote modem's answer tone.  In Answer mode, the modem sends the DSR after it sends an answer tone.

&S2	This option is for specialized equipment such as automatic callback units.  On loss of carrier, the modem sends a pulsed DSR signal with Clear to Send (CTS) following Carrier Detect (CD).

&S3	This is the same as &S2, but without the CTS following CD.

&S4	The modem sends the computer a DSR signal at the same time that it sends the Carrier Detect (CD) signal.

Flow Control

Flow control allows the modem to monitor the amount of data coming from the computer or the remote modem, and notify either end if its buffers are too full, so that they stop sending data for a moment.

The modem uses either hardware or software flow control.  Your software and machine must support whichever type you select.

Hardware Control

The modem drops the Clear to Send (CTS) signal it's been sending to the computer or terminal when the modem's buffer nears 90% capacity.  It starts sending CTS again when the buffer is about half full.

Software Control

The modem sends the computer or terminal the standard ASCII Transmit OFF (XOFF) character, <Ctrl>-S, when its buffer nears 90% capacity.  The  modem sends the ASCII Transmit ON character, <Ctrl>-Q, when the buffer is about half full.  ASCII definitions are as follows:

XON      <Ctrl>-Q       (ASCII 17 Decimal, 11 Hex)

XOFF     <Ctrl>-S       (ASCII 19 Decimal, 13 Hex)

NOTE:  You should set your software as well to either hardware or software flow control.  Some programs also require that you turn off the type you are not using.

The ASCII characters may be user-defined.  See Registers S22 and S23 in Appendix B.  That appendix also includes an ASCII chart.

WARNING:  If possible, always use hardware flow control, the factory default.  You may lose data if XON/XOFF (<Ctrl>-S, 
<Ctrl>-Q) characters occur in the data stream from other sources.  They may, for example, come from the remote system:  an XON from the remote system, after your modem has sent an XOFF, can result in buffer overflow.

<Ctrl>-S (XOFF) and <Ctrl>-Q (XON) characters also occur in binary files, and are used by Xmodem-type protocols.  You risk having these characters misinterpreted as modem flow control characters and dropped from the data stream.

If you cannot use hardware flow control and if you're transferring non-text (binary) files, or using an Xmodem-type protocol, disable flow control entirely (&H0).  In addition, be sure the modem is set to &B0 and &N0, so that the serial port and connection rates are equal.

### Transmit Data Buffer Sizes

The Transmit Data refers to the data from the computer, which the modem is to transmit over the phone line.

The size of the Transmit data buffer depends on whether the connection is under error control or not, as follows.

•	ARQ connections:  3.25K bytes.

•	Non-ARQ connections:  1.5K bytes, allowing use of error control file transfer protocols such as Xmodem and Ymodem without flow control.

If bit 3 of Register S15 is turned on, the non-ARQ buffer size is reduced to 128 bytes, for the convenience of BBS operators taking calls from remote users of slower modems.  See S-Register Summary, S15, in Appendix B.

### Received Data Buffer Size

Received Data refers to the data the modem receives over the phone link, which the modem passes on to the attached computer.

The size of this buffer remains constant at 2K bytes.

Transmit Data Flow Control

&Hn	This type of flow control is for data transmitted to the modem by its attached computer or terminal.  The modem monitors its buffer as data comes from the computer or modem.  If the buffer approaches 90% capacity, the modem signals the computer or terminal to stop transmitting.  When the modem has sent enough data over the link to half empty the buffer, it signals the computer or terminal to resume transmitting.

&H0	Transmit Data flow control disabled.

&H1	Hardware flow control.  Default.  Requires that your computer or terminal and software support Clear to Send (CTS) at the RS-232 interface.

&H2	Software flow control.  Requires that your software support XON/XOFF signaling.

&H3	Use both hardware and software flow control.  If you are unsure about what your equipment supports, select this option.  But keep the warning, above, in mind about software flow control.

Received Data Flow Control

Separate commands, &Rn (hardware) and &In (software), control the flow of Received Data passed by the Courier to your computer or terminal.

Your software and machine must support whichever type you select, although we recommend hardware flow control, if possible.

### Hardware Control(&Rn)

&R0	Delay Clear to Send Response after Request to Send signal (RTS/CTS delay).  The delay is required by some synchronous mainframes and does not apply to asynchronous calls.

&R1	The modem ignores RTS.  This setting is required if your computer or terminal or software does not support RTS.

&R2	Hardware flow control of received data enabled.  Default.  The modem sends data to the computer or terminal only on receipt of the RTS signal.

### Software Control (&In)

WARNING:  In ordinary operation, the only characters the modem recognizes during a call are the three pluses (+++) of the escape code.  But when software flow control is enabled, the modem also looks for <Ctrl>-S or <Ctrl>-Q characters.  If these characters occur in a file or as part of a protocol, the modem reads them as XON/XOFF characters and acts on them.  In some cases, the modem drops them from the data stream.

&I0	Disables XON/XOFF flow control of received data.  Default.  Recommended for non-ARQ (Normal mode) calls, but see &I5.  The I0 option provides transparency for all characters except the escape code sequence (+++), because at this setting the modem does not look for control characters.

NOTE:  Because of the risk described in the above warning, the settings that follow are only recommended for users whose data does not include XON/XOFF control characters.

&I1	The Courier acts on your typed XON/XOFF commands, <Ctrl>-S or <Ctrl>-Q, and passes them to the remote computer.  Use in ARQ mode only, but keep in mind that the XON/XOFF characters sent to the remote computer may interfere with XON/XOFF signaling between the remote computer and remote modem.  See &I2.

&I2	The Courier acts on your XON/XOFF commands, but removes them from the data stream instead of passing them to the remote computer.  This ensures that the remote computer does not confuse your XON/XOFF characters with those from its attached modem.  This is the recommended setting for ARQ mode.

However, if the call is not in ARQ mode, there is no flow control on the phone link.  If you send an XOFF to your modem and it stops passing data, it has no way to tell the remote computer and modem to stop sending for a while, and your modem's buffer may overflow.  For more reliable control in non-ARQ mode, see &I5.

&I3	Hewlett Packard¾Host mode.  Applies only to modems attached to an HP mainframe that uses the ENQ/ACK protocol.  Use in ARQ mode only.  See Appendix G.

&I4	Hewlett Packard¾Terminal mode.  Applies only to modems attached to terminals in an HP system that uses the ENQ/ACK protocol.  Use in ARQ mode only.  See Appendix G.

&I5	This setting is designed to enable flow control on the phone link when the connection is not under error control.  For this to work for you, the remote modem must also have &I5 capability.

In ARQ mode, a Courier set to &I5 operates the same as it does when set to &I2.  It acts on your XON/XOFF commands, but does not pass them to the remote system.  The error control protocol enables the modems to control the flow of data on the phone link.

In non-ARQ mode, a Courier set to &I5 operates as though flow control were disabled (&I0);  it does not look for your typed XON/XOFF commands.  However, it looks for XON/XOFF characters coming in over the phone link.  When the remote operator sends XON/XOFF commands, the Courier either resumes or stops transmitting data over the link and drops the characters from the data stream.

If both modems are set to &I5, operators at each end can signal the remote modem to stop sending, thereby controlling the data flow on the phone link and preventing their own modem's buffer from overflowing.  At the computer or terminal/modem interfaces, the modems independently control the flow of data through their Transmit Data (&H) settings.

### Guidelines

Use of software flow control may prove satisfactory if you're only transferring text files.  However, if you're transferring non-text (binary) files, or using an Xmodem-type protocol, disable flow control entirely (&R1, &I0).  In addition, set the modem to &B0 and &N0, so that the serial port and connection rates are equal.

S-REGISTERS

The S-Registers are used to set various timing parameters, redefine selected ASCII characters, and other configuration options.  A detailed summary of the S-register functions is in Appendix B.  A less detailed summary is in the Quick-Reference card.

Sr=n	Set S-Register value:  r is any S-Register; n must be a decimal number between 0 and 255.

Sr.b=n	Alternative command for setting bit-mapped registers:  r is the bit-mapped register; .b is the bit; n is 0 (off) or 1 (on).

Sr?	Query contents of register r.

INQUIRY AND HELP

The modem displays information such as the current modem settings, product code, and call duration.  It also displays summary information for every command that the modem supports.

For more information on Inquiry and Help commands, including sample displays, see Chapter 6.

TESTING

The modem can perform a number of tests including Analog Loopback, Digital Loopback, and Remote Digital Loopback.  These tests can be used to check the operations of the modem's transmitter and receiver, or to locate a problem with a remote modem or telephone channel.  Error control must be disabled for these tests.

See Appendix H for more information.

INTERNATIONAL CALLS

There are three commands that apply to international calls above 1200 bps¾Bn, &Gn, &Pn.  Bn is described earlier in this chapter.

&Gn	This setting applies only to overseas calls at 2400 or 1200 bps.  British phone switching systems require the modem to send an 1800 Hz guard tone after it sends an answer tone.  Some other European phone networks require a 550 Hz guard tone.  Guard tones are not used in the United States or Canada.

&G0	No guard tone.  This is used in the U.S. and in Canada.  Default.

&G1	This sets a 550 Hz guard tone, and is used in some European countries.

&G2	This sets an 1800 Hz guard tone, and is used in the U.K. and some Commonwealth countries.  &G2 requires the B0 setting.

&Pn	This command sets the ratio of the off-hook/on-hook (make/
break) interval for pulse dialing.  The default sets the modem for use in North America.  The ratio must be changed if the modem is used in the United Kingdom and some Commonwealth countries.

&P0	Make/break ratio, U.S./Canada:  39%/61%.  Default.

&P1	Make/break ratio, United Kingdom, some Commonwealth countries:  33%/67%.

MISCELLANEOUS COMMANDS

Cn	Transmitter enabled/disabled.

C0	Transmitter disabled; receive-only condition.

C1	Transmitter enabled.  Default.

Kn	Modem clock operation:  Call-duration or Real-time mode.  Displayed with ATI3 and ATI6 commands.

K0	Display current call-duration if online.  Display
last call-duration if offline.  Default.

K1	Return actual time at ATI3.  Clock is set using ATI3=HH:MM:SS K1.

&Yn	Break handling.  This command allows you to send a break to abort data transfer without disconnecting from the phone link.

&Y0	Destructive, don't send Break.

&Y1	Destructive, expedited (Default).

&Y2	Nondestructive, expedited.

&Y3	Nondestructive, unexpedited; modem sends Break in sequence with data received from computer or terminal.

NOTE:  If the call is under MNP5 data compression, destructive Breaks cause both modems to reset their data compression tables.  When transmission resumes, the modems build new tables, and the result is lower than normal throughput.

&ZC=s	Write the following command string s to NVRAM.  The command string may be up to 30 characters long; spaces are not counted.  This command can be used so that you can call another modem without loading your communications software.

After storing a command, you can program the voice/data switch to execute the stored command string when pressed.  The following example assigns a command string that displays the link diagnostics screen when you press the voice/data switch.

AT&ZC=I6 <Enter>

The function of the voice/data switch is determined by the setting of Register S32, as described in Appendix G.  Set the voice/data switch function to execute stored command string by setting Register S32 to 9 with the following command:

ATS32=9 <Enter>

Note that you can reset the voice/data switch at any time to one of the other available functions.  Additionally, you can overwrite the stored command string with a new one at any time.

Once you've stored your command string and set Register S32, all you need to do is press the voice/data switch whenever you want the command string executed.

&ZC?	Display the stored command string.

%T	Enables the modem, when off hook, to detect the tone frequencies of dialing modems.  %T is meant primarily for use with network applications, but may also be integrated into certain software programs.  For example, %T could be used in a security program to identify incoming tone security codes.

To enable %T, type ATH1 <Enter> to force the modem off hook.  Then type AT%T <Enter>.

To return the modem to Command mode, press any key or drop the computer's or terminal's DTR signal.  The modem responds OK.