# Appendix I. Software Upgrades

The Courier V.34 modem is software upgradable.  You can download upgrades from the USR Bulletin Board Service (BBS) to obtain maintenance fixes or new features.

We suggest you retain a copy of the most recent upgrade on disk so you can download it to your modem again, should the modem lose its code for any reason.

The software download program requires DOS version 3.0 or higher and may be run from the DOS shell under Microsoft Windows.

Call the USR BBS

1.	Call the USR BBS:

ATDT 708 982 5092 <Enter>

2.	Unless you are running an ASCII-based communications program, answer YES at the graphics prompt when you connect to the BBS.

3.	Press Enter with each prompt until you come to the main menu.

Download the Upgrade File

1.	At the command line of the main menu, type F (File) and press Enter.  Select area 5 (Courier) from the file area menu  The most recent zipped files will display.

2.	Use the Enter key to scroll through the list and use the spacebar key to highlight a file.

Scroll through the list and highlight the file named USRSDL.EXE.  Press Enter when it is highlighted to flag the file for download.

3.	Press Enter to go back to the main menu.

4.	At the command line on the main menu, type D and press Enter to initiate a download.

5.	Answer the download prompts according to your system requirements.  The file will be downloaded to the directory specified in your communications software.

6.	When the file transfer is complete and you are ready to leave the BBS, type G (Goodbye) from the main menu.

Upgrade your Modem

NOTE:  Your modem must be turned on for the following operation.

1.	At the DOS prompt of the directory where your copy of the file has been downloaded, type USRSDL and press Enter.  The zipped file will self-extract several files onto your computer hard disk.

2.	Type SDL and press Enter.

The .EXE file checks the code embedded in itself.  If there is a problem, an error message appears and the operation is terminated.  If you receive an error message, download the file again.

3.	A screen appears with default COM port information.

If you want to alter the defaults, use the guidelines below.

/c=n	Enter this command to select one of four predefined COM ports on an IBM-compatible PC.  Valid entries for n are 1, 2, 3, and 4.  The default is COM 2.  If you change the COM port setting, a compatible IRQ will be displayed.

/i=n	Enter this command to set up a custom port.  It specifies which IRQ (interrupt request) the COM port will use to interrupt the processor for service.  Valid entries for n are 2, 3, 4, 5, and 7.  The default is IRQ 3.

/a=n	Enter this command to specify a port address, where n is the hexadecimal address of a custom port only.

/b=n	Enter this command to specify the bit rate at which the COM port will communicate with the modem.  Valid entries for n are 9600, 19200, 38400, 57600, and 115200.  The default is 57600 bps.

/q	This command allows you to shut off most of the screen display as the program upgrades your modem code for batch-mode execution.  You will not be prompted to accept settings.

/?	This command displays a help screen.

4.	A prompt asks if you want to “Download Using These Settings? (Y/N).”  Type Y if you want to continue.  Type N if you want to abort the operation.

NOTE:  This is the only opportunity you will have to abort the operation.

5.	Once you have accepted the settings, the software download will begin.  During this operation, the Modem Ready (MR) LED goes out.

6.	When the operation has successfully completed, the “Modem reports download successful” message displays and the MR LED lights up again.

Troubleshooting

If your modem indicates an error, try running the SDL program at a lower serial port rate.  If your computer doesn’t have a 16550 UART, a slower serial port rate can make all the difference.

You can also try running the program on a different PC.  An idiosyncracy of an off-brand PC or an uncommon version of DOS my hang up the SDL program.