APPENDIX B.  SUMMARIES AND TABLES

CONTENTS

The RS-232 Interface, with Pin Definitions

Front Panel Indicators

DIP Switch Summary

Default Settings

S-Register Summary

ASCII Chart

THE RS-232 INTERFACE

DESCRIPTION

The RS-232 interface is a standard developed by the Electronic Industries Association (EIA).  It defines the signals and voltages used when data is exchanged between a computer or terminal and a modem or serial printer.  Data is transmitted between the devices via a cable with 25-pin, 9-pin, 8-pin or custom-built connectors.

The modem takes a DB-25P (25-pin plug, or male) connector at one end of the cable.  Computer equipment varies, however.  Check the serial port at the rear of your machine, which may be labeled serial, comm port, or some other term (e.g., RS-232).  If there are no labels, review your machine documentation to find out which is the serial port.  (There may be more than one.)  Don't use a port marked parallel, printer or aux.

The physical serial port on the computer or terminal will be either a socket (female) or plug (male) that typically accommodates 25 or 9 pins.  For example, the port on the IBM PC, PC/XT and most compatibles requires a DB-25S (socket) connector, while the port on the IBM PC/AT and some compatibles requires a DB-9S connector.  Apple computers require a DB-25P, DB-9P or, more typically, an 8-pin round plug connector.  Check your computer documentation or with your computer dealer.

## PIN ASSIGNMENTS

The entire standard covers many more functions than are used in most data communications applications.  Pin assignments are factory set in the Courier to match the standard DB-25 assignments in the following table.  DB-9 connectors for AT-compatible computers should be wired at the computer end of the cable as shown in the DB-9 column.  If you're using an Apple computer, we strongly recommend that you purchase a Hardware Handshaking cable to get the highest possible reliability performance.

Signal Flow

DB-25	DB-9	Circuit	Function	Computer¾
					Modem

1	_	AA	Chassis Ground	both directions

2	3	BA	Transmitted Data	to modem

3	2	BB	Received Data	to computer

4	7	CA	Request to Send	to modem

5	8	CB	Clear to Send	to computer

6	6	CC	Data Set Ready	to computer

7	5	AB	Signal Ground	both directions

8	1	CF	Carrier Detect	to computer

12	_	SCF	Speed Indicate	to computer

15	_	DB	Synchronous TX* Timing	to computer

17	_	DD	Synchronous RX* Timing	to computer

20	4	CD	Data Terminal Ready	to modem

22	9	CE	Ring Indicate	to computer

24	_	DA	Synchronous TX* Timing	to modem

* Indicates Transmitter (TX) or Receiver (RX)

Table B.1¾RS-232 Interface Pin Definitions

Minimum Requirements

Some computer/terminal equipment supports only a few of the RS-232 signal functions set in the Courier.  The minimum required for the modem to operate are as follows:

### Asynchronous Calls

DB-25	DB-9

Pin	Pin	Function

2	3	Transmitted Data

3	2	Received Data

7	5	Signal Ground

20	4	Data Terminal Ready*

* Required only if you have the Data Terminal Ready Operations switch OFF (DIP switch 1 OFF).

### Synchronous Calls

You will need all of the above functions as well as pin 15 for Transmitter timing signals, and pin 17 for Receiver timing signals.  You may need pin 24, which is assigned the external timing source, rather than the internal (modem) source assigned to pin 15.  See Appendix F for more detailed information.

### Additional Flow Control Functions

If your computer and software support Clear to Send and you wish to use Transmit Data hardware flow control (&H1), Pin 5 (DB-25) or Pin 8 (DB-9) is required.

If your computer and software support Request to Send and you wish to use Received Data hardware flow control (&R2), Pin 4 (DB-25) or Pin 7 (DB-9) is required.

## For 38.4K or higher Serial Port rate

Your terminal or computer and software must support the 115.2K, 57.6K, or 38.4K bps rate.  Make sure the RS-232 cable is shielded.  Cables are normally six feet long, but longer lengths are possible.  If you encounter problems with signal degradation, try a shorter cable.

If you decide to build your own cable, use a low-capacitance cable.  To further minimize the capacitance, connect only those functions (pins) that your application requires.

FRONT PANEL INDICATORS

Symbol	Meaning	Status

HS	High Speed	All calls above 2400 bps:  ON during call progress, after completion of dialing; OFF during HST-mode link negotiations at 2400 bps, then ON during connection.  Remains ON after disconnect until next call is originated or answered, or the modem is reset.

AA	Auto Answer/	Answer mode only:  ON when your modem is

Answer	in Auto Answer mode, and when answering a call; in HST-mode, goes OFF if the channel is reversed and your answering modem transmits at 450 or 300 bps.  Also goes OFF when the modem originates a call.  Flashes ON for incoming ring detect.

CD	Carrier Detect	ON if DIP switch 6 is OFF (factory setting) and the Courier receives a valid data signal (carrier) from a remote modem, indicating that data transmission is possible.  Also ON when the CD override is on, DIP switch 6 ON.

OH	Off Hook	ON when the Courier takes control of the phone line to establish a data link.

RD	Received Data	Flashes when the modem sends result codes or passes received data bits to the computer or terminal.

SD	Send Data	Flashes when the computer or terminal sends a data bit to the Courier.

TR	Data Terminal	ON if DIP switch 1 is OFF (factory setting) and

Ready 	the modem receives a DTR signal from the computer or terminal.  Also ON when the DTR override is on, DIP switch 1 ON.

Symbol	Meaning	Status

MR	Modem Ready/	ON when the Courier is powered on.

Test Mode	Flashes when the modems retrain, including online fallback, or while the modem is in Test mode.

RS	Request to 	ON if your terminal or software supports RTS

Send  	and sends the RTS signal.  OFF if the Courier is set to &R2 (Received Data hardware flow control) and the computer or terminal lowers RTS.

CS	Clear to Send	ON until the modem lowers CTS when Transmit Data hardware flow control is enabled (&H1, &H3).  Always ON during synchronous connections.

SYN	Synchronous	ON when the modem is set to &M1, &M6,

Mode	&M7 and enters synchronous mode.  Flashes when Dial Security is in operation.

ARQ/	Error Control/	Data Mode:  Automatic Repeat Request.  ON

FAX	Fax Operations	when the Courier is set to &M4 or &M5 and successfully connects with another modem under error control.  Flashes randomly when the Courier retransmits data to the remote modem.

Fax Mode:  Flashes steadily to indicate fax mode.

DIP SWITCH SUMMARY

PURPOSE

The DIP switches, located at the rear of the modem, are for adapting the modem to your equipment and personal requirements.  If necessary, review your software documentation.

Some users are able to move a single switch with a finger tip.  If this doesn't work for you, use a toothpick or other small instrument.  Do not use a lead pencil.

## OPERATIONS

The DIP switch settings are power-on defaults, read by the Courier when it is turned on.  If changed when the modem is on, switches 1-7, and 9 require the ATZ (software reset) command to initiate the new settings.  If you've set switch 8 OFF to disable command recognition, and want to return the modem to Smart mode so that it responds to commands, just reset switch 8 to ON.

When you issue the ATZ command, the modem reads its DIP switch settings and resets either to its defaults (DIP switch 10 OFF) or factory settings (DIP switch 10 ON).

Factory

Switch	Setting	Function

1	OFF	Data Terminal Ready Operations

OFF	Normal DTR operations:  computer must provide DTR signal for modem to accept commands; dropping DTR terminates a call

ON	DTR always ON (Override)

2	OFF	Verbal/Numeric Result Codes

OFF	Verbal (word) results

ON	Numeric results

Factory

Switch	Setting	Function

3	ON	Result Code Display

OFF	Results suppressed

ON	Results enabled

4	OFF	Command Mode Local Echo

OFF	Keyboard commands displayed

ON	Echo suppressed

5	ON	Auto Answer

OFF	Modem answers on first ring

ON	Auto answer disabled

6	OFF	Carrier Detect Operations

OFF	Courier sends CD signal when it connects with another modem, drops CD on disconnect

ON	CD always ON (Override)

7	OFF	Auxiliary, DIP Switch 3 ON

OFF	Result codes in Originate and Answer mode

ON	Result codes in Answer mode disabled

8	ON	AT Command Set Recognition

OFF	Command recognition disabled (Dumb mode)

ON	Recognition enabled (Smart mode)

9	OFF	Escape Code (+++) Response

OFF	Modem hangs up, returns to Command mode, sends NO CARRIER result

ON	Modem maintains connection, returns to Command mode, sends OK result

10	OFF	Power-on and ATZ Reset Software Defaults

OFF	Load from nonvolatile memory (NVRAM)

ON	Load factory settings from read only memory (ROM)

# DEFAULT SETTINGS

USER-PROGRAMMABLE DEFAULTS

You can create your own default configuration and store it in nonvolatile random access memory (NVRAM) using the &W command described in Chapter 4.  As long as DIP switch 10 is OFF when you power on the modem, your defaults are loaded into the modem's random access memory (RAM).  To view your NVRAM settings at any time, use the ATI5 command.

Tables on the next several pages list the options you can store in NVRAM, including S-Register settings.  If DIP switch 10 is ON at power-on, the factory template 0 settings are loaded instead.  The modem has four factory setting templates (&F0-F3).  By default, the first time the modem is turned on, the modem loads the settings stored in NVRAM, which are the same as the settings in factory template 1 (&F1).

The following command example substitutes several user-defined defaults for factory settings.  The modem also stores the rate, word length and parity it detects from the AT command prefix.

AT X1 &B0 &M5 &H0 M3 &W <Enter>

The modem is shipped with DIP switch 10 OFF, so when it is powered on it loads the settings from NVRAM.  Until these settings are changed, they are the same as the settings permanently stored in factory settings template 1 (&F1).  You can alter any of these settings, create your own power-on defaults, and then save them with the &W command.  See Chapter 4.

Table B.3¾&F1 Hardware Flow Control Default Template

NVRAM Options	Setting	Description

Handshake option	B0	ITU-T answer sequence

Error control/sync	&M4	Normal/error control

Data compression	&K1	Enabled

Transmit data hardware 	&H1	Hardware flow control

Rec'd data hardware flow control	&R2	Enabled

Rec'd data software flow control	&I0	Disabled

Serial port rate select	&B1	Serial port rate fixed higher than connect rate

Link rate select	&N0	Variable

Result code subset	X7	Extended.  Includes all codes 				except VOICE

Protocol response codes	&A3	Full protocol codes

Tone/Pulse dialing	P	Pulse dial

Online local echo	F1	Disabled

Speaker control       	M1	ON during dial through connect

Remote Digital Loopback (RDL)	&T5	Deny RDL

Normal/Leased/Cellular line	&L0	Normal phone line

Data Set Ready operations	&S0	Override enabled

Break handling	&Y1	Clear buffer, send immediately

Stored telephone number	&Z0-9=0	Blank

Pulse dial make/break ratio	&P0	U.S./Canada

Guard tone	&G0	U.S./Canada

Word length*	8

Parity*	0	None

DTE rate* (Kbps)	19.2	_

*  Detected by the modem from the AT prefix of the &W command that writes your defaults to NVRAM.  Set your software to the desired word length, parity, and serial port rate defaults before sending the modem the AT . . . &W string.

The &F2 and &F3 factory setting templates are similar to the &F1, with the exception of the commands which are highlighted in bold in tables B.4 and B.5.

Table B.4¾&F2 Software Flow Control Template

NVRAM Options	Setting	Description

Handshake option	B0	ITU-T answer sequence

Normal/error control/sync   	&M4	Normal/error control

Data compression	&K1	Enabled

Transmit data flow control	&H2	Software flow control

Rec'd data hardware flow control	&R1	Disabled

Rec'd data software flow control	&I2	Enabled

Serial port rate select	&B1	Serial port rate fixed higher than connect rate

Link rate select	&N0	Variable

Result code subset	X7	Extended.  Includes all codes 				except VOICE

Protocol response codes	&A3	Full protocol codes

Tone/Pulse dialing	P	Pulse dial

Online local echo	F1	Disabled

Speaker control       	M1	ON during dial through connect

Remote Digital Loopback (RDL)	&T5	Deny RDL

Normal/Leased/Cellular line	&L0	Normal phone line

Data Set Ready operations	&S0	Override enabled

Break handling	&Y1	Clear buffer, send immediately

Stored telephone number	&Z0-9=0	Blank

Pulse dial make/break ratio	&P0	U.S./Canada

Guard tone	&G0	U.S./Canada

Word length*	8

Parity*	0	None

DTE rate* (Kbps)	19.2	_

*  Detected by the modem from the AT prefix of the &W command that writes your defaults to NVRAM.  Set your software to the desired word length, parity, and serial port rate defaults before sending the modem the AT . . . &W string.

Table B.5¾&F3 HST Cellular Template

NVRAM Options	Setting	Description

Handshake option	B1	HST-mode/Bell answer tone

Normal/error control/sync	&M4	Normal/error control

Data compression	&K1	Enabled

Transmit data flow control	&H1	Hardware flow control

Rec'd data hardware flow control	&R2	Enabled

Rec'd data software flow control	&I0	Disabled

Serial port rate select	&B1	Serial port rate fixed higher than connect rate

Link rate select	&N0	Variable

Result code subset	X7	Extended.  Includes all codes 				except VOICE.

Protocol response codes	&A3	Full protocol codes

Tone/Pulse dialing	P	Pulse dial

Online local echo	F1	Disabled

Speaker control       	M1	ON during dial through connect

Remote Digital Loopback (RDL)	&T5	Deny RDL

Normal/Leased/Cellular line	&L2	HST Cellular

Carrier loss wait time	S10=30	Waits 3 seconds before hanging up

Data Set Ready override	&S0	Enabled

Break handling	&Y1	Clear buffer, send immediately

Stored telephone number	&Z0-9=0	Blank

Pulse dial make/break ratio	&P0	U.S./Canada

Guard tone	&G0	U.S./Canada

Word length*	8

Parity*	0	None

DTE rate* (Kbps)	19.2	_

*  Detected by the modem from the AT prefix of the &W command that writes your defaults to NVRAM.  Set your software to the desired word length, parity, and serial port rate defaults before sending the modem the AT . . . &W string.

If DIP switch 10 is ON when the modem is powered on, or you load factory template 0 (&F0), the following settings take effect.  The differences between factory template 0 (&F0) and factory template 1 (&F1) are noted in bold.

Table B.6¾&F0 No Flow Control Low PerformanceTemplate

NVRAM Options	Setting	Description

Handshake option	B0	ITU-T answer sequence

Normal/error control/sync	&M4	Normal/error control

Data compression	&K1	Enabled

Transmit data flow control	&H0	Disabled

Rec'd data hardware flow control	&R1	Disabled

Rec'd data software flow control	&I0	Disabled

Serial port rate select	&B0	Detect from AT command: variable rate

Link rate select	&N0	Variable

Result code subset	X1	Basic

Error-control response codes	&A1	Enabled

Tone/Pulse dialing	P	Pulse dial

Online local echo	F1	Disabled

Speaker control       	M1	ON during dial through connect

Remote Digital Loopback (RDL)	&T5	Deny RDL

Normal/Leased/Cellular line	&L0	Normal phone line

Data Set Ready operations	&S0	Override enabled

Break handling	&Y1	Clear buffer, send immediately

Stored telephone number	&Z0-9=0	Blank

Pulse dial make/break ratio	&P0	U.S./Canada

Guard tone	&G0	U.S./Canada

Word length*	7

Parity*	1	Even

DTE rate* (bps)	9600	_

*  Detected by the modem from the AT prefix of the &W command that writes your defaults to NVRAM.  Set your software to the desired word length, parity, and serial port rate defaults before sending the modem the AT . . . &W string.

Table B.7¾NVRAM Options

Factory

NVRAM S-Register Options 	Setting

S0	Auto Answer		1
S2	Escape code character		43

S3	Carriage Return character		13
S4	Line Feed character		10
S5	Backspace character		8
S6	Dial wait-time, sec.		2
S7	Carrier wait-time, sec.		60
S8	Dial pause, sec.		2
S9	Carrier Detect time, 1/10th sec.		6
S10	Carrier loss wait-time, 1/10th sec.		7
S11	Tone duration, spacing, msec.		70
S12	Escape code guard time, 1/50th sec.		50
S13	Bit-mapped functions*		0
S15	Bit-mapped functions*		0
S19	Inactivity/hang up timer		0
S21	Break length, 1/100th msec.		10
S22	XON character		17
S23	XOFF character		19
S24	Pulsed DSR duration, 2/100th sec.		150
S26	RTS/CTS delay time, 1/100th sec.		1
S27	Bit-mapped functions*		0
S28	V.32 handshake time, 1/10th sec.		8
S29	V.21 handshake time, 1/10th sec.		20
S32	Voice/Data Switch Options		9
S33	Bit-mapped functions*		0
S34	Bit-mapped functions*		0
S38	Disconnect wait time, sec.		0
S41	Allowable remote login attempts		0
S42	Remote Access ASCII character		126
S43	Remote guard time, 1/50th sec.		200
S44	Re-establish leased-line connect, sec.		15
S51	Bit-mapped functions*		0
S53	Bit-mapped functions*		0
S54	Bit-mapped functions*		0
S55	Bit-mapped functions*		0
S56	Bit-mapped functions*		0
S57	Bit-mapped functions*		0

*  Bit-mapped registers have up to eight functions.  See 
   descriptions later in this appendix or a briefer summary in the 
   Quick Reference Card.

S-REGISTER SUMMARY

USAGE

The default values are those users typically require.  Change the settings of an S-Register with the ATSr=n command, where r is the register and n is a decimal value from 0-255:

ATS13=8 <Enter>

The modem does not perform a value-range check.  Some values you select may not work with some equipment, and you'll have to readjust the settings.

Some registers (S13, S14, S15, S16, S27, S34) are bit-mapped (bits 0-7).  For example, turning on bit 0 of S13 causes the modem to reset each time the computer or terminal drops its Data Terminal Ready (DTR) signal.  Turning on bit 3 of S13 causes the modem, on receipt of DTR, to auto dial the number stored at position 0 in NVRAM.

To turn on one or more bits in any bit-mapped register, use the total of the values shown below.  For example, S13=9 turns on bits 0 (value of 1) and 3 (value of 8).

Alternatively, identify the bits to be turned on with the following format:  Sr.b=1, where r is the register and .b is the bit.  This format does not require knowledge of the bit's value.  S13.0=1 .3=1 is the equivalent of S13=9, above.  To turn off a bit function, set it to zero:  S13.0=0.

To display the contents of a register, use ATSr? as in this example:

ATS19? <Enter>

Register  	Default	Function

S0	See DIP	Sets the number of rings on which to

Switch 5	answer when in Auto Answer mode.  S0=0 disables Auto Answer, the same as DIP switch 5 ON (factory setting).  S0=1 enables Auto Answer and the modem answers on the first ring.

NOTE:  If DIP switch 5 is OFF and S0=0, 
Auto Answer remains disabled.

S1	0	Counts and stores the number of rings from an incoming call.

S2	43	Stores the ASCII decimal code for the escape code character.  Default character is “+”.  A value of 128-255 disables the escape code.

S3	13	Stores the ASCII decimal code for the Carriage Return character. Valid range is 
0-127.

S4	10	Stores the ASCII decimal code for the Line Feed character.  Valid range is 0-127.

S5	8	Stores the ASCII decimal code for the Backspace character.  A value of 128-255 disables the Backspace key's delete function.

S6	2	Sets the number of seconds the modem waits before dialing.  If set to X2, X4, X6, or X7, the modem dials as soon as it detects a dial tone (fast dials).  If there is no dial tone, the modem observes the normal S6 timeout.

S7	60	Sets the number of seconds the modem waits for a carrier.  May be set for much longer duration if, for example, the modem is originating an international connection.

Register  	Default	Function

S8	2	Sets the duration, in seconds, for the pause (,) option in the Dial command and the pause between command re-executions (> and A> commands).

S9	6	Sets the required duration, in tenths of a second, of the remote modem's carrier signal before recognition by the Courier.

S10	7	Sets the duration, in tenths of a second,  that the modem waits after loss of carrier before hanging up.  This guard time allows the modem to distinguish between a line hit, or other disturbance that momentarily breaks the connection, from a true disconnect (hanging up) by the remote modem.

While we do not recommend connecting the modem to a line with call waiting, if you have call waiting you may wish to adjust this setting upward to prevent the modem from misinterpreting the signal for a second call as a disconnect by the remote modem.  A better alternative is to contact your phone company to find out how to temporarily disable call waiting.

S11	70	Sets the duration and spacing, in milliseconds, of dialed tones.

S12	50	Sets the duration, in fiftieths of a second, of the guard time for the escape code (+++) sequence.

Register  	Default	Function

S13	0	Bit-mapped register.  Select the bit(s) you 
			want on and set S13 to the total of the 
			values in the Value column.  For example, 
			ATS13=20 enables bit 2 (value = 4) and bit 
			4 (value = 16).  Or use ATSr.b=0 (OFF) or 1 
			(ON).  For example, ATS13.0=1 .3=1 turns 
			on bits 0 and 3.   To turn a bit off, set that 
			bit to zero, as in ATS13.3=0.

Bit	Value	Result

0	1	Reset when DTR drops

1	2	Reverse normal Auto Answer operation:  on incoming RING, enter Originate Mode and look for Answer tone

2	4	Disable 250 msec. pause before result code display

3	8	On DTR signal, Auto Dial the number stored in NVRAM at position 0

4	16	At power on/reset, Auto Dial number stored in NVRAM at position 0

5	32	Disable HST (used for testing V.32 terbo in Dual Standard modems)

6	64	Disable MNP Level 3 (used for testing Level 2)

7	128	Custom applications

S14	0	Bit-mapped register.  Select the bit(s) you 
			want on and set S14 to the total of 
			the values in the Value column.

Bit	Value	Result

0	1	Disconnect on escape code

1–6	–	Reserved

7	128	Disable retrains

Register  	Default	Function

S15	0	Bit-mapped register.  To set the register, see the instructions for S13.

Bit	Value	Result

0	1	Disable the modem's extra high-frequency equalization if it causes problems on shorter-link calls¾HST-modems only

1	2	Disable online fallback

2	4	Disable 450 bps back channel¾ HST only

3	8	Reset non-ARQ mode Transmit buffer from 1.5K bytes to 128*

4	16	Disable MNP Level 4; retrans-
mitting the larger Level 4 data blocks may be a problem if you expect a great number of errors during a call

5	32	Set backspace key to delete

6	64	Some earlier 2400 bps MNP modems, not made by U.S. Robotics or Microcom, were not fully compatible with the MNP protocol.  If you have difficulty making a successful 2400 bps MNP connection with a remote MNP modem, it may be because of this incompatibility.  Set S15 to 64 and try again to make the connection.

7	128	Custom applications only

* The default 1.5K byte non-ARQ buffer allows data transfer with X- and Ymodem-type file transfer protocols without using flow control.

The 128-byte option allows remote users with slower modems to stop data you're transmitting from scrolling off their screens.  When remote users send your computer an XOFF (<Ctrl-S>) and you stop transmitting, the data in transit from your modem's buffer doesn't exceed the size of their screen.

Register  	Default	Function

S16	0	Bit-mapped test register.  To set the register, see the instructions for S13.  For information on testing, see Appendix H.

Bit	Value	Result

0	1	Analog Loopback

1	2	Dial test

2	4	Test pattern

3	8	Remote Digital Loopback

4	16	Reserved

5	32	Reserved

6	64	Reserved

S17	0	Reserved.

S18	0	Test timer for software-initiated loopback testing (&Tn), disabled when S18 is set to 0.  See Appendix H.  Used to set the duration of testing, in seconds, before the modem automatically times out and terminates the test.

S19	0	Sets the duration, in minutes, for the Inactivity Timer.  The timer activates when there is no data activity on the phone line and at the timeout the modem hangs up.  S19=0 disables the timer.

S20	0	Reserved.

S21	10	Sets, in 10-millisecond units, the length of Breaks sent from the modem to the computer or terminal.  Applies to ARQ mode only.

S22	17	Stores the ASCII decimal code for the XON character.

S23	19	Stores the ASCII decimal code for the XOFF character.

Register  	Default	Function

S24	150	Sets the duration, in 20-millisecond units, between pulsed DSR signals when the modem is set to &S2 or &S3.  The default is 3 seconds.

S25	5	Sets DTR recognition time in 1/100th of a second units.

S26	1	Sets the duration, in 1/100th of a second units, of the delay between RTS and the modem's CTS response in synchronous mode.

S27	0	Bit-mapped register.  To set the register, see the instructions for S13.

Bit	Value	Result

0	1	Enable ITU-T V.21 modulation at 300 bps for overseas calls.  In V.21 mode, the modem answers both Bell 103 and V.21 calls, but only originates V.21 calls.

1	2	Enable unencoded (non-trellis-coded) modulation in V.32 mode; this option is part of the ITU-T V.32 recommendation, but is rarely used.

2	4	Disable V.32 modulation; used for testing HST modulation in Dual Standard modems.

3	8	Disable 2100 Hz answer tone to allow two V.42 modems to connect more quickly.

4	16	See next page.

5	32	See next page.

6	64	Reserved.

Register  	Default	Function

7	128	Unusual software incompatibility.  Some software may not accept 7200, 12000, 14400, 16800, 19200 and 21600 bps result codes.  This setting disables the codes and displays the 9600 code instead.  The call's actual rate can be viewed on the ATI6 screen.

Error control handshaking options:  select the total values of bits 4 and 5.

Bit 4	Bit 5	Result

0	0	Complete handshaking sequence:  V.42 Detection, LAPM error control, MNP

16	0	Disable MNP

0	32	Disable V.42 Detection and LAPM

16	32	Disable Detection phase, if you know that the remote modem does LAPM, but not the Detection phase.

S28	8	Sets the duration, in tenths of a second, of the extra 3000/600 Hz answer tones sent during V.32 handshaking.  This gives V.32 modems additional time to connect in V.32 mode before timing out.

If there is difficulty answering older, manually operated V.32 modems, for example, modems that require a button to be pushed in order to dial, try lengthening the duration of the extra tones.

Register  	Default	Function

Setting S28 to zero eliminates the extra 
				tones, resulting in a faster connect time if, 
				for example, the modem is set to use V.21 
				modulation (300 bps) or V.23 modulation 
				(1200 bps).

S29	20	Sets the duration, in tenths of a second, of the answer tones sent during V.21 handshaking.  Default = 20 (2 seconds).  This gives V.21 modems additional time to connect in V.21 mode before timing out.

S30-S31	0	Reserved.

S32	9	Sets the function for the voice/data switch.  This is not a bit-mapped register.  Select the value for the desired function, for example, ATS32=6.

Value	Result

0	Disabled

1	Voice/data, Originate mode.  See value 9.

2	Voice/data, Answer mode

3	Redial last number

4	Dial number stored at 
position 0

5	Auto Answer on/off toggle

6	Reset the modem

7	Initiate Remote Digital Loopback

8	Busy out phone line toggle

9	Default if a command string is stored with the &ZC=string command.  When voice/data switch is pushed and &ZC has been enabled, modem executes stored command string.  Otherwise acts like value 1, Originate mode

NOTE:  Only use the voice/data switch 
			when the modem is in Command mode .

Register  	Default	Function

S33	0	Setting this register to 1 (S33=1) enables a reduced packet size.

S34	0	Bit-mapped register.  See instructions for S13.

Bit	Value	Result

0	1	Disable V.32 bis.  Used for troubleshooting; U.S. Robotics' Technical Support may require that you disable V.32 bis for testing purposes.

1	2	Disable the modem's enhanced, proprietary V.32 bis modulation.  Used for troubleshooting.

2	4	Disable the faster retrains that occur during proprietary V.32 terbo modulation.  Used for troubleshooting.

3	8	Enable V.23.  Required for some British connections.

4	16	Change MR LED to DSR.

5	32	Enable MI/MIC; see Appendix G.

6	64	Disable the remote access busy message.

7	128	Disable V.32 terbo.

S35-S37	0	Reserved.

S38	0	Sets the duration, in seconds, before a forced hang-up and clearing of the Transmit buffer, when DTR drops during an ARQ call.  This is provided to allow time for a remote modem to acknowledge receipt of all transmitted data.  Default = 0:  the modem immediately hangs up when DTR drops.  If the modem receives the ATH command, it ignores S38 and immediately hangs up.

Register  	Default	Function

S39-S40	0	Reserved.

S41	0	Sets the number of allowable remote access login attempts, thus enabling or disabling remote access.  The default setting of zero allows no remote login attempts, thus disabling remote access.  A value of 1 or greater enables remote access.  If the number of unsuccessful login attempts exceeds the limit set by this register, the modem returns online and any further login attempts during the remainder of that connection are refused.

S42	126	Stores the ASCII decimal code for the remote access escape character.  The default character is a tilde (~).

S43	200	Sets the duration, in fiftieths of a second, of the guard time for the remote access (~~~~) sequence.

S44	15	Sets the duration, in seconds, of the delay
between when the modem senses loss of carrier and when it attempts to re-establish a leased-line connection.

S45-S50	0	Reserved.

Register  	Default	Function

S51	0	Bit-mapped register.  See instructions 
			for S13.

Bit	Value	Result

0	1	Disable MNP/V.42 for
					V.22 (1200 bps)

1	2	Disable MNP/V.42 for
					V.22 bis (2400 bps)

2	4	Disable MNP/V.42 for
					V.32/V.32 bis/V.32 terbo 
					(9600/14,400/19,200/
					21,600 bps).

3-6	_	Reserved

7	128	Custom Applications.  
		Handset Exclusion Delay.  
		If the telephone and 
		modem share the same 
		line, and DTR is raised, 
		the modem takes control 
		of the phone line and 
		disconnects the voice call.  
		The modem stays on hook 
		for the duration specified 
		in S6 before it becomes a 
		data call.

Since there is no way to 
		detect if the telephone is in 
		use, the S6 delay occurs 
		even if the telephone is not 
		in use.

The phone is re-enabled 
		once the modem returns 
		on hook.

S52	Reserved.

Register  	Default	Function

S53	0	Bit-mapped register.  Select the Dial Security features you want enabled by setting S53 to the total of the values in the Value column in the table below.  For example, S53=3, enables Dial Security with prompting.  S53=5 enables Dial Security, and local-access password protection.  Or use ATSr.b=0 (OFF) or 1 (ON).  For example, ATS53.0=1 .2=1 turns on bits 0 and 2.   To turn a bit off, set that bit to zero, as in ATS53.2=0.

Bit	Value		Result

0	1	Dial security enabled

1	2	Prompting enabled

2	4	Local-access password
						protection enabled

NOTE:  In addition, enabling local access password protection disables the &Zn=s command which stores up to ten phone numbers because stored phone numbers occupy the same space in NVRAM as the dialback numbers for Dial Security accounts.

S54	0	Symbol rate bit-mapped register used 
			primarily by U.S.Robotics Technical 
			Support for debugging purposes.

Bit 6 (ATS54.6=1) disables Call Indicate 
			(CI).

Bit 7 (ATS54.7=1) disablesV.8.

Register  	Default	Function

S55	0	Trellis code bit-mapped register used 
			primarily by U.S.Robotics Technical 
			Support for debugging purposes.

S56	0	Bit-mapped register primarily used
			by U.S.Robotics Technical Support for 
			debugging purposes.

Bit 6 (ATS56.6=1) disables V.34 
			modulation.

Bit 7 (ATS56.7=1) disables V.FC 
			modulation.

S57	0	Reserved for German operations.

# ASCII cHART