CHAPTER 6.  QUERIES AND HELP SCREENS

user inquiries (In)

The Inquiry command has 11 options.  The most commonly used options display the following information:

ATI3 	Call duration

ATI4 	Current settings

ATI5 	NVRAM settings

ATI6	Link diagnostics summary

I0	The modem returns a 4-digit product code.  If you have a problem and call U.S. Robotics' Technical Support Department, you may be asked for this product code.

I1	The modem performs a checksum of its read-only memory (ROM) and returns the result to the screen.  This function is used only in factory testing.  The modem should always read the same number.

I2	The modem performs a test of its random access memory (RAM) and returns either the OK (0) or ERROR (4) result code, followed by OK when the test is completed.  You may want to use this command as a checkpoint if the modem appears to be malfunctioning.

I3	The modem returns the duration of the last call if set to K0.  It displays the actual time if set to K1.  See the description of the Kn command in Chapter 4.

I4	The modem displays its current configuration.  Figure 6.1 on the following page is an example.

ati4

USRobotics Courier Dual Standard V.34 Fax Settings...

B0  C1  E1  F1  M1  Q0  V1  X1

BAUD=57600  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK   TIMER

&A1  &B0  &C1  &D2  &G0  &H0  &I0  &K1  &L0  &M4  &N0

&P0  &R1  &S0  &T5  &X0  &Y1  %N6

S00=000  S01=000  S02=043  S03=013  S04=010  S05=008  S06=002  S07=060

S08=002  S09=006  S10=007  S11=070  S12=050  S13=000  S14=000  S15=000

S16=000  S17=000  S18=000  S19=000  S20=000  S21=010  S22=017  S23=019

S24=150  S25=005  S26=001  S27=000  S28=008  S29=020  S30=000  S31=000

S32=009  S33=000  S34=000  S35=000  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=126  S43=200  S44=015  S45=000  S46=000  S47=000

S48=000  S49=000  S50=000  S51=000  S52=000  S53=000  S54=000  S55=000

S56=000  S57=000

LAST DIALED #:

OK

Figure 6.1¾Sample Result of ATI4 Command

I5	The modem displays the configuration stored in nonvolatile random access memory (NVRAM), as in the following example.  If your modem connects to a modem that has Dial Security and local access enabled, you cannot view the stored phone numbers.

ati5

USRobotics Courier Dual Standard V.34 Fax NVRAM Settings...

DIAL=PULSE  B0  F1  M1  X7

BAUD=57600  PARITY=N  WORDLEN=8

&A3  &B1  &G0  &H1  &I0  &K3  &L0  &M4  &N0

&P0  &R2  &S0  &T5  &X0  &Y1  %N6

S00=001  S02=043  S03=013  S04=010  S05=008  S06=002  S07=060  S08=002

S09=006  S10=007  S11=040  S12=050  S13=000  S15=000  S19=000  S21=010

S22=017  S23=019  S24=150  S25=005  S26=001  S27=000  S28=008  S29=020

S31=000  S32=009  S33=000  S34=000  S35=000  S36=000  S37=000  S38=000

S39=000  S40=000  S41=000  S42=126  S43=200  S44=015  S51=000  S53=000

S54=000  S55=000  S56=000  S57=000

STORED PHONE NUMBERS

0:                                      1:

2:                                      3:

4:                                      5:

6:                                      7:

8:                                      9:

STORED COMMAND =

OK

Figure 6.2¾Sample NVRAM Settings Screen

I6	During a connection, the modem monitors and stores information about link operations.  When the call is ended, you can request a diagnostic summary, as in the following example.  The duration of the last call or real time is displayed depending on the Kn setting.

ati6

USRobotics Courier Dual Standard V.34 Fax Link Diagnostics...

Chars sent	0	Chars Received	0

Chars lost	0

Octets sent 	0	Octets Received	0

Blocks sent	0	Blocks Received	0

Blocks resent	0

Retrains Requested 	0	Retrains Granted 	0

Line Reversals 	0	Blers   	0

Link Timeouts 	0 	Link Naks   	0

Data Compression	 NONE

Equalization	Long

Fallback	Disabled

Last Call	00:00:00

Disconnect Reason is Keypress Abort

OK

Figure 6.3¾Sample Link Diagnostics Screen (ATI6)

For calls under data compression, the number of characters sent may be less than the number of octets sent, due to buffering operations.  Line Reversals only apply to HST-mode operations, when the modems switch the high and low speed channels.  At this time, online fallback is only reported Enabled in HST-mode.

Most terms used in the display are self-explanatory except for the following:

Octets:  Compressed data units.  If the number of octets is greater than the number of characters sent, the modems probably used MNP5 compression on an already compressed file, and the result was expanded data.

Line Reversals:  The number of times HST-mode modems switched the high and low speed channels.

Blers:  Errors in data and protocol blocks.  If there were many block errors, your receiver may have experienced problems on the line.

Blocks Resent:  These represent blocks the remote modem resent due to the previous category, Blers.

Link Timeouts:  Protocol detection problems:  communications were severed momentarily, and the modems probably recovered.  This does not indicate the retry timeout.

Link Naks:  Negative acknowledgments (one or more blocks).

Data Compression:  Indicates the type of data compression negotiated for the call (V42BIS or MNP5) or NONE.  A V42BIS response includes the size of the dictionary and the maximum string length used, for example, 2048/32.  See Appendix A for more information.

Equalization Long/Short:  Status of S15 bit 0; long if bit 0=0, short if bit 0=1.  Short equalization applies only to HST modems.

Fallback:  Enabled/Disabled:  indicates whether or not the modems negotiated online fallback during the connection sequence.

Protocol:  Indicates the error control protocol negotiated (LAPM, HST, MNP, NONE) or SYNC for a synchronous call.

Speed:  The last rates at which the receiver/transmitter were operating before disconnecting.

Disconnect Reason:  Possible reasons the modem hung up are as follows:

A Rootless Tree:  The modem received an invalid V.42 bis (compression) frame.

Break Timeout:  Incompatible processing of a Break signal occurred.

DISC:  The remote modem sent a V.42 Disconnect frame.

DTR dropped:  The computer or terminal dropped the Data Terminal Ready signal, terminating the call.

Escape code:  The operator sent the modem the +++ escape code.

Extra Stepup:  The modem received an invalid V.42 bis (compression) frame.

GSTN (General Switch Telephone Network) Clear Down:  The connection was non-ARQ and DTR was dropped from one side of the connection, or the DISC frame was corrupted due to noise.

Illegal Command Code:  The modem received an invalid V.42 bis (compression) frame.

Inactivity timeout:  The modem detected no activity on the line for the duration specified in Register S19 (default is 0, timer disabled).

Invalid Codeword:  The modem received an invalid V.42 bis (compression) frame.

Invalid speed:  The modem is set to &N1 or higher, for a fixed link rate, and the remote modem is not operating at the same rate.

LD received:  The remote modem sent an MNP error control Link Disconnect request.

Loop loss disconnect:  The modem detected a loss of current on the loop connecting it with the telephone company central office.  This usually occurs because the remote modem has hung up:  the central office drops current momentarily when there is a disconnect at the other end of a call.  Unless Register S38 is set higher than zero, the modem immediately hangs up at loop loss.

Loss of carrier:  The modem detected loss of the remote modem's carrier and waited the duration specified in Register S10 (default is 0.7 seconds).

MNP incompatibility:  The modem is set to &M5 and the remote modem does not have MNP capability, or there was an MNP negotiation procedure error.

Retransmit limit:  The modems reached the maximum of twelve attempts to transfer a data frame without error.

SABME Timeout (Set Asynchronous Balance Mode Extended):  The modems failed this part of V.42 link negotiation.

Unable to Retrain:  After several attempts, disturbances on the phone line prevented the modems from retraining, and they could no longer transmit or receive data.

XID Timeout:  The modems failed to negotiate the V.42 Detection (XID Exchange) phase.

Dial Security Disconnect Reason:  Possible reasons the answering modem may have hung up during a Dial Security session are as follows:

Security Abort:  The modem hung up because it received an invalid password three times.

Prompting Not Enabled:  The modem hung up because the originating modem did not send an autopass password, and prompting wasn't enabled.

No Prompting in Sync:  The originating modem did not send an autopass password, and the answering modem cannot prompt for a password in any synchronous mode.

Non-ARQ Mode:  The modem hung up because the originating modem was set for error control and the answering modem was set for non-error control.

Mode Incompatible:  The modem hung up because both modems were not set to the same error control setting.

No Prompting in Non-ARQ:  Prompting was enabled, but the modem hung up because the originating modem was set for error control, and the answering modem was set for non-error control.  The answering modem cannot prompt when it is set for non-error control.

I7	The modem returns a product configuration.  If you have a problem and call U.S. Robotics' Technical Support staff, you may be asked to read this screen.

I8	Not used

I9	Not used

I10	View Dial Security Account status.  For security administrators only, unless local security is disabled, S53=0 or S53.2=0.

ati10

USRobotics Courier Dual Standard V.34 Fax

DIAL SECURITY STATUS

DIAL SECURITY ENABLED:[N]	LOCAL SECURITY ENABLED:[N]
	PROMPTING ENABLED:[N]  	FORCED AUTOPASS:[N]
	LOCAL ACCESS PASSWORD:[NO PSW] 	AUTOPASS PASSWORD:[NO PSW]

ACCOUNT   PSW      ACCT/E    DIAL/B   NEW_#    PHONE #

#0     [NO PSW]               [N]               [N]           [N]

#1     [NO PSW]               [N]               [N]           [N]

#2     [NO PSW]               [N]               [N]           [N]

#3     [NO PSW]               [N]               [N]           [N]

#4     [NO PSW]               [N]               [N]           [N]

#5     [NO PSW]               [N]               [N]           [N]

#6     [NO PSW]               [N]               [N]           [N]

#7     [NO PSW]               [N]               [N]           [N]

#8     [NO PSW]               [N]               [N]           [N]

#9     [NO PSW]               [N]               [N]           [N]

OK

Figure 6.4¾Sample Dial Security Account Status Screen

I11	Connection report used for debugging purposes.  A U.S. Robotics Technical Support representative may ask you for information provided on this screen.

## S-Register Query (Sr?)

This command allows you to view the contents of a particular S-Register, as in the following example that requests the contents of Register S0 ("On what ring will the modem answer?"):

ATS0? <Enter>

## Phone Number Query (&Zn?)

At this command, the modem returns the phone number stored in NVRAM at position n, as in the following example that includes a sample modem response:

AT&Z3? <Enter>
5551234

## Last-Dialed Number query (DL?)

At this command the modem displays the number stored in the last-dialed number buffer:

ATDL? <Enter>

## Stored Command String Query (&ZC?)

At this command the modem displays the command string stored in NVRAM with the &ZC=s command:

AT&ZC? <Enter>

## HELP Screens

Courier modems provide five Help screens:  summaries of the basic AT command set, extended ampersand (&) command set, Dial command options, S-Register functions, and percent (%) command set.

NOTE:  The Help screens are not available when the modem makes a connection in synchronous mode:  &M1, &M6, or &M7.

### Stop/Restart Display

The following command stops the display. Hold down the Control key and type "S":

<Ctrl>-S

To restart the display, use the same command or press any key.

### Cancel Display

Either of the following commands cancels the display.

<Ctrl>-C

<Ctrl>-K

Basic Command Set ($)

At AT$, the Courier displays a screen that shows a partial summary of the command set.  A second screen, activated by pressing any key, shows the remaining commands.  The first screen is shown in Figure 6.5.

at$

HELP,  Command Quick Reference (CTRL-S to Stop, CTRL-C to Cancel)

&$	HELP, Ampersand Commands	 Kn	n=0  Call Duration Mode

%$	HELP, Percent Commands         		n=1  Real Time Clock Mode

A/	Repeat Last Command          	Mn	n=0  Speaker Off

A>	Continuously Repeat Command  	  	n=1  Speaker On Until CD

AT	Command Mode Prefix             		n=2  Speaker Always On

A 	Answer Call                                 		n=3  Speaker Off During Dial

Bn 	n=0  V.32 originate mode        	On	n=0  Return Online

n=1  HST originate mode             		n=1  Return Online & Retrain

Cn 	n=0  Transmitter Off                        		n=2  Return Online & Speed Shift

n=1  Transmitter On                   	P 	Pulse Dial

Dn 	Dial a Telephone Number        	Qn	n=0  Result Codes Sent

n=0..9#*TPR,;"W@!()-                    	 	n=1  Quiet (No Result Codes)

DL 	Dial Last Phone Number        		n=2  Verbose/Quiet On Answer

DSn	Dial Stored Phone Number         	Sr=n	Sets Register "r" to "n"

D$  	HELP, Dial Commands             	Sr?     Query Register "r"

En 	n=0  No Command Echo     	S$ 	HELP, S Registers

n=1  Echo Command Chars      	T	Tone Dial

Fn 	n=0  Online Echo                     	Vn 	n=0  Numeric Responses

n=1  No Online Echo                      		n=1  Verbal Responses

Strike a key when ready . .

Figure 6.5¾Sample Basic Commands HELP Screen

Extended Command Set (&$)

At AT&$, the Courier displays a screen that shows a partial summary of the extended ampersand command set.  A second screen, activated by pressing any key, shows the remaining command set.  The first screen is shown in Figure 6.6.

at&$

HELP, Ampersand Commands (CTRL-S to Stop, CTRL-C to Cancel)

&An 	n=0  Disable /ARQ Result Codes	&Nn	n=0  Highest Link Speed

n=1  Enable /ARQ Result Codes     		n=1  300 bps

n=2  Enable /Modulation Codes          	n=2  1200 bps

n=3  Enable /Extra Result Codes                	n=3  2400 bps

&Bn	n=0  Floating DTE Speed                                	n=4  4800 bps

n=1  Fixed DTE Speed                                    	n=5  7200 bps

n=2  DTE Speed Fixed When ARQ            	n=6  9600 bps

&Cn	n=0  CD Always On                                      	n=7  12000 bps

n=1  Modem Controls CD                            	n=8  14400 bps

&Dn	n=0  Ignore DTR                                             	n=9  16800 bps

n=1  On-Line Command Mode            	n=10 19200 bps

n=2  DTE Controls DTR                         	&Pn	n=0  N.American Pulse Dial

&Fn	n=0  Load Factory Configuration               	n=1  UK Pulse Dial

n=1  Hardware Flow Control Cnfg.   	&Rn	n=0  CTS Follows RTS

n=2  Software Flow Control Cnfg.              	n=1  Ignore RTS

n=3  HST/Cellular w/ HW FC Cnfg.               	n=2  RX to DTE/RTS high

&Gn	n=0  No Guard Tone                           	&Sn	n=0  DSR Always On

n=1  550 Hz Guard Tone                               	n=1  Modem Controls DSR

n=2  1800 Hz Guard Tone                               	n=2  Pulse DSR, CTS=CD

Strike any key when ready . . .

Figure 6.6¾Sample Ampersand Commands HELP Screen

Dialing (D$)

At ATD$, the Courier displays this Dial command summary:

atd$

HELP, Dial Commands (CTRL-S to Stop, CTRL-C to Cancel)

0-9	Digits to Dial

*	Auxiliary Tone Dial Digit

#	Auxiliary Tone Dial Digit

T 	Tone Dialing

P	Pulse Dialing

R	Call an Originate Only Modem

, 	Pause (Wait for S8 Time)

;	Remain in Command Mode After Dialing

" 	Used to Dial Alpha Phone #'s

W 	Wait for 2nd Dial Tone (X3-X7)

@ 	Wait for an Answer (X3-X7)

! 	Flash Switch Hook

OK

Figure 6.7¾Sample Dial Command HELP Screen

S-Register Functions (S$)

At ATS$, the Courier displays a screen that shows a partial summary of the S-Register functions.  More screens, activated by pressing any key, show the remaining registers.  The first screen is as follows.

ats$

HELP, S Register Functions (CTRL-S to Stop, CTRL-C to Cancel)

S0	Ring to Answer On	S33 Reserved

S1	Counts # of Rings 	S34 Bit Mapped

S2	Escape Code Char   		1 = Disable V32bis

S3	Carriage Return Char           		2 = Disable Enhanced V32 mode

S4	Line Feed Char                             		4 = Disable Quick V32 retrain

S5	Backspace Char               		8 = Enable V23 Fallback

S6	Wait Time/Dial Tone (sec)           		16 = Change MR to DSR

S7 	Wait Time/Carrier (sec)               		32 = Enable MI/MIC

S8	Comma Time (sec)                        		64 = Disable RA Busy Msg

S9	Carrier Detect Time (1/10sec)       		128 = Disable Terbo

S10	Carrier Loss Time (1/10sec)        	S35 Reserved

S11	Dial Tone Spacing (msec)       	S36 Reserved

S12	Escape Code Time (1/50sec) 	S37 Reserved

S13	Bit Mapped                                	S38 Disconnect Wait Time (sec)

1 = Reset On DTR Loss              	S39 Reserved

2 = Do Originate in Auto Answer	S40 Reserved

4 = No Pause Before Result Codes	S41# of Allowed Login Attempts

8 = Do DS0 On DTR                        	S42 Remote Escape Code Char

16 = Do DS0 On Reset                     	S43 Remote Escape Code Time (1/50sec)

Strike a key when ready . . .

Figure 6.8¾Sample S-Register HELP Screen

Percent Commands (%$)

At AT%$, the Courier displays a screen that shows a partial summary of the percent command functions.  A second screen, activated by pressing any key, shows the remaining registers.  The first screen is as follows.

at%$

HELP, Percent Commands (CTRL-S to Stop, CTRL-C to Cancel)

%An=	Security Account Information   	%Fn	Remote DTE Data Format

Command Structure                      	n=0  8, No parity

%An=	PW,ACCT E,DIAL B,NEW#,PH#           	n=1  7, Mark parity

n = (0-9)                                                       	n=2  7, Odd parity

PW = Password                                     	n=3  7, Even parity

ACCT E = Account Enable        	%L=PWn	Security Local Access Psw

DIAL B = Dial Back Enable         		PWn = (0-9)

NEW# = New Dial Back #      	%Nn	V.25bis Synchronous Clock Rate

PH# = Dial Back Phone #                	n=0  RESERVED

%Bn	 Remote DTE Data Rate                     	n=1  RESERVED

n=0    110  bps                               	n=2   1200 bps

n=1    300  bps                            		n=3   2400 bps

n=2    600  bps                                   	n=4   4800 bps

n=3   1200  bps                                         	n=5   7200 bps

n=4   2400  bps                                   	n=6   9600 bps

n=5   4800  bps                                           	n=7  12000 bps

n=6   9600  bps                                          	n=8  14400 bps

n=7  19200  bps                           	n=9  16800 bps

n=8  38400  bps                               	n=10 19200 bps

Strike a key when ready . . .

Figure 6.9¾Sample Percent HELP Screen