# CHAPTER 2.  Modem SET UP

Figure 2.1—Courier V.34 Modem

### Reminder

To prevent overheating, do not cover the vents on the top of the modem case.

## switches on the modem

### Voice/Data

This push-button switch is used primarily to switch between voice and data communications during a call.  Detailed instructions are in the Voice/Data Calls section in Appendix G.

Volume Control

This is a slide switch underneath the right side panel of the modem, near the front corner.  Sliding it toward the front of the modem increases the modem speaker’s volume; sliding it toward the rear of the modem decreases the volume.

## Front panel indicators on the modem

The modem has twelve status lights, or LEDs.  See Appendix B for descriptions of their operations.

## PACKAGE components

Your Courier modem package contains the following items:

•	The modem you purchased:  Courier V.34

•	An RJ11C phone cord

•	A power adapter

•	Fax software and manual

•	A Quick-Reference card

## What You Need

The Courier modem has minimal operational requirements.  Be sure to read the information in the front of this manual about radio and television interference and connecting to the phone company.  In addition, you should be aware of the following requirements.

An RS-232 Cable

You need an RS-232 cable to connect the modem to your computer or terminal.  Use a shielded cable to ensure minimal interference with radio and television reception.

NOTES:

•	Refer to Appendix B for a listing of RS-232 pin assignments required to operate the modem.  Be sure to check the appendix if you're not sure what type of cable you need, or if you're building your own.

•	If your machine has other than a 25- or 9-pin port, check your computer documentation or consult your dealer to find out what type of RS-232 connector is required.

WARNING:  If you're planning to use the high speed computer-to-modem rates of 115.K, 57.6K or 38.4K bps, follow the instructions concerning the RS-232 cable in Appendix B.  The guidelines there will help you to avoid signal degradation at very high speeds.

Communications Software. . .

. . .if the modem is attached to a computer instead of a terminal.

The software uses the modem's AT command set to control many communications functions, including configuring the modem, dialing, and answering calls, and also enables the transfer of files and other operations.

Some users prefer their communications software to take control of the modem, and are more comfortable with a program that makes the modem almost transparent.   Others prefer a program that allows them to use the modem's AT command set sometimes, and their software at other times, depending on the task at hand.

Review Chapter 4 so that you have a basic understanding of the modem's requirements and operation.

Telephone Adapter. . .

. . .if you have an older telephone installation that does not have the appropriate modular wall jack and plug.

Adapters and RJ11C connectors are available from your telephone company or computer dealer.

## You Should. . .

Refer to Appendix F. . .

. . .if your phone line is user-installed or if it is leased from the telephone company.

Refer to Appendix G. . .

. . . if your modem is installed in a Hewlett Packard system that uses the Ack/Enq communications protocol.

## Installing the Modem

1.	Turn off the computer or terminal and its peripheral devices.

2.	Examine the label on the bottom of the modem.  In addition to the summaries and other information, the label contains icons to aid in modem installation.

4.	Make sure that the modem’s power switch is OFF; press it towards the zero in the 0/1 icon on the bottom label.

5.	Plug the power supply adapter's small connector into the power jack at the back of the modem.  Plug the adapter into a standard 115-volt AC wall socket.

6.	Disconnect your present phone cable from the wall jack.  Plug one end of the supplied phone cable into the modem's phone jack (refer to the bottom label).  Plug the other end into the wall jack.

If you want to keep your telephone connected for conventional calls, plug its cord into the modem’s telephone jack (refer to the bottom label).

7.	Check the positions of the bank of Dual In-Line Package (DIP) Switches located in the well at the bottom of the modem.  These switches are set at the factory to the positions most users require.

Figure 2.2—DIP Switch Factory Settings

Check your software documentation for its requirements, particularly for DIP switches 1, 4, 5 and 6.  You'll also find descriptions of switch functions and options in Appendix B.

NOTE:  If you have built your own RS-232 cable and it does not support the Data Terminal Ready (DTR) signal, set DIP switch 1 ON, for DTR override.  The override causes the modem to operate as if the DTR signal were always ON, and enables the modem to accept commands.  Most purchased communications software, however, requires “normal” DTR.

8.	The final step is to connect the modem to the computer's or terminal's serial port with the RS-232 cable.  Attach the appropriate connectors to the modem and to the serial port.

Figure 2.3—Cabled Courier Modem

## Testing the Installation

Use the following procedures to verify that your modem is working properly.

1.	Turn on your computer or terminal.  Then turn the Courier's power switch ON.  The following front panel indicators, or LEDs, will light up on the modem.

CD	Carrier Detect, if you have set DIP switch 6 ON, enabling the CD override

TR	Data Terminal Ready, if you have set DIP switch 1 ON, enabling the DTR override

MR	Modem Ready/Power ON

CS	Clear to Send

2.	If you're using a personal computer, load your software to start the program.  Set your terminal or software to 19.2K bps or 9600 bps.  In addition, set a word length of either 7 bits plus 1 parity bit, or 8 bits with no parity—it doesn't matter which at this time—and 1 Stop bit.  Set the software to use the correct serial port (port through which the modem connects to the computer).

3.	Perform the function that lets you send AT commands to the modem, that is, puts the computer in Terminal mode.  Some communications programs do this automatically upon loading.  Others require you to display a communications or terminal screen, type a Function key, or perform some other operation.

Review your communications software documentation for instructions.

4.	Send the following attention command.  Type either upper or lower case letters, not a combination, and then press the Enter (Carriage Return) key, shown in the example below between angle brackets.  (Don't type the angle brackets.)

AT <Enter>

If everything is correct, the modem responds as follows:

OK

Go on to step 5.

Troubleshooting

If your entered command is not displayed, your local echo is OFF.  To turn the local echo ON, send the modem the following command:

ATE1 <Enter>

If double characters appear on the screen, both your modem and software are set to local echo ON.  Either set your software to local echo OFF, or turn the modem's echo OFF with the following command:

ATE0 <Enter>

If no OK appears on your screen after you completed Step 4, review the following checkpoints:

a.	Make sure you type all upper or lower case letters and press <Enter>.

b.	Check to see that you set your communications software to the correct serial port.  The correct serial port is the port through which the modem is connected to the computer.

c.	Make sure your software has put the computer in Terminal mode, so that you can send the modem commands.  Then review Step 4, on the previous page.

d.	Be sure that DIP switches 1 and 6 are set ON or OFF according to your terminal or software requirements.  The table in Appendix B explains each function, and you may also need to review your terminal or communications software documentation.

e.	If you set DIP switch 8 OFF, for Dumb mode, reset the modem to Smart mode:  set DIP switch 8 to ON.

f.	The modem is shipped with DIP switch 3 ON, enabling the result codes.  If DIP switch 3 is OFF, set it to the ON position.  Then initiate the new switch setting with the following reset command:

ATZ <Enter>

5.	As a final check, make sure the modem gets a dial tone.  Type the following Dial command:

ATD <Enter>

On receipt of the command, the modem goes off hook and waits for a dial tone.  The OH indicator lights up, and you'll hear the dial tone from the modem's speaker.  To cancel the operation, press any key.

Troubleshooting

If you don't hear the dial tone, first increase the volume by sliding the volume control switch towards the front of the modem.  If that doesn't work, check to see that the phone cable from the wall jack is connected to the correct jack on the modem (See Figure 2.3).  If necessary, reconnect the phone cable correctly.  Then try the Dial command again, ATD <Enter>.