# appendix A.  link negotiation (handshaking)
AND eRROR cONTROL

This appendix includes information on how U.S. Robotics modems  negotiate with remote modems for the rate and other characteristics of each connection.   In addition, you'll find information on error control and, especially useful, some statistics and guidelines on using the modem for the best throughput.

Some of the following text includes the term ARQ, which means Automatic Repeat Request.  ARQ is a method used in many error control protocols to ensure that any data that has been corrupted in transit is retransmitted.  We use the term in our documentation to designate a connection under error control.

NOTE:  High speed calls are highly vulnerable to errors unless the data is protected by error control.  The V. protocol operations described below take place even if one of the modems is not set for error control, thereby prohibiting error control for the call.  If your modem connects with a modem at high speed but without error control, and if you are not using an error control protocol for your call, you may lose data.

Dual Standard modems in HST mode, in contrast, are unable to connect with other HST modems at higher than 2400 bps, unless error control is negotiated for the call.

## V.34 handshaking

The Courier defaults to V.34 and tries for the highest possible speed when it attempts to connect with another modem, 28.8K bps.  The entire V.FC range comprises 28.8K, 26.4K, 24K, 21.6K, 19.2K, 16.8K, 14.4K, 9600, 7200, 4800, and 2400 bps.   If the remote modem is not V.34 capable, a connection is made using the highest compatible modulation scheme (V.FC, V.32 terbo, V.32 bis, and so on, down to as low as Bell 103, or 300 bps).

If the remote modem has V.34 capability, the two modems use a line probing technique to determine the highest speed possible under current line conditions, and complete the connection.  If the remote modem does not have V.34 capability, a calling Courier modem listens to the other modem's answer tones to identify what standard rate the remote modem is operating at, and adjusts to that rate.  An answering Courier modem sends out a series of answer tone signals until both modems can negotiate the best connection rate.

## V. Fast Class (V.fc) handshaking

The Courier defaults to V.Fast Class and tries for the highest possible speed when it attempts to connect with another modem, 28.8K bps.  The entire V.FC range comprises 28.8K, 26.4K, 24K, 21.6K, 19.2K, 16.8K, and 14.4K bps.   If the remote modem is not V.FC capable, a connection is made using the highest compatible modulation scheme (V.32 terbo, V.32 bis, and so on, down to as low as Bell 103, or 300 bps).

If the remote modem has V.FC capability, the two modems use a line probing technique to determine the highest speed possible under current line conditions, and complete the connection.  If the remote modem does not have V.FC capability, a calling Courier modem listens to the other modem's answer tones to identify what standard rate the remote modem is operating at, and adjusts to that rate.  An answering Courier modem sends out a series of answer tone signals until both modems can negotiate the best connection rate.

## U.S. Robotics V.32 terbo to U.S. Robotics V.32 terbo

On these connections, Couriers have two features that result in outstanding performance:  Quick Connect and Adaptive Speed Leveling (ASL).

Quick Connect allows the modems to connect in approximately 7 seconds, a far shorter time than normal with most modems.

ASL (described below in Other V.Protocol Operations) is used by U.S. Robotics modems operating in V.32 terbo and V.32 bis modes.

Other V. Protocol Operations

Earlier, lower-speed V. protocols do not employ line probing.  Instead they use predefined answer tones to specify, or identify, speed capabilities.  These protocols define the following maximum speeds.

·	V.32 terbo:  19.2K bps, with an additional Courier-to-Courier speed of 21.6K bps.

·	V.32 bis:  14.4K bps.

·	V.32:  9600 bps.

ASL (used in V.32 terbo and V.32 bis modes) is a strategy that allows the modems' receivers and transmitters to act independently of each other.  We have always featured a fallback/fall forward feature with error-correcting modems that allows them to slow down if there are problems with the phone line, in order to avoid data errors, and then speed up again.  But the independence of the receiving and transmitting channels means that one channel or the other may slow down and then speed up, without affecting the data flow on the other.  The result is more efficient line operation.

WARNING:  Answering V.32 terbo modems shift their serial port rate up to 38.4K (for 21.6K connections) if the calling V.32 terbo modem dials in at 21.6K.  The answering V.32 terbo modem then sends data to its computer at 38.4K bps.   V.32 bis modems shift their serial port rate to 19.2K bps in order to make 14.4K bps connections.  If your ocomputer or terminal does not support these higher serial port rates, you need to disable V.32 terbo and/or V.32 bis.  See Appendix B, Register S34.

NOTE:  While many modems on the market now use the more efficient speeds, there may be a problem in answering older, "dumb" V.32 modems at 9600 bps.  Register S28 is used to modify the duration of the extra tones used in V.32 negotiations, in the rare instance that this may be necessary.  See S-Registers, in Appendix B.

Dual Standard Handshaking

We  recommend that Dual Standard modems be set to retain the default B0 and &N0 settings.  This allows them to connect with V. protocol modems and HST modems in both Originate and Answer modes at a variety of speeds.

An originating Dual Standard modem set to B1 sends out a Bell answer tone, which is the prevalent standard in the U.S. and Canada for connections at 2400 bps and lower.   At higher speeds, the Dual Standard also recognizes the ITU answer tones necessary for connecting with V. protocol modems, and adjusts to the answering modem.

However, in Answer mode, a Dual Standard modem sending out the Bell answer tone (B1) won't be recognized by V. protocol modems.  The calling V. protocol modem, instead, will wait until it detects a tone it recognizes, the V.22 bis tone used at 2400 bps.   If you wish to have your Dual Standard connect with V. protocol modems at high speeds, make sure it is set to B0 for the ITU answer tones.  It will also connect with HST modems at speeds up to 16.8K bps.

## error control and throughput

### Overview

Error control is available for calls at 1200 bps and above.  It can be disabled, although high speed calls (above 2400 bps) should always be under error control.  The operations defined in an error control protocol include the following:

•	Establishment of compatibility

•	Data frame formatting

•	Error detection through Cyclic Redundancy Checking (CRC)

•	Retransmission of corrupt data frames

The Courier is set at the factory to &M4, causing it to try for an error control connection and, if that isn't possible, to proceed with the call in Normal mode.  The modem first tries for a V.42 connection, then an MNP connection.  The following information is based on the Courier's setting of &M4.

V.42 Handshaking

This international standard includes a two-stage handshaking process:

•	A Detection phase that is based on an exchange of predefined characters.

•	LAPM (Link Access Procedures for Modems) Negotiation.  In this phase, the modems identify their capabilities concerning maximum data block size and the number of outstanding data blocks allowed before an acknowledgment is required.

MNP Handshaking

This protocol is supported by the ITU-T V.42 Recommendation.  It was originally developed by Microcom, Inc. and is now in the public domain.

MNP is based on special protocol frames.  If the remote modem doesn't recognize an MNP Link Request, error control isn't possible. (In HST asymmetrical mode, U.S. Robotics modems use a proprietary scheme similar to MNP.)

Data Compression

If the modems successfully establish a V.42 connection, they also negotiate for V.42 bis data compression.  If they successfully establish an MNP connection, they negotiate for MNP5 data compression.  The type of compression for a call, if any, is reported in the ATI6 display, and in the CONNECT message if the modem is set to &A3.

Modems using V.42 bis compression negotiate the following options and report them in the ATI6 display.

•	Dictionary size, that is, the amount of memory available for compression table entries.  (Entries are codes devised for redundant data.  The data is packed into shorter data units, called code words, and unpacked by the receiving modem.)

Possible sizes are as follows:

Bits	Entries

9	512

10	1024

11	2048

U.S. Robotics modems use 11-bit, or 2048-entry dictionary, but drop down if the remote modem uses a 9- or 10- bit dictionary.  The size of the dictionary for a call is reported in the ATI6 display.

•	Maximum string length of each entry.  As the dictionary fills, the modem deletes the oldest unused strings.

V.42 bis compression is more efficient than MNP5 compression in part because it dynamically deletes entries that are no longer used.  In addition, it works better with files that are already compressed.  These include .ZIP files downloaded from many Bulletin Boards and 8-bit binary files, which seem to the modem to be compressed.

MNP5 compression should not be used with such files because it adds data to them, which lessens throughput.  (The additional data is stripped when the file is decompressed by the remote modem.)  When transferring such files, it's best to set the modem to &K3:  this allows V.42 bis compression to work dynamically with the compressed data, but disables MNP5.

Flow Control

Flow control of data from the computer is required under error control for two reasons:

1.	The transmitting modem buffers a copy of each frame it transmits to the remote end until it is acknowledged by the receiving modem.

2.	If errors are encountered, retransmission activity can cause a steady stream of data from the computer to overflow the buffer.

Throughput Guidelines

The following guidelines should help to make the most of the modem's advanced performance features.  In many instances, experimentation and experience will indicate what works best for your applications.

1.	Optimal throughput is attained under the following conditions:

•	The communications software allows fixing the serial port rate higher than the connection rate, by setting the software to 115.2K, 57.6K, or 38.4K bps and setting the modem to &B1.

If the software automatically switches serial port rates to follow the connection rate, the modem's serial port rate must be also set to follow the connection rate for each call, &B0, and throughput will be limited.

Installations with specialized software may want to enable a fixed serial port rate for ARQ calls and a variable serial port rate for non-ARQ calls.  See the &B2 command in Chapter 4.

•	The call is under data compression.

•	The data is comprised of text files rather than binary files such as .EXE or .COM files.  See the table at the end of this appendix.

2.	MNP5 compression is disabled for files that are already compressed, and 8-bit binary files that appear to the modem to be already compressed.  MNP5 is disabled by setting the modem to &K3.

3.	The file transfer is not slowed down by a file-transfer protocol.  Many non-text files require a file transfer protocol, but the results vary.  For example, certain public domain file transfer protocols have the following effects:

Kermit	Newer versions support packets up to 9K and a sliding window design to eliminate turn-around delay.  With earlier versions, however, throughput may be severely reduced due to short block lengths (possibly under 128 bytes) and acknowledgment turnaround time.

Xmodem	Throughput may be reduced if your version uses short block lengths (128 bytes).  Some versions use larger blocks (1K blocks).  Throughput is also reduced by overhead (error control protocol information).

Ymodem	There is an improvement over Xmodem, due to larger block lengths (1K bytes), but throughput is still reduced by the protocol's error control overhead.

The above protocols further reduce throughput when an error control connection is established.  The accuracy of the data is checked both by the file transfer protocol and the modem.  To avoid redundancy, use the above protocols only for non-ARQ connections, and only at speeds of 2400 bps and below.

For the best throughput, but on error-controlled connections only and with hardware flow control, we recommend the most current version of Zmodem.  Overhead is minimal with this protocol, with throughput almost equal to that obtained with no file-transfer protocol.  Zmodem should also be used for non-ARQ connections.  Leave the modem at its &M4 and &K1 settings for both error control and data compression.  Ymodem-G is another good choice, but never without both the local and remote modems using error control:  if Ymodem-G detects an error, it aborts the transfer.  Do not use either protocol with software flow control (XON/XOFF signaling).

Typical Throughput

The maximum connection rate between two V.34 modems is 28.8K bps.  Ocassionally, connections occur at 26.4K, 24K, and 21.6K bps because line quality differs from location to location  Line conditions and data rate affect throughput.  Also remember, your serial port rate must match or exceed your connection rate.  If you set your serial port rate at 19.2K bps, the V.34 modem will only connect at or below 19.2K bps.

The tables below indicate the typical throughput, in characters per second (cps), that can be expected under the following conditions.

•	Connection (link) rates of 14.4K, 21.6K, and 28.8K bps, respectively

• 	Serial port rates set at 57.6K bps for the 14.4K bps connection and 115.2K bps for the 21.6K and 28.8K bps connections

•	Modem set to &B1 (fixed serial port rate)

•	V.42 bis compression negotiated for the call, and the default size 11-bit, 2048-entry dictionary

•	Straight data (not already compressed, no file-transfer protocol)

•	Transmission from a fast (486) computer

NOTE:  .ZIP files that are already compressed or files that appear to the modem to be compressed yield lower throughput.  We recommend setting the modem to &K3 when transferring these files, to allow V.42 bis but disable MNP5.

### 14.4K bps

File Type	Typical throughput (cps)

Text file	3400

.ZIP files 	1600

Database files	4600

Graphic files	2900

### 21.6.4K bps

File Type	Typical throughput (cps)

Text file	5100

.ZIP files 	2400

Database files	7200

Graphic files	4300

### 28.8K bps

File Type	Typical throughput (cps)

Text file	6800

.ZIP files 	3200

Database files	9600

Graphic files	5800