# CHAPTER 1.  FEATURES AND COMPATIBILITY

INTRODUCTION

Congratulations!  The Courier modem you've purchased represents powerful, advanced electronic design that offers exceptional reliability, compatibility and flexibility at all standard rates up to 28,800 or 28.8K bits per second (bps).  The transmission rate between the computer and modem, depending on your equipment and software support, can be as high as 115.2K bps.

Features

The following features and capabilities assure you of superior reliability and performance.

### Connections up to 28.8K bps

With the V.34 standard and the V.Fast Class modulation scheme, two modems can connect at rates up to 28.8K bps, twice as fast as the rates of standard ITU-T (formerly CCITT) V.32 bis modems, which are limited to calls of 14.4K bps or less.

### Software Upgrades

Courier high speed modems are now software upgradable, allowing you quick, easy access to the latest advances in data communication technology.  See Appendix I.

### Quick Connect

In V.32 terbo mode, Courier high speed modems can handshake and start transmitting much faster than typical training times that range between 9 and 18 seconds for other modems.

### Universal Connect

Courier high speed modems automatically detect and connect at the fastest available speed.

### Adaptive Speed Leveling (ASL)

Like most high speed modems, Courier modems fall back to the next lower speed—for example, 19.2K, then 16.8K in V.terbo mode—if poor line conditions warrant.  In addition, Courier V.32 bis, and V.32 terbo modems detect improved line conditions and shift upward again to the next higher speed.  both transmit and receive channels adapt independently, each detecting and adjusting to line conditions.  ASL keeps the modems online, always operating at the highest possible speed, and constantly ensuring data integrity.

### Fax Capability

You can use your modem with Class 1 or Class 2.0 facsimile software to exchange faxes with millions of Group III fax machines worldwide.  See Chapter 5.

### Error Control¾V.42/MNP

Data integrity is ensured when the modems connect with remote modems that use the V.42 (LAPM), HST, or MNP error control protocols.  Error control is available on calls at 1200 bps and above.

### Data Compression¾V.42 bis/MNP5

Data compression enables potential throughput of up to 115.2K bps on 28.8K  bps connections.  Couriers connecting under V.42 or HST error control use V.42 bis compression.  Couriers connecting under MNP error control use MNP Level 5 compression.  Typical throughput of text and other types of files using V.42 bis is provided in Appendix A.

### Flow Control/Variable Serial Port Rates

Flow control, required under error control, also allows the local serial port (DTE) rate to be set higher than the link (connection) rate, enabling greater efficiency and throughput.  If your equipment and software support high rates, data can be sent from the computer to the modem at 115.2K, 57.6K, 38.4K or 19.2K bps, regardless of the link rate.

### Voice/Data Switch

A switch on the modem's front panel allows you to change from voice to data and back again, without issuing a command.  See Appendix G.

### Stored Command String

If you don't ordinarily use voice and data in the same call, you can assign the voice/data switch a different function that normally requires a command, such as resetting the modem or executing a stored command string.  See Voice/Data in Appendix G for instructions on use of the voice/data switch.

### Asynchronous/Synchronous Capability

Courier high speed modems operate synchronously as well as asynchronously.  A personal computer equipped with a synchronous interface adapter can call computers that use standard synchronous protocols.  See Appendix F for more information.

### Synchronous Operations with V.25 bis

Used with computers that have a synchronous card and port, Courier high speed modems combine the power of older modems and automatic calling units in the mainframe environment.  They make synchronous connections by utilizing V.25 bis, character-oriented (similar to BISYNC) and HDLC protocols.  See Appendix F for more information.

### Dial Security

With Dial Security, you will be able to prevent unauthorized access to a system with the use of Autopass, Prompting, and Dialback.  See Appendix D for more information.

### Programmable Nonvolatile Memory

You can tailor your own default settings and store them in nonvolatile random access memory (NVRAM).  Each time the Courier is powered on or reset, it operates at the settings you've specified.  See Chapter 4, Appendix B, and Appendix C.

### Link Rate Negotiation

The Courier automatically lowers its link rate to match a lower rate of a remote modem, in both Originate and Answer Modes, allowing connections with a wide range of installed modems.

### Link Diagnostics

After each call, you can display a Link Diagnostics screen containing information about the last call, including the number of data characters transferred, line statistics, the call's rate and the reason the call was disconnected.  See Chapter 6.

### Modem Diagnostics

ITU-T V.54 loopback testing with the &T command options, and earlier Courier Register S16 test options are available.  The modem performs three loopback tests:  analog, digital, and remote digital.  See Appendix H for information.

### Inactivity Timer

You can set the modem to automatically hang up after a specified number of minutes if there is no activity on the phone line.  See Register S19 in Appendix B.

### Call Duration Reporting

The modem records the duration of your calls in hours, minutes, and seconds.  This feature enables you to display and print an audit of your calling activities.  You can optionally use the modem clock as a real-time clock.  See the I3 and I6 screens in Chapter 6 and the K command in Chapter 4 and Appendix C.

### Call Progress Detection

An optional set of result codes (screen messages) lets you know when a line is busy, a person rather than a modem has answered the phone, there is no dial tone, or the distant phone is ringing.

### Modem Settings Displays

On command, the modem displays its current settings, a handy way to check your transmission rate, S-Registers and other operational controls.  The modem also displays the defaults stored in nonvolatile memory as well as its default configuration templates.  See Chapter 6.

### HELP Screens

The modem displays screens that summarize the command sets, Dial command options, and S-Register functions.  See Chapter 6.

### Bottom Panel Reference

Command summaries and other information are printed on the bottom of the modem case.  A Dual In-Line Package (DIP) switch guide makes it easy to tailor the switch settings to your terminal or software requirements.

### Dialing the Last-Dialed Number

The modem has a buffer that stores each dialed number until it is cleared by another Dial command.  A few keystrokes cause the modem to redial the number in the buffer without your having to enter the number again.  See the DL command in Appendix C.

### Automated Redialing

You can put the modem into Repeat Mode to continuously redial if a previous dial attempt fails to connect.  This is especially useful in dialing services whose lines are often busy.  See Chapter 4.

### Quote Mode

Set the modem to Quote Mode if you want it to dial an alphabetic acronym instead of a numeric number.  See Chapter 4.

COMPATIBILITY

The Courier adheres to the following modulation schemes and standards, ensuring compatibility with a wide base of installed modems.  Unless otherwise indicated, Dual Standard V.34 modems conform to the listed standards.

NOTE:  The International Telecommunication Union (ITU-T) was formerly the International Telegraph and Telephone Consultative Committee (CCITT).

ITU-T V.34	28.8K/26.4K/24K/21.6K/19.2K/16.8K/14.4K/12K/ 
9600/7200/4800/2400 bps

V.FC	28.8K/26.4K/24K/21.6K/19.2K/16.8K/14.4K bps

V.32 terbo	21.6K/19.2K/16.8K/14.4K/12K/9600/
7200/4800 bps

HST	16.8K/14.4K/12K/9600/7200/4800 bps (Dual Standard modems in HST mode with ASL up to 21.6K bps)

HST Cellular	Cellular connections at 12K/9600/7200/ 4800/2400/1200/300 bps  (Dual Standard modems in HST mode)

ITU-T V.32 bis	14.4K/12K/9600/7200/4800 bps

ITU-T V.32	9600/4800 bps

ITU-T V.22 bis	2400 bps

Bell 212A	1200 bps (also V.22)

ITU-T V.23	1200 bps with 75 bps back channel (some U.K. and European phone systems)

ITU-T V.25 	Answer sequence for calls originating outside the U.S. and Canada

ITU-T V.25 bis	For synchronous communications using HDLC and character-oriented protocols

Bell 103 	300 bps (ITU-T V.21 optional)

ITU-T V.42	LAPM error control, 1200 bps and higher

ITU-T V.42 bis	Data compression, 1200 bps and higher

MNP	Levels 2, 3 and 4 error control, level 5 data compression, 1200 bps and higher

ITU-T V.54	Analog, digital and remote digital loopback testing

Fax Standards

The Courier modem provides Group III-compatibility when combined with Class 1 or Class 2.0 fax software.  In addition, the modem adheres to the following standards.

TIA/EIA-578	Service Class 1 Asynchronous Facsimile DCE Control Standard

TIA/EIA-592		Service Class 2.0 Asynchronous Facsimile 
	DCE Control Standard

ITU-T V.17		14.4K/12K bps

ITU-T V.29		9600/7200 bps

ITU-T V.27 ter		4800/2400 bps

ITU-T V.21 		300 bps

Other Compatibility Features

Other compatibility features include the following standards and certification:

•	Can be used with any computer or terminal that is compatible with the RS-232 standard interface.

•	Can be used with any computer or terminal that uses ASCII, the standard character code supported by most equipment manufacturers.

•	Is fully FCC- and IC-certified for the uses described in this manual.