# Appendix F.  Synchronous AND 
DEDICATED AND LEASED LINE OPERATIONS

SYNCHRONOUS APPLICATIONS

Synchronous mode is required for users who need to call, or receive calls, from a Host computer of a large network.  The Host is usually a mainframe.  There are two ways the Courier can operate in synchronous mode:

•	Selecting the ITU-T standard V.25 bis protocol, which formats data in HDLC or character-oriented frames.  This method is used by mainframe operators and synchronous terminal users.

V.25 bis acts as an interface between the mainframe and modem, sending synchronous responses.  An asynchronous device or a "dumb" terminal is used to configure the modem before it dials out in synchronous mode.

•	Dedicating a PC as a synchronous device by installing the proper hardware and software so it can communicate with a mainframe.  The modem is configured and dials out in asynchronous mode, then switches to synchronous mode once a connection is made.

NOTE:  HST modulation is not supported for synchronous communications.

## GENERAL REQUIREMENTS

Courier modems in asynchronous mode adapt to many conditions of remote asynchronous modems.  But synchronous connections to a mainframe require strict adherence to specific operating parameters.  If you are operating a terminal designed for a particular network, you probably need only set the Courier properly before calling or answering.

NOTE:  The term DTE in the following discussion means Data Terminal Equipment, the end-to-end equipment involved in data communications.  DTE denotes your terminal or computer and the remote computer.

What You Need to Know

The network's communications management staff can supply information on the following requirements:

•	The protocol needed for your calls

•	The software support needed to log into the network; for example, a specific communications package or interface board

•	The phone number, if you will be calling instead of answering

•	Any restrictions about when you can call

The RS-232 Interface

Transmit and Receive synchronous timing pins are required at the RS-232 interface.  You'll need either pin 15 or pin 24 for Transmitter timing signals, depending on whether the modem (pin 15) or the DTE (pin 24) generates the signals.  You'll also need pin 17, for Receiver timing signals.  If you're building your own cable, review the RS-232 Interface, in Appendix B.

Protocol Compatibility

The devices at both ends of the link must use the same protocol.  These protocols format data into blocks or frames and add control information.

If the modem is in V.25 bis mode, the link protocol must be HDLC (High-Level Data Link Control), or character-oriented.  If the modem is in Online Synchronous mode it may use HDLC, character-oriented, or another protocol determined by the mainframe manufacturer.

Data Rate Synchronization (&Xn)

During synchronous operations, transmit and receive clocks at both ends of the phone link control the precise timing of the data flow.  The communications equipment at the remote DTE and your modem and DTE must all handle the data at the same speed.

The transmit clock timing signals setting, &Xn, determines whether the modem or DTE will generate the timing signals.  For Online synchronous operations, the source for this setting must be the same on both systems.  See Connection Rate (%Nn, &Nn) later in this appendix.

Most Online synchronous users will require the default setting, &X0.

&X0	The Courier is the source of the Transmit clock timing signals and sends them to your DTE over the RS-232 interface.  The DTE rate will follow the connection rates.  Default.

&X1	The DTE is the source of the Transmit clock timing signals and sends them to the Courier over the RS-232 interface.  This setting is used typically in leased line multiplexed operations.  (Multiplexors divide the phone channel so that the channel carries several calls at the same time.)

The DTE ignores the Courier's clock timing signals and negotiates the DTE and connection rates.

&X2	The Courier's Receiver clock is the source of the timing signals.  The signals are looped to the Transmit clock and sent to your DTE over the RS-232 interface.  This setting is only used in those systems that require synchronization of data flowing in both directions.

## v.25 bis Requirements

V.25 bis is an ITU-T standard that uses the HDLC or character-oriented protocols to format data.

Before you attempt to connect to a synchronous network, you must configure the modem in command (asynchronous) mode by using either an asynchronous device or dumb terminal.  V.25 bis commands are used for this purpose.  The computer or terminal sends V.25 bis commands to the modem to ready the line for synchronous transmission.  Once the synchronous connection is made and the modem is in synchronous mode, V.25 bis commands are no longer necessary and are ignored.  The modems on each end of the connection are transparent to the host computers or terminals.

To set the modem, follow the instructions in Chapter 4, Command Set Usage.  Commands begin with a required AT prefix and end with a required Carriage Return, which we denote with the symbol <Enter>.  For example, the following command causes the modem to set the connection rates, enable normal result codes, enable auto answer, select HDLC as a link protocol, and save the settings to NVRAM.  Spaces have been added only for readability.

AT &N0 %N6 X0 S0=1 &M7 &W <Enter>

Be sure that DIP switch 10 is OFF so that the modem loads the settings you've stored in nonvolatile random access memory (NVRAM) when it powers on.  Until you customize your own settings, the settings in NVRAM are the same as the &F1 Hardware Flow Control template.

Connection Rate (&Nn, %Nn)

There are three phases to obtaining and maintaining a connection rate during synchronous communication.

### Clock Speed Control

The first phase is in deciding where the clock speed will be determined.  (See Data Rate Synchronization, above.)

### Offline Host/Modem Clock Speed

The second phase involves the data rate between the host computer or terminal and its modem during offline mode.  The %N command is used to set the clock speed between the modem and host, but this speed is only during offline mode, before the synchronous connection is made.

The %Nn command works in conjunction with &Xn.  If the modem is set so that it is the source of the Transmit clock timing signals (&X0¾default), the %Nn commands set the computer or terminal-to-modem V.25 bis clock speed.  If the modem is set to &X1, the computer is the source of the Transmit clock signals.

If %Nn is set to 0 or 1, you will receive an error message, since they are not valid values.  The %Nn rates are as follows:

%N0	   Reserved		%N6	9600 bps (default)

%N1	   Reserved		%N7	12.K bps

%N2	   1200 bps		%N8	14.4K bps

%N3	   2400 bps		%N9	16.8K bps

%N4	   4800 bps		%N10	19.2K bps

%N5	   7200 bps

### Online Connection Rate

The &N command sets the data rate during the synchronous connection.

If &Nn is set for 2-10, the modem ignores the %Nn rate and follows the &Nn rate to set the Online connection rate.  The &Nn rates are as follows:

&N0	   Variable (default)	&N6	9600 bps

&N1	   Reserved		&N7	12K bps

&N2	   1200 bps		&N8	14.4K bps

&N3	   2400 bps		&N9	16.8K bps 	
&N4	   4800 bps			(terbo-to-terbo only)

&N5	   7200 bps		&N10	19.2K bps 
					(terbo-to-terbo only)

### Recommended Settings

When the connection is made and the data rate is determined, host/modem rates may change dramatically to match the connection rate (when in online synchronous mode, the modem is transparent on the line).  To avoid this dramatic rate switching (which can be hard on some computers), we recommend that the modem be set with a fixed rate between the computer or terminal and modem (%Nn) and that the connection rate (&Nn) be set to the same rate.

Result Codes (Xn)

The Courier displays normal or extended synchronous result codes, depending on the setting of the Xn command.  By default, the modem is set to X1 for extended result codes.  To change to normal result codes, set the modem to X0.

The normal result codes return messages such as VAL or INV (VALID or INVALID), whereas the extended codes offer explanations¾INVPS (INVALID Parameter Syntax Error).

Automatic Answering (S0=1)

When the Courier is operating in V.25 bis mode, it ignores the DIP switch 5 setting, which controls Auto Answer.  To set the modem to automatically answer incoming calls, set the modem to S0=1, so it answers on the first ring.  You can substitute a higher value.  See the S-Register summary in Appendix B.

### Suppressing Auto Answer

To disable Auto Answer, set the modem to answer on zero rings, S0=0.

Choosing a Synchronous Protocol (&Mn)

Once the &Xn, %Nn, &Nn, Xn, and S0 commands are configured, you must use the &Mn command to choose the HDLC or character-oriented link protocol so that your synchronous software can properly format its commands.

&M6	Use the character-oriented protocol for synchronous communications.

The Courier and the remote modem must use the same eight-bit data format.  The character length must be 7 bits and either ODD or EVEN parity (ODD is preferred), or 8 bits and NO parity.

&M7	Use the High Level Data Link Control (HDLC), an 
ITU-T standard for synchronous communications.  HDLC ignores parity.

V.25 bis Commands and Result Codes

### Commands

·	CIC (Connect incoming call) 
Instructs the modem to answer an incoming call.

·	CRN (Call request using number provided )
Instructs the modem to dial the number following this command.

Example:

CRN18001234

·	CRS (Call Request with memory location)
Instructs the modem to dial a number stored in memory.

Example:

CRS3

·	DIC (Disregard incoming call)
Instructs the modem to disregard an incoming call¾overrides auto answer for this call.

·	PRNn (Program number)
Stores a number in NVRAM.

Example:

PRN3; 18001234

·	RFN (Request list of forbidden numbers)
Instructs the modem to list the numbers with which the modem is unable to connect.

·	RLN (Request list of stored numbers)
Instructs the modem to list those numbers previously stored in NVRAM.

### Dial Options

0-9	Digits

&	Flash

:	Wait for dial tone

>	(Greater Than) separator

<	Pause

=	(Equal Sign) separator

P	Pulse

T	Tone

.	(Period) separator

-	(Minus) separator

### Result Codes

·	SFI (Call failure indication) with optional parameters:

-	CFAB (Abort call)

-	CFCB (Local modem busy)

-	CFET (Engaged tone)

-	CFFC (Forbidden call)

-	CFNS (Number not stored)

-	CFNT (Answer tone not detected)

·	CFRT (Ring tone)

·	CNX (Connect)

·	LS (List of numbers)

·	LSF (List of forbidden numbers)

·	LSN (List of stored numbers)

·	INC (Incoming call)

·	INV (Invalid) with optional parameters:

-	INVMS (Message syntax error)

-	INVCU (Command Unknown )

-	INVPS (Parameter syntax error )

-	INVPV (Parameter value error )

·	VAL (Valid)

### Commands and Result Codes NOT Supported

·	CRI (Call request with identification number)

·	PRI (Program identifier)

·	RLI (Request list of identification numbers)

·	RLD (List of delayed call numbers)

Hanging Up

Since the modem cannot accept commands once it is connected in synchronous mode, you cannot use the ATH (hang up) command or the +++ escape sequence.

The only way for the modem to disconnect is to drop its DTR signal.  This is normally done with software.  However, if the modem is initially set to S32=6 when it is in asynchronous mode, the modem will reset when the voice/data switch is pressed, thus dropping DTR.

Returning to Asynchronous Mode

Once you've completed a communication session, you can switch between synchronous and asynchronous modes by flipping DIP switch 10 OFF (factory settings, asynchronous mode) and then ON (NVRAM settings, synchronous mode if the modem is set to &M6 or &M7).

The modem cannot switch between synchronous and asynchronous while a call is connected.

## Online Synchronous REQUIREMENTS

Although personal computers do not usually support synchronous communications, they can be dedicated to do so.  If you have a personal computer you must find out what hardware and software you need before setting the Courier for synchronous calls.

The Synchronous Adapter Card

You will probably have to purchase and install a synchronous adapter card.  These cards are multifunction boards that provide the following functions:

•	A synchronous port from the DTE to the modem

•	One or more synchronous protocols.  Be sure to find out which protocol the Host mainframe requires before you purchase an adapter.

•	Additional software functions.  For example, you need to identify the type of computer or terminal you are using to the mainframe software.  You will most likely also have to specify your application¾that is, identify the mainframe resources you want to use.

Setting the Modem

When the modem is set to Online Synchronous mode, it remains in command (asynchronous) mode until it makes a synchronous connection with a remote modem.  Upon connection, the Courier enters synchronous mode and sends synchronous timing signals to your DTE.

Because the modem will not accept commands when it is in synchronous mode, you will have to configure it in asynchronous mode before trying to connect to a synchronous network.

NOTE:  Be sure that DIP switch 1 is OFF (factory setting).  The Data Terminal Ready (DTR) override must be OFF so that the Courier detects when the DTE raises and lowers the DTR signal.

To set the modem, follow the instructions for issuing commands in Chapter 4, Command Set Usage.  Commands begin with a required AT prefix and end with a required Carriage Return, which we denote with the symbol <Enter>.

For example, the following command causes the modem to set the modulation, the connection rate, and choose a timing source.  Spaces have been added only for readability.

AT B0 &N0 &X0 <Enter>

Modulation/Connection Rate (Bn, &Nn)

Use the following guidelines for your Courier type.  V.Fast Class does not support synchronous mode.

### V.32 terbo modems

If the modem is connecting with another U.S. Robotics V.32 terbo modem, set both modems to B0 and to a variable connection rate, &N0.  The modems will connect at the highest possible rate.

If the Courier V.32 terbo is to connect with a V.32 modem, set the Courier to B0 and try a variable connection rate first, &N0.  If that doesn't work, try a fixed connection rate of &N6 (9600 bps) or &N3 (2400 bps).

### Modems in HST mode

HST modulation for synchronous communications is not supported.

### Dual Standard modems

Set the Dual Standard modem to B0.  Try a variable connection rate setting (&N0) first.  If that doesn't work, you may have to set a fixed connection rate, for example, &N6 (9600 bps).

NOTE:  If your modem is set to a fixed rate, and the remote modem is not set to the same rate, your modem hangs up.

Connection Rate (&Nn)

Use this command to set variable or fixed rates at the link interface.  The default is &N0, variable rate.  The Courier negotiates with the remote modem for the highest possible connection rate, depending on the capabilities of the remote modem.  If &N0 does not work, try a fixed rate.

NOTE:  The modem is not capable of connecting at 21.6K bps in synchronous mode.

When you set the modem to a fixed rate, it will only connect if the remote modem is operating at the same rate.  If not, your modem hangs up.

The fixed rate options are as follows:

&N0	  Variable rate	&N6	9600 bps

&N1	  Reserved	&N7	12K bps

&N2	  1200 bps	&N8	14.4K bps

&N3	  2400 bps	&N9	16.8K bps 
&N4	  4800 bps		(terbo-to-terbo only)

&N5	  7200 bps	&N10	19.2K bps (terbo-to-terbo only)

Generating Clock Timing Signals

The &Xn setting specifies whether the Courier or your DTE generates the Transmit clock timing signals for a synchronous call.  Most users will require the default setting, &X0.  See Data Rate Synchronization (&Xn), earlier in this appendix, for more information.

Dialing Out

The modem's stored command feature (&ZC=s) allows you to configure the modem for a synchronous connection and to dial out to the Host computer by just pressing the voice/data switch.  Below are our recommended procedures.

1.	If your communications software isn't running, load the program and put the computer in Terminal mode, as described in Chapter 2.  Terminal mode allows you to send AT commands directly to the modem.

2.	Using the guidelines in Chapter 4, store a command string (&ZC=s) that configures the modem according to the Host computer's requirements.  In addition, include the &M1 command, to have the modem enter synchronous mode, followed by the appropriate Dial string.  The following is an example:

AT &ZC = &F &X1 &M1 DT5551234 <Enter>

In the example, &F sets the modem's defaults as those of the &F0 template settings, while &X1, which specifies the computer as the synchronous timing source, is the only non-default setting used besides synchronous mode (&M1).  Any non-default settings should follow the &F command.  The Dial command should be the last entry before the Carriage Return.

3.	Set the voice/data switch to option 9, so that the modem executes the stored command string when you press the switch.  Type the following:

ATS32 = 9 <Enter>

NOTE:  If you usually use the voice/data switch for another purpose, you can include the execute stored command option (S32 = 9) in the stored command string.  However, the total number of characters in the string may not exceed 30.

If S32=9 is used with the stored command string, it over-writes any other S32 option.  Once the communication session is over, you will have to reset S32 if you want to select another option.

4.	Press the voice/data switch when you want to connect with the synchronous Host computer.  You need not have your communications software loaded at the time.

5.	After the call, restore the modem to asynchronous operations by powering it off and on again.

Answering

To configure the modem for Answer mode using the voice/data switch, follow the instructions in the previous section, Dialing Out, but with these modifications:

•	Insert S0 = 1 in the stored command, Step 2.  This sets the modem to Auto Answer.

•	Omit the Dial string shown in the same step (DT and phone number).

If you don't wish to use a stored command and the voice/data switch, send the modem the appropriate configuration string, ending with &M1.  The following command adapts the dial-out command example.  Note again that all non-default settings follow the &F command.

AT &F S0=1 &X1 &M1 <Enter>

The modem responds automatically to an incoming call, enters synchronous mode and, in this case (because it is set to &X1), waits for synchronous timing signals from your DTE.

Hanging Up

The modem remains online until the remote modem disconnects or your software causes the DTE to drop the Data Terminal Ready signal (DTR).  The Courier sends the NO CARRIER result code if result codes are enabled, and returns to asynchronous Command mode.

Testing and Inquiry Commands

The modem testing commands (&Tn) and inquiry commands (In) cannot be used when the modem is operating in synchronous mode.

## Enabling Remote Access

Because synchronous equipment cannot use AT commands to configure modems, a special function was designed so the modem can receive AT commands remotely.  To do this, the modem must be placed in remote access mode with the voice/data switch.

Press and hold down the voice/data switch while powering on the modem.  Pressing the switch during power-on causes the modem to perform its normal self-test, enable Auto Answer, enable Remote Access (by changing the S41 setting to 1), and disable password security.

WARNING:  If you enable remote access by pressing the voice/data switch while powering on the modem, previously set passwords will be erased.

DEDICATED AND LEASED LINE OPERATIONS

The following operations apply in installations where the modem's phone line is not part of a public-access switched telephone network.  Instead, the modem is connected to a special user-installed telephone line or a line that is leased from the telephone company.  These lines are often referred to as dedicated (to a pair of modems) or private lines.

In both types of installation there is a continuous point-to-point connection between two modems.  No dialing of phone numbers is required.  The modems may be in either Smart or Dumb mode (determined by the position of DIP switch 8).

The User-Installed or Leased Telephone Line

User-installed lines are most commonly 2-wire lines, similar to the 2-wire lines that connect residential phones to the public switched network.

If you are leasing a line from the telephone company, request a 2-wire line, the type of line the modem is designed to work with.  If the telephone company only makes a 4-wire line available, you'll need a 4-wire to 2-wire converter at each end of the connection.  If the phone company does not install the converters, you will have to supply them.

For optimal operations, we recommend that the physical length of these lines not exceed 5 miles.

Setting the Modem

If the Courier is set to &L1, as described in what follows, and the remote AT-compatible modem has a comparable setting, they automatically connect when they are powered on.  They also reconnect, without any operator intervention, if a disturbance on the line is severe enough to break the connection.

Set the modem as follows:

1.	Set your terminal or communications software to the rate at which you want the modems to communicate.  For example, use a terminal/software setting of 19.2K bps and, if both modems have the capability, they will connect at 14.4K bps.  The following instructions assume that you are familiar with the guidelines on using the &B and &H commands (Chapter 3) and the &W command (Chapter 3).

2.	Send the modem the following command:

AT &B1 &S2 &H1 &L1 &W <Enter>

&B1 fixes the modem's computer interface rate at the same serial port rate you selected when setting up your communications software.  &S2 causes the modem to send a Clear to Send (CTS) signal only after it sends the Carrier Detect (CD) signal, that is, only after it connects with the remote modem.  (See the note that follows.)  &H1 enables hardware (CTS) flow control.

&L1 forces the modems off hook at power on and enables them to re-establish the connection should it be broken.  &W writes the settings to nonvolatile memory (NVRAM) as power-on defaults.

NOTE:  We recommend using the &S2 setting to delay CTS until after the connection is made, as a precaution.  If the modems are in the process of connecting or reconnecting, the Courier interprets any keyboard data entry, including an accidental key stroke, as a key-press abort, and hangs up.  Delaying CTS until after carrier detection prevents this from happening, for example, if you are typing data to the remote modem when the modems momentarily disconnect and begin to reconnect.  However, you have to set the modem for hardware flow control, &H1.

If your software or machine does not support Clear to Send (CTS), don't include &S2 and &H1 in the command string as suggested above.  Follow the Transmit Data flow control (&H) guidelines in Chapter 3.  But keep in mind that if the modems fail to connect or reconnect, the reason could be a key-press abort.

3.	Set the modem to load NVRAM settings at power-on, DIP switch 10 OFF.  It does not matter if the modem is in Dumb or Smart mode (DIP switch 8).

4.	Decide which modem is to be the calling modem and which the answering modem.  Set the answering modem to Auto Answer, DIP switch 5 OFF, and the calling modem to Auto Answer suppressed, DIP switch 5 ON.

5.	Power off and power on the modems.  This initiates the new DIP switch settings and loads the power-on defaults, including &L1.  The modems go off hook and establish the connection.

NOTE:  If the modems cannot restore the connection and you could not set the modem to &S2, the reason could be a key-press abort.  If the problem persists, however, you may need to call your telephone company to have them check your line.