aPPENDIX C.  ALPHABETICAL COMMAND SUMMARY

Additional command summaries are in Chapter 4, on the bottom panel of the modem, and in the Quick-Reference Card.

## COMMAND SET USAGE

Follow these guidelines:

1.	Your software must be loaded and if you are using a computer, it must be in Terminal mode.

Some communications programs put the computer in terminal mode automatically when they are loaded.  Others require you to display a communications terminal screen, press a Function key, or perform some other operation.  Refer to your communications software documentation for instructions.

In Terminal mode the computer acts as if it were a standard terminal such as a teletypewriter, rather than a data processor.  Keyboard entries go directly to the modem, whether the entry is a modem command or data to be transmitted over the phone lines.  Received data is output directly to the screen.

2.	Type commands in either upper or lower case, not a combination (AT or at—not At).

3.	All commands except A/, A> and +++ are preceded by the AT (attention) prefix and are executed with the Enter/Carriage Return key (<Enter>).

4.	Command length = 60 characters maximum.  The modem doesn't count the AT prefix, Carriage Return character, or spaces.  It counts (but doesn't act on) punctuation such as hyphens and parentheses.

5.	A missing numeric parameter is assumed to be zero, as in the command to hang up:  ATH <Enter> is the equivalent of ATH0 <Enter>.

Example (spaces are not required, but are added here for readability):

AT &K3 X2 DT 071 312 1234 <Enter>

AT	Attention; a command follows.

&K3	Disable MNP5 data compression; use only V.42 bis compression.

X2	Use the X2 result code subset.

DT	Dial the following number using tone dialing.

<Enter>	Execute the commands.

NOTE:  The defaults listed are based on the modem's shipping configuration:  load from nonvolatile random access memory (NVRAM), DIP switch 10 OFF, which is the same as the &F1 configuration template.  For a complete listing of default configuration templates, see Appendix B.

## Basic Command set

+++	Escape code operations.  Once the modem is online to another system, the only command it recognizes is an escape code of three typed pluses, which forces the modem back to Command mode.  Do the following when issuing the command:

•	Wait one second after sending the last item of data

•	Type: +++

•	Wait one second before typing any data

Do not type the AT prefix or a Carriage Return.  The guard time of one second before and after the code prevents the modem from misinterpreting the occurrence of +++ in the transmitted data stream.

If necessary, the character used in the escape code or the duration of the guard time can be changed by resetting Register S2 or S12.  See the S-Register Summary in Appendix B.

In response to +++ the modem returns to Command mode.  However, it keeps the line open or hangs up, depending on the setting of DIP switch 9:

DIP Switch 9	Response to +++

OFF	Modem goes on hook (hangs up), sends NO CARRIER result code (factory setting)

ON	Modem maintains connection (Online-Command mode), sends OK result code

The factory setting (OFF) forces an automatic disconnect when you issue +++.   An advantage of this is that you are not likely to inadvertently run up an all-night phone bill.

Set DIP switch 9 ON if you want the modem to respond to +++ by entering Online-Command mode, enabling it to execute commands and return online.  (See the O command later in this appendix.)

>	If you know the modem you are calling is frequently busy, include the Repeat command in the Dial string, as follows:

AT > DT 1234567 <Enter>  or
AT DT 1234567 > <Enter>

The modem enters Repeat mode, dials the number, waits 60 seconds for a carrier (default), and hangs up.  Then after a two-second pause, it redials.

The cycle continues until the modems connect or the modem reaches a maximum of 10 attempts.

A	Force Answer mode when the modem hasn't received an incoming call.

A/	Re-execute the last issued command.  A/ doesn't take the AT prefix or a Carriage Return, and can be used to redial.

A>	This command combines the features of both the A/ and > commands.  The modem enters Repeat mode and redials the Dial string in the command buffer.  Like the A/ command, A> does not take the AT prefix or a Carriage Return.

AT	Attention command prefix.  Use AT alone to test for the OK result code.  AT must prefix all commands except A/, A> and +++.

Any	Terminate the current dialing operation resulting from an

key	issued Dial command; terminate Repeat mode (> or A>).

Bn	Handshake options for international calls above 1200 bps.

B0	ITU-T (formerly CCITT) answer sequence.  Default.  This is required to answer all V.32-type calls as well as calls from overseas.

B1	Bell answer tone.  This setting selects HST modulation in Dual Standard modems; but should only be used if the modem is not required to answer V.32-type calls.

NOTE:  This setting is required for HST cellular calls.

Cn	Transmitter enabled/disabled.

C0	Transmitter disabled; receive-only condition.

C1	Transmitter enabled (Default).

Dn	Dial the specified phone number; also execute Dial options.

The maximum number of characters allowed is 36, including the AT prefix, punctuation and spaces.  The Carriage Return (Enter key) isn't counted as a character.

NOTE:  With the exception of the Dial options, the modem ignores any commands issued after D in the same command string.

D	Dial the number that follows and enter Originate mode.  Optional parameters:

P 	Pulse dial (Default).

T	Tone dial.

,	(Comma) Pause for 2 seconds before continuing to dial.

;	Return to Command mode after dialing.

" 	Dial the letters that follow (in an alphabetical phone number).

!	Transfer a call (flash the switch-hook).  This command applies to modems in installations where other modems share the phone line.  The modem flashes the switch-hook (goes off hook 0.5 seconds, on hook for 0.5 seconds and off hook again) to dial the specified extension.

W	This command is useful in situations where you must wait for a second dial tone before continuing dialing.

@	Wait for an answer (with X3 or higher).  The @ command can be used in the Dial string to tell the modem to detect at least one ring, wait for five seconds of silence at the other end of the call, and then continue to execute the Dial string.

/	A slash (/) causes a pause of only 125 milliseconds.

R	Reverse frequencies.  This command allows calls to an originate-only modem.  It reverses the modem's originate/answer frequencies, forcing the Courier to dial out at the answer frequency.

DL	Dial the last-dialed number.  The modem stores each Dial command until it receives the next Dial command.  Use DL instead of A/, described on the next page, if you wish to send the modem non-Dial commands before dialing again.

DSn	Dial the number stored in nonvolatile random access memory at position n, where n = 0-9.

En	Command mode local echo.  Enables/disables the display of your typed commands.  If double characters appear on the screen, both the modem's local echo and your software's local echo are on.

The Courier is shipped with DIP switch 4 OFF, enabling local echo.  The En command controls the local echo for a current session, independently of the switch setting.

E0	Command mode echo OFF.  The modem does not display keyboard commands.

E1	Command mode echo ON.

Fn	Online local echo.  This command causes the modem to display a copy of the data it is transmitting to another system.

F0	Online echo ON.  Sometimes called half duplex.  As the modem transmits data to a remote system, it also sends a copy of the data to the screen.

F1	Online echo OFF.  Sometimes called full duplex.  Default.

Hn	On/off hook control.

H0	Hang up (go on hook).

H1	Go off hook.

In	Inquiry

I0	Display product code

I1	Display results from ROM checksum

I2	Display results from RAM test

I3	Display the duration of the last call

I4	Display current modem settings

I5	Display NVRAM settings

I6	Display Dial diagnostics summary of the last call

I7	Display product configuration information

I8	Reserved

I9	Reserved

I10	Display dial security account status information

I11	Reserved

Kn	Modem clock operation:  Call-duration or Real-time mode.  Displayed with ATI3 and ATI6 commands.

K0	Display current call-duration if online.  Display
last call-duration if offline.  Default.

K1	Return actual time at ATI3.  Clock is set using ATI3=HH:MM:SS K1.

Ln	Speaker volume control (internal modems only)

L0	Low

L1	Low

L2	Medium (default)

L3	High

NOTE:  External modems will return an OK message to this command, but the volume will not change.  The volume button must be used to alter volume.

Mn	Speaker (audio monitor).

M0	The speaker is always OFF.

M1	The speaker is ON until carrier is established (Default).

M2	The speaker is always ON, including during data transfer.

M3	The speaker is ON after the last digit is dialed and remains ON until carrier is established.

On	If DIP switch 9 is ON (on detection of the escape code the modem maintains the connection), you can issue commands and then toggle the modem back online with the On command.

There are two ways to return online.

ATO0	Return online (normal).

ATO1	Return online and retrain.  Use to have the modem re-synchronize if there were errors in a non-ARQ data transfer.

Qn 	Enable/suppress the display of result codes.  The Courier is shipped with DIP switch 3 ON, to display result codes.  Use the Qn command to control the display for a current session, independently of the switch setting.

Q0	Result codes displayed.

Q1	Result codes suppressed (quiet).

Q2	Result codes suppressed in Answer mode.

Sr=n	Set S-Register value:  r is any S-Register; n must be a decimal number between 0 and 255.

Sr.b=n	Alternative command for setting bit-mapped registers:  r is the bit-mapped register; .b is the bit; n is 0 (off) or 1 (on).

Sr?	Query contents of register r.

Vn	Return result codes in words or numbers (Verbal/Numeric mode).

V0	Numeric mode.

V1	Verbal mode.

Xn	Result code set options.  Use the following table (Default = X7, all codes except 12/VOICE).  For result codes in synchronous operations, see Appendix F.

Setting

Result Codes	X0	X1	X2	X3	X4	X5	X6	X7

0/OK	·	·	·	·	·	·	·	·

1/CONNECT	·	·	·	·	·	·	·	·

2/RING	·	·	·	·	·	·	·	·

3/NO CARRIER	·	·	·	·	·	·	·	·

4/ERROR	·	·	·	·	·	·	·	·

5/CONNECT 1200		·	·	·	·	·	·	·

6/NO DIAL TONE			·		·		·	·

7/BUSY				·	·	·	·	·

8/NO ANSWER				·	·	·	·	·

9/RESERVED

10/CONNECT 2400		·	·	·	·	·	·	·

11/RINGING						·	·	·

12/VOICE						·	·

13/CONNECT 9600		·	·	·	·	·	·	·

18/CONNECT 4800		·	·	·	·	·	·	·

20/CONNECT 7200 		·	·	·	·	·	·	·

21/CONNECT 12000		·	·	·	·	·	·	·

25/CONNECT 14400		·	·	·	·	·	·	·

47/CONNECT 16800 		·	·	·	·	·	·	·

85/CONNECT 19200		·	·	·	·	·	·	·

91/CONNECT 21600		·	·	·	·	·	·	·

99/CONNECT 24000		·	·	·	·	·	·	·

103/CONNECT 26400		·	·	·	·	·	·	·

107/CONNECT 28800		·	·	·	·	·	·	·

Functions

Adaptive Dialing			·	·	·	·	·	·

Wait for 2nd Dial Tone (W)				·	·	·	·	·

Wait for Answer (@)				·	·	·	·	·

Fast Dial			·		·		·	·

Z	Software reset to NVRAM settings when DIP switch 10 is OFF (factory setting).  If DIP switch 10 is OFF, the modem resets to the &F0 configuration template, with no flow control.

## Ampersand (&) Command Set

&An	Enable/disable additional result code subsets.  See the Xn command.

&A0	ARQ result codes are disabled.

&A1	ARQ result codes enabled.

&A2	Additional VFC, HST, or V32 modulation indicator.

&A3	Additional error control indicator (LAPM, HST, MNP, SYNC, or NONE) and data compression type (V42BIS or MNP5).  Default.

&Bn	Serial port rate variable or fixed.

&B0	Variable rates.

&B1	Fixed rate.  Default.  The modem always communicates with the computer at the rate at which you have set the terminal or software, regardless of the connection rate.

This setting is not affected by the &N setting.  However, the serial port rate must be equal to or higher than the &Nn rate.

&B2	Fixed for ARQ calls/Variable for non-ARQ calls.  Answer mode only.  When the modem goes off hook and connects in ARQ mode, it shifts its serial port rate up to a user-specified rate, for example, 38.4K bps.  If the connection is not under error control, the modem behaves as if it were set to &B0 and switches its serial port rate to match the connection rate of each call.

&Cn	Carrier Detect operations.  At power-on and reset, the modem operates according to the setting of DIP switch 6.  This command is not stored in nonvolatile memory as a power-on/reset default.

&C0	CD override, CD always ON.

&C1	Normal CD operations.  The Courier sends a CD signal when it connects with another modem and drops the CD when it disconnects.

&Dn	Data Terminal Ready (DTR) operations.  At power-on and reset, the modem operates according to the setting of DIP switch 1.  This command is not stored in nonvolatile memory as a power-on/reset default.

&D0	DTR override.  The modem operates as though the DTR is always ON.

&D1	Advance usage:  If issued before connecting with another modem, the modem can enter online command mode during a call by toggling DTR.  &D1 functions similarly to the escape code (+++), except that this setting is independent of DIP switch 9.

If DIP switch 1 is ON (DTR override) when you issue the &D1 command, the DTR override is automatically turned off.  However, if you change the setting of DIP switch 1 after issuing &D1, the DIP switch setting tales precedence.

Return online with the On command, or hang up with the Hn command.

&D2	Normal DTR operations.  The terminal or computer must send a DTR signal for the modem to accept commands.  Dropping DTR terminates a call.

&Fn	The modem is shipped with four configurations (templates), &F0-&F3, stored in permanent nonprogrammable memory (ROM).  Appendix B includes configuration listings for each template.  Any one of the templates may be loaded into current memory (AT &Fn) or written to nonvolatile memory and reset default (AT &Fn &W).  Note, however, that &F0 is always loaded into memory if DIP switch 10 is ON.

&F0	Load No Flow Control template settings

&F1	Load Hardware Flow Control template settings (Default)

&F2	Load Software Flow Control template settings

&F3	Load HST Cellular template settings

&Gn	This setting applies only to overseas calls at 2400 or 1200 bps.  British phone switching systems require the modem to send an 1800 Hz guard tone after it sends an answer tone.  Some other European phone networks require a 550 Hz guard tone.  Guard tones are not used in the United States or Canada.

&G0	No guard tone.  This is used in the U.S. and in Canada (Default).

&G1	This sets a 550 Hz guard tone, and is used in some European countries.

&G2	This sets an 1800 Hz guard tone, and is used in the U.K. and some Commonwealth countries.  &G2 requires the B0 setting.

&Hn	Transmit data flow control is for data transmitted to the modem by its attached computer or terminal.  The modem monitors its buffer as data comes from the computer or modem.  If the buffer approaches 90% capacity, the modem signals the computer or terminal to stop transmitting.  When the modem has sent enough data over the link to half empty the buffer, it signals the computer or terminal to resume transmitting.

&H0	Transmit Data flow control disabled.

&H1	Hardware flow control.  Default.  Requires that your computer or terminal and software support Clear to Send (CTS) at the RS-232 interface.

&H2	Software flow control.  Requires that your software support XON/XOFF signaling.

&H3	Use both hardware and software flow control.  If you are unsure about what your equipment supports, select this option.

&In	Received data software flow control.

&I0	Disables XON/XOFF flow control of received data.  
	Default.

&I1	The Courier acts on your typed XON/XOFF commands, Ctrl-S or Ctrl-Q, and passes them to the remote computer.

&I2	The Courier acts on your XON/XOFF commands, but removes them from the data stream instead of passing them to the remote computer.  This is the recommended setting for ARQ mode.

&I3	Hewlett Packard¾Host mode.  Applies only to modems attached to an HP mainframe that uses the ENQ/ACK protocol.  Use in ARQ mode only.  See Appendix G.

&I4	Hewlett Packard¾Terminal mode.  Applies only to modems attached to terminals in an HP system that uses the ENQ/ACK protocol.  Use in ARQ mode only.  See Appendix G.

&I5	This setting is designed to enable flow control on the phone link when the connection is not under error control.  For this to work for you, the remote modem must have &I5 capability.

&Kn	Enable/disable data compression.

&K0	Data compression disabled.

&K1	Auto enable/disable.  Default.  The modem enables compression if the serial port rate is fixed, &B1, and disables compression if the serial port rate follows the connection rate, &B0, because compression offers no throughput advantage when the serial port and connection rates are equal.  Compression may even degrade throughput.

&K2	Data compression enabled.  Use this setting to keep the modem from disabling compression.

&K3	Selective data compression.  The modem negotiates only for V.42 bis compression, and disables MNP Level 5 (MNP5) compression.  Use this setting to transfer 8-bit binary files, .ZIP files, and other files that are already compressed.

&Ln	Normal/Leased phone line.

&L0	Normal phone line.  Default.

&L1	Leased line; enables modem to reconnect if disconnected.

&L2	Cellular operations; with B1, enables HST Cellular mode.

&Mn	Enable ARQ (error control) or synchronous protocols.  Both your modem and the remote modem must use the same protocol.

&M0	Normal mode, no error control.  Due to the nature of phone line channels, this is never recommended for calls above 2400 bps.

&M1	This setting is exclusive of the modems' error control and is used only for online synchronous mode without V.25 bis.  See Appendix F for more information.

&M2	Reserved.

&M3	Reserved.

&M4	Normal/ARQ mode.  Default.  If an ARQ connection isn't made, the modem operates in Normal mode, as though it were set to &M0.

&M5	The modem enters ARQ asynchronous mode.  The modem hangs up if an ARQ connection cannot be made.

&M6	The modem enters V.25 bis synchronous mode, using a character-oriented link protocol similar to BISYNC.  See Appendix F for more information.

&M7	The modem enters V.25 bis synchronous mode, using the HDLC link protocol.

&Nn	Connection rate variable or fixed.

&N0	Variable rates.  Default.  The Courier negotiates with the remote modem for the highest possible connection rate, depending on the capabilities of the remote modem.  This is the recommended setting, and is required for the Courier V.32 terbo to connect at 21.6K bps.  Both modems must be U.S. Robotics modems with V.32 terbo to make a 21.6K bps connection.

&N1-	Fixed rate.  The modem only connects if the remote

&N14	modem is operating at the same rate.  If not, the modem hangs up.  If you wish, you can filter out calls at other than a specific rate, for security or other reasons, by fixing the connection rate.

The connection rate must always be lower than, or equal to, the serial port rate, never higher.

The options are as follows.

&N1	300 bps	&N9	16.8K bps (HST, terbo,

&N2	1200 bps		V.FC, and V.34 only)

&N3	2400 bps	&N10	19.2K bps (terbo, V.FC, 
&N4	4800 bps		and V.34 only)
&N5	7200 bps	&N11	21.6K bps (terbo, V.FC,
&N6	9600 bps		and V.34 only)

&N7	12K bps	&N12	24K bps (V.FC and V.34 only)

&N8	14.4K bps	&N13	26.4K bps (V.FC and V.34 only)
				&N14	28.8K bps (V.FC and V.34 only)

&Pn	This command sets the ratio of the off-hook/on-hook (make/
break) interval for pulse dialing.  The default sets the modem for use in North America.  The ratio must be changed if the modem is used in the United Kingdom and some Commonwealth countries.

&P0	Make/break ratio, U.S./Canada:  39%/61%.  Default.

&P1	Make/break ratio, United Kingdom, some Commonwealth countries:  33%/67%.

&Rn	Received data hardware (RTS) flow control

&R0	Delay Clear to Send Response after Request to Send
	signal (RTS/CTS delay).

&R1		The modem ignores RTS.  This setting is required if your computer or terminal or software does not support RTS.

&R2	Hardware flow control of received data enabled.  Default.  The modem sends data to the computer or terminal only on receipt of the RTS signal.

&Sn	The modem sends the computer or terminal a Data Set Ready (DSR) signal via the RS-232 interface.  (Data Set is industry jargon for modem.)

&S0	DSR is always ON (override).  Default.

&S1	In Originate mode, the modem sends the DSR after it dials, when it detects the remote modem's answer tone.  In Answer mode, the modem sends the DSR after it sends an answer tone.

&S2	This option is for specialized equipment such as automatic callback units.  On loss of carrier, the modem sends a pulsed DSR signal with Clear to Send (CTS) following Carrier Detect (CD).

&S3	This is the same as &S2, but without the CTS following CD.

&S4	The modem sends the computer a DSR signal at the same time as it sends the Carrier Detect (CD).

&Tn	Modem testing

&T0	End test

&T1	Initiate Analog Loopback (AL) testing

&T2	Reserved

&T3	Initiate Local Digital Loopback (LDL) testing

&T4	Grant Remote Digital Loopback (RDL)

&T5	Deny RDL

&T6	Initiate RDL testing

&T7	Initiate RDL with self test and error detection

&T8	Initiate AL with self test and error detection

&W	Write the desired template to NVRAM, using the &W command.

&W0	Write to NVRAM template 0

&W1	Write to NVRAM template 1

&Xn	Synchronous transmit clock timing signals setting.  Determines whether the modem or DTE will generate the timing signals.

&X0	The Courier is the source of the Transmit clock timing signals and sends them to your DTE over the RS-232 interface.  The DTE rate will follow the connection rates.  Default.

&X1	The DTE is the source of the Transmit clock timing signals and sends them to the Courier over the RS-232 interface.  This setting is used typically in leased line multiplexed operations.  (Multiplexors divide the phone channel so that the channel carries several calls at the same time.)

The DTE ignores the Courier's clock timing signals and negotiates the DTE and connection rates.

&X2	The Courier's Receiver clock is the source of the timing signals.  The signals are looped to the Transmit clock and sent to your DTE over the RS-232 interface.  This setting is only used in those systems that require synchronization of data flowing in both directions.

&Yn	Break handling.  This command allows you to send a break to abort data transfer without disconnecting from the phone link.

&Y0	Destructive, don't send Break.

&Y1	Destructive, expedited (Default).

&Y2	Nondestructive, expedited.

&Y3	Nondestructive, unexpedited; modem sends Break in sequence with data received from computer or terminal.

&ZC?	Display the stored command string.

&ZC=s	Write the following command string s to NVRAM.  The command string may be up to 40 characters long; spaces are not counted.  This command can be used so that you can call another modem without loading your communications software.

After storing a command, you can program the voice/data switch to execute the stored command string when pressed.  The following example assigns a command string that displays the link diagnostics screen when you press the voice/data switch.

AT&ZC=I6 <Enter>

The function of the voice/data switch is determined by the setting of Register S32, as described in Appendix G.  Set the voice/data switch function to execute stored command string by setting Register S32 to 9 with the following command:

ATS32=9 <Enter>

Note that you can reset the voice/data switch at any time to one of the other available functions.  Additionally, you can overwrite the stored command string with a new one at any time.

Once you've stored your command string and set Register S32, all you need to do is press the voice/data switch whenever you want the command string executed.

&Zn=s	This command stores up to ten numbers, where n is the position 0-9 in nonvolatile memory, and s is the phone number string.  The number-string may be up to 36 characters long, including any Dial command options.

AT &Z2=555-6789 <Enter>

Do not include modem settings in the &Zn string.  If the call requires a special setting, insert it in the command string before the DSn command.  In the following example, &M0 (no error control) is inserted before the Dial command:

AT&M0 DS2 <Enter>

NOTE:  The &Zn=s command functions differently when Dial Security is enabled.  See Appendix D for more information.

&Zn?	Display the phone number stored in NVRAM at position n 
(n = 0-9).

## Percent Command Set

%An	Create and configure security accounts.  See Appendix D.

%Bn	Remotely configure the Courier's serial port rate.  See Appendix D.

%B0	110 bps			%B6	9600 bps

%B1	300 bps			%B7	19,200 bps

%B2	600 bps			%B8	38,400 bps

%B3	1200 bps		%B9	57,600 bps

%B4	2400 bps		%B10	115,200 bps

%B5	4800 bps

%Cn	Remote configuration control.  See Appendix D.

%C0	Defer configuration.   This is the default.  Configuration changes are deferred until the call is ended; they take effect for ensuing connections.  You do not need to enter this command; it is the default unless you enter %C1 or %C2.

%C1	Restore configuration.  Use this command to cancel any configuration changes made during remote access, and restore the original configuration.  However, commands that have been written to NVRAM (with &W) will not be restored to their previous settings.  Additionally, if you have forced immediate configuration changes (with %C2), those changes cannot be reversed with %C1.

%C2	Execute configuration.  Use this command to force configuration changes to take effect immediately, during the current connection.  We recommend against forcing immediate configuration changes unless absolutely necessary, as this can result in an unreliable connection or even a loss of connection.

%E=n 	Make security system edits.  See Appendix D.

%E=1	Erase local access password.

%E=2	Erase autopass password.

%E=3	Erase passwords in accounts 0–9.

%E=4	Erase phone numbers in accounts 0-9.

%E=5	Disable Account, Dialback, and New Number fields in accounts 0-9 disabled.

%Fn	Remotely configure data format.  See Appendix D.

%F0	No parity, 8 data bits.

%F1	Mark parity, 7 data bits.

%F2	Odd parity, 7 data bits.

%F3	Even parity, 7 data bits.

%L=	Assign an account password as the local access password.  See Appendix D.

%N	Works in conjunction with &Xn in synchronous mode.  If the modem is set so that it is the source of the Transmit clock timing signals (&X0¾default), the %Nn commands set the computer or terminal-to-modem V.25 bis clock speed.  If the modem is set to &X1, the computer is the source of the Transmit clock signals.

If %Nn is set to 0 or 1, you will receive an error message, since they are not valid values.  The %Nn rates are as follows:

%N0	   Reserved		%N6	9600 bps (default)

%N1	   Reserved		%N7	12.K bps

%N2	   1200 bps		%N8	14.4K bps

%N3	   2400 bps		%N9	16.8K bps

%N4	   4800 bps		%N10	19.2K bps

%N5	   7200 bps

%Pn=	Disables password security (n=0 or 1) when no character follows the equal sign.  See Appendix D.

%Pn=s	Specify the following password string (s) for viewing privileges only (n = 0) or view and configuration privileges (n= 1).  See Appendix D.

%Pn?	Display password n.  See Appendix D.

%S=n	Obtain access to security accounts without disabling security.  See Appendix D.

%T	Enables the modem, when off hook, to detect the tone frequencies of dialing modems.  %T is meant primarily for use with network applications, but may also be integrated into certain software programs.  For example, %T could be used in a security program to identify incoming tone security codes.

To enable %T, type ATH1 <Enter> to force the modem off hook.  Then type AT%T <Enter>.

To return the modem to Command mode, press any key or drop the computer's or terminal's DTR signal.  The modem responds OK.

%V=PWn	Assign the password in account n in your Courier modem’s security as your autopass password.  See Appendix D.