Chapter 5.  Fax Operations and Call Detection

FAX OPERATIONS

### Compatibility

The Courier modem provides Group III-compatibility when combined with Class 1 or Class 2.0 fax software.  In addition, the modem adheres to the following standards.

NOTE:  The International Telecommunication Union (ITU-T) was formerly the International Telegraph and Telephone Consultative Committee (CCITT).

TIA/EIA-578	Service Class 1 Asynchronous Facsimile DCE Control Standard

TIA/EIA-592	Service Class 2.0 Asynchronous Facsimile DCE Control Standard

ITU-T V.17	14.4K/12K bps

ITU-T V.29	9600/7200 bps

ITU-T V.27 ter	4800/2400 bps

ITU-T V.21 	300 bps

### Fax Modem Guidelines

Fax operations require facsimile-compatible software that can send or receive Group III faxes.  Follow the instructions in your fax software manual.

The modem's normal operating mode is Data mode.  If your fax software is typical, it automatically switches the modem to Fax mode when you run the program, and resets the modem to Data mode when you exit the program.

If you have a problem, however, and think the modem may be in the wrong mode, you can use one of the following AT commands to manually switch the modem:

AT+FCLASS=0 (Switch to Data mode) <Enter>

AT+FCLASS=1 (Switch to Class 1 Fax mode) <Enter>

AT+FCLASS=2.0 (Switch to Class 2.0 Fax mode) <Enter>

If you are not sure whether the modem is in Data or Fax mode, type the following command.

AT+FCLASS? <Enter>

The modem  returns a value of 0 to indicate Data mode,  1 to indicate Class 1 Fax mode, or 2.0 to indicate Class 2.0 Fax mode.

NOTE:  Whenever the fax modem is reset using the ATZ command, by toggling the DTR signal, or by turning the power off and on, the modem will be set to Data mode.

Fax Mode Flow Control Setting

Many facsimile software products use software flow control when the modem is in Fax mode.  Throughout our documentation, we recommend that you use hardware flow control for Data mode (factory setting).  However, to allow compatibility with software products that use software flow control by default, U.S. Robotics fax modems now automatically change to software flow control when entering Fax mode.

FCC Notice

FCC part 68, rules regarding fax operation, has been amended as follows:

Telephone facsimile machines—identification of the sender of the message:  It shall be unlawful for any person within the United States to use a computer or other electronic device to send any message via a telephone facsimile machine unless such a message clearly contains, in a margin at the top or bottom of each transmitted page or on the first page of the transmission, the date and time it is sent and an identification of the business, other entity, or individual sending the message and the telephone number of the sending machine or of such business, other entity, or individual.  Telephone facsimile machines manufactured on and after December 20, 1992 must clearly mark such identifying information on each transmitted page.

## NOTES to Programmers

Lists of supported Class 1 fax commands and optional Class 2.0 commands are in Appendix K.

If you want to know more about the supported Class 1 fax commands, refer to the standard for the Service Class 1 fax protocol.

ANSI/EIA/TIA-578-1990 (EIA-578)

Asynchronous Facsimile DCE Control Standard

November, 1990   Approved:  October 22, 1990

For more information on Class 2.0, refer to the standard for the Service Class 2.0 fax protocol.

ANSI/EIA/TIA-592-1993 (EIA-592)

Asynchronous Facsimile DCE Control Standard

May, 1993

You can obtain copies of these standards by contacting Global Engineering Documents, at 1-800-854-7179.

Call DETection

Courier High Speed modems support Call Detection, which is a method of reporting whether an incoming call is Data, Fax Class 1, or Fax Class 2.0.  It is especially useful for Bulletin Board systems, as it automates recognition of different calls from multiple users.

Call Detection is an optional Service Class 2.0 feature, and is also implemented by U.S. Robotics for Fax Class 1 applications.

To obtain a copy of the technical specification of U.S. Robotics' implementation of Call Detection for Fax Class 1, call our BBS at (708) 982-5092 and download the file CALLSEL.TXT.

For information on implementing Fax Class 2.0 Call Detection, see the standard listed above.  For a list of the optional Fax Class 2.0 commands supported by U.S. Robotics, see Appendix K.