# APPENDIX K.  TECHNICAL SPECIFICATIONS

Your modem uses multiple standard modulation protocols and is also compatible with many nonstandard schemes.

### V.34

28.8K, 26.4K, 24K, 21.6K, 19.2K, 16.8K, 14.4K, 12K, 9600, 7200, 4800, and 2400 bps asynchronous Trellis Coded Modulation (TCM)

### V.Fast Class (V.FC)

28.8K, 26.4K, 24K, 21.6K, 19.2K, 16.8K, 14.4K bps asynchronous Trellis Coded Modulation (TCM)

### U.S. Robotics V.32 terbo

21.6K, 19.2K, 16.8K, 14.4K. 12K, 9600, 7200 bps asynchronous, 19.2K, 16.8K, 14.4K. 12K, 9600, 7200 bps synchronous, Trellis Coded Modulation (TCM)

4800 bps, synchronous/asynchronous, Quadrature Amplitude Modulation (QAM)

### U.S. Robotics High Speed Technology (HST)

16.8K, 14.4K, 12K, 9600, 7200 bps, synchronous/asynchronous, asymmetrical, 450 bps back channel with automatic handshake adjustment to 300 bps, Trellis Coded Modulation (TCM), Quadrature Amplitude Modulation (QAM)

4800 bps, synchronous/asynchronous, asymmetrical, 450 bps back channel with automatic handshake adjustment to 300 bps, Quadrature Amplitude Modulation (QAM)

### ITU-T V.32 bis

14.4K. 12K, 9600, 7200 bps, synchronous/asynchronous, Trellis Coded Modulation (TCM)

4800 bps, synchronous/asynchronous, Quadrature Amplitude Modulation (QAM)

Additional Compatibility Features

ITU-T V.32, 9600 bps, synchronous, asynchronous, Trellis Coded Modulation (TCM); 4800 bps, synchronous, asynchronous, Quadrature Amplitude Modulation (QAM)

ITU-T V.25 2100 Hz tone

ITU-T V.23, 1200 bps, asymmetrical (1200/75 bps), Frequency Shift Keying (FSK)

ITU-T V.22 bis, 2400 bps, synchronous/asynchronous, Quadrature Amplitude Modulation (QAM)

ITU-T V.22, 1200 bps, synchronous/asynchronous, Differential Phase Shift Keying (DPSK)

Bell 212A, 1200 bps, synchronous/asynchronous, Differential Phase Shift Keying (DPSK)

Bell 103, 300 bps, asynchronous, Frequency Shift Keying (FSK)

ITU-T V.21, 300 bps, asynchronous, Frequency Shift Keying (FSK)

### Error Control Standards

ITU-T V.42 error control protocol at 14.4K, 12K, 9600, 7200, 4800 bps (V.32 bis mode) and at 2400/1200 bps

U.S. Robotics HST error control protocol, asymmetrical mode, at 16.8K, 14.4K, 12K, 9600, 7200, 4800 bps, 450/300 bps back channel

Microcom Networking Protocol (MNP) error control protocol, Levels 2-4 at 14.4K, 12K, 9600, 7200, 4800 bps (V.32 bis mode) and at 2400/1200 bps

### Data Compression Protocols

ITU-T V.42 bis data compression (all modes and speeds of 1200 bps and higher)

Microcom Networking Protocol (MNP) Level 5 data compression (all modes and speeds of 1200 bps and higher)

### Fax Standards

The Courier modem provides Group III-compatibility when combined with Class 1 or Class 2.0 fax software.  In addition, the modem adheres to the following standards.

TIA/EIA-578	Service Class 1 Asynchronous Facsimile DCE Control Standard

TIA/EIA-592		Service Class 2.0 Asynchronous Facsimile 
	DCE Control Standard

ITU-T V.17		14.4K/12K bps

ITU-T V.29		9600/7200 bps

ITU-T V.27 ter		4800/2400 bps

ITU-T V.21 		300 bps

## serial port Rates

115.2K, 57.6K, 38.4K, 19.2K, 9600, 4800, 2400, 1200, 300 bps

## adaptive speed leveling

21.6K, 19.2K, 16.8K, 14.4K, 12K, 9600, 7200, 4800 bps

## Phone Line Interface

RJ11, RJ45S phone jacks

Communications Channel

Full/half duplex on 2-wire dial-up, dedicated, or leased phone lines; demand-driven high speed channel turnaround in HST mode; symmetrical speeds in V.32 bis mode

Operational Modes

Synchronous/Asynchronous, Auto Dial/Answer, Manual Originate/Answer, Smart/Dumb mode, Auto Dial/Auto Answer, Auto Answer only, Forced Originate (MI/MIC)

Fax Modems:  The above modes plus fax mode

## DIALING

Dialing Rotary (pulse 0-9), Tone (DTMF 0-9, #, *), a-z when in Quote (") Mode

Data Format

Binary, serial; defaults to 8-bit word length, no parity, and 1 stop bit

Word	Parity	Stop

Length	(1 Bit)	Bits

7	Even, Odd	1

Mark, Space

7	None	2

8	None	1

## Front Panel status lights

HS	High Speed (above 2400 bps)

AA	Auto Answer/Answer

CD	Carrier Detect

OH	Off Hook

RD	Received Data

SD	Send Data

TR	Terminal Ready (DTR)

MR	Modem Ready/Test mode

RS	Request to Send

CS	Clear to Send

SYN	Synchronous mode

ARQ/	Error control connection established

FAX	Modem in Fax mode

## Flow Control Buffers

Transmit Buffer

Error control:  3.25k bytes

Non-Error control:  1.5k bytes, 128-byte option

Receive Buffer:  2K bytes

Command Buffer

60 characters, exclusive of AT prefix, Carriage Return and spaces

Test Options

Analog loopback with test pattern

Remote digital loopback

Digital loopback

Test pattern

Dial test

## Call Progress Codes

FAX

DATA

NO DIAL TONE

BUSY

NO ANSWER

RINGING

VOICE

## Failed Call Timeout

60-sec. default, programmable 2-255 sec.

Answer Tone Timeout

60 sec.

## Fax Service Class 1 Commands

+FCLASS=n (0,1)	Class identification and control

+FTS=n (0,255)	Stop transmission and pause, 10ms.

+FRS=n (0,255)	Wait for silence, 10 ms.

+FTM=n (3,24,48,72,73,74,96,121,122,145,146)
Transmit data with carrier

+FRM=n (3,24,48,72,73,74,96,121,122,145,146)
Receive data with carrier

+FTH=n (3,24,48,72,73,74,96,121,122,145,146)
Transmit HDLC data with carrier

+FRH=n (3,24,48,72,73,74,96,121,122,145,146)
	Receive HDLC data with carrier

## FAX service class 2.0 commands

Class 2.0 fax commands are too numerous to be listed here.  For information on Class 2.0 technical specifications, contact Global Engineering Documents, at 1-800-854-7179.  The document that covers this information is:

ANSI/EIA/TIA-592-1993 (EIA-592)

Asynchronous Facsimile DCE Control Standard

May, 1993

Optional Class 2.0 FAX commands supported

U.S. Robotics implements the following optional Class 2.0 fax commands :

+FNS=0,1		Pass-through non-Standard negotiation 				byte string

+FCR=0,1		Capability to receive

+FAA=0,1		Adaptive Answer mode

+FCT=0-255 sec.	Phase C Timeout

+FHS=0-255		Hangup Status Code, read only

+FMS=0-3		Minimum Phase C Speed

+FBS?=500,100	Buffer size, read only

## V.25 bis Synchronous Commands and Result Codes

Commands:  Connect incoming call (CIC); Call request using number provided (CRN); Call Request with memory location (CRS); Disregard incoming call (DIC); Program number (PRNn); Request list of forbidden numbers (RFN); Request list of stored numbers (RLN).

Dial options: 0-9 & : > < = P T);

Result Codes:  Call failure indication (CFI) with optional parameters:  Abort call (CFAB); local modem busy (CFCB); Engaged tone (CFET); Forbidden call (CFFC); Number not stored (CFNS); Answer tone not detected (CFNT); Ring tone (CFRT); Connect (CNX); List of numbers (LS); List of forbidden numbers (LSF); List of stored numbers (LSN); Incoming call (INC); Invalid (INV) with optional parameters:  Message syntax error (INVMS); Command Unknown (INVCU); Parameter syntax error (INVPS); Parameter value error (INVPV); Valid (VAL).

Commands and Result Codes not supported:  Call request with identification number (CRI); Program identifier (PRI); Request list of identification numbers (RLI); List of delayed call numbers (RLD).

## Answer Tone Detector

2200-2300 Hz

## Loss of Carrier (Disconnect Timer)

0.7-sec. default, programmable 0.2-25.5 sec.

Equalization

Adaptive

Transmitter Carrier Frequencies

V.34

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

Originate Mode:	1829 Hz

Answer Mode:	1829 Hz

Originate Mode:	1867 Hz

Answer Mode:	1867 Hz

Originate Mode:	1920 Hz

Answer Mode:	1920 Hz

Originate Mode:	1959 Hz

Answer Mode:	1959 Hz

Originate Mode:	2000 Hz

Answer Mode:	2000 Hz

V.Fast Class

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

Originate Mode:	1875 Hz

Answer Mode:	1875 Hz

Originate Mode:	1920 Hz

Answer Mode:	1920 Hz

USR-V.32 terbo/V.32 bis/V.32

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

USR-HST, 450 bps back channel

Originate Mode:	375 Hz

Answer Mode:	1800 Hz

USR-HST, 300 bps back channel

Originate Mode:	350 Hz

Answer Mode:	1800 Hz

V.23

Originate Mode:

Mark:	390 Hz

Space:	450 Hz

Answer Mode:

Mark:	1300 Hz

Space:	2100 Hz

V.22 bis, V.22, Bell 212A

Originate Mode:	1200 Hz

Answer Mode:	2400 Hz

Bell 103

Originate Mode:

Mark:	1270 Hz

Space:	1070 Hz

Answer Mode:

Mark:	2225 Hz

Space:	2025 Hz

V.21

Originate Mode:

Mark:	980 Hz

Space:	1180 Hz

Answer Mode:

Mark:	1650 Hz

Space:	1850 Hz

Receiver Carrier Frequencies

V.34

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

Originate Mode:	1829 Hz

Answer Mode:	1829 Hz

Originate Mode:	1867 Hz

Answer Mode:	1867 Hz

Originate Mode:	1920 Hz

Answer Mode:	1920 Hz

Originate Mode:	1959 Hz

Answer Mode:	1959 Hz

Originate Mode:	2000 Hz

Answer Mode:	2000 Hz

V.Fast Class

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

Originate Mode:	1875 Hz

Answer Mode:	1875 Hz

Originate Mode:	1920 Hz

Answer Mode:	1920 Hz

USR-V.32 terbo/V.32 bis/V.32

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

USR-V.32 terbo/V.32 bis/V.32

Originate Mode:	1800 Hz

Answer Mode:	1800 Hz

USR-HST, 450 bps back channel

Originate Mode:	1800 Hz

Answer Mode:	375 Hz

USR-HST, 300 bps back channel

Originate Mode:	1800 Hz

Answer Mode:	350 Hz

V.23

Originate Mode:

Mark:	1300 Hz

Space:	2100 Hz

Answer Mode:

Mark:	390 Hz

Space:	450 Hz

V.22 bis, V.22, Bell 212A

Originate Mode:	2400 Hz

Answer Mode:	1200 Hz

Bell 103

Originate Mode:

Mark:	2225 Hz

Space:	2025 Hz

Answer Mode:

Mark:	1270 Hz

Space:	1070 Hz

V.21

Originate Mode:

Mark:	1650 Hz

Space:	1850 Hz

Answer Mode:

Mark:	980 Hz

Space:	1180 Hz

## Receive Sensitivity

- 44 dBm + 2 dBm

## Transmit Level

- 9 dBm maximum

## Transmitter Frequency Tolerance

.01%

## Certification

FCC Part 68 and Part 15, Class B Domestic; IC (Canada), UL listed

Power Consumption

5 watts

## Size

6.375 x 10.3 x 1.337 inches