# CHAPTER 3.  Internal MODEM SET UP

## PACKAGE components

Your Courier modem package contains the following items:

•	The modem you purchased:  Courier V.34

•	An RJ11C phone cord

•	A power adapter

•	Fax software and manual

•	A Quick-Reference card

## What You Need

The Courier modem has minimal operational requirements.  Be sure to read the information in the front of this manual about radio and television interference and connecting to the phone company.  In addition, you should be aware of the following requirements.

Communications Software. . .

. . .if the modem is attached to a computer instead of a terminal.

The software uses the modem's AT command set to control many communications functions, including configuring the modem, dialing, and answering calls, and also enables the transfer of files and other operations.

Some users prefer their communications software to take control of the modem, and are more comfortable with a program that makes the modem almost transparent.   Others prefer a program that allows them to use the modem's AT command set sometimes, and their software at other times, depending on the task at hand.

Review Chapter 4 so that you have a basic understanding of the modem's requirements and operation.

A Telephone Adapter. . .

. . .if you have an older telephone installation that does not have the appropriate modular wall jack and plug.

Adapters and RJ11C connectors are available from your telephone company or computer dealer.

## You Should. . .

Refer to Appendix F. . .

. . .if your phone line is user-installed or if it is leased from the telephone company.

Refer to Appendix G. . .

. . . if your modem is installed in a Hewlett Packard system that uses the Ack/Enq communications protocol.

## BEFORE INSTALLING THE MODEM

A typical new modem user has a PC with a printer connected to the PC's parallel printer port, and a mouse cable attached to the PC's first serial port.  For this user, the modem is the only device that requires a second serial port.  If this is your situation, you can skip this entire section except for DIP Switches, just before Installing the Modem.  Just remember that after you load your communications software, you'll have to set the software to use the COM2 serial port.

IBM PC-compatible serial ports are referred to as COM ports.  Two COM ports are standard:  COM1 and COM2.  DOS recognizes up to four COM ports, although more can be programmed.  The Courier is set at the factory to use COM2.  This is because many computers are shipped with COM1 equipped with an external serial connector for attaching a device such as a serial printer or serial mouse.  If you have one or more of these devices, you're probably already familiar with COM ports on a PC.

If you're only using COM1 for a device, you can skip this section except for DIP Switches, just before Installing the Modem.  If you've already installed devices at both COM1 and COM2, you will have to select either COM3 or COM4.  Carefully review the following section for complete instructions.

Select a Serial Port

WARNING:  If you are going to use COM1, COM3, or COM4, you must change some switches on the modem board, called jumpers.  First, you need to select the correct Interrupt Request (IRQ) to use at that serial port.  Peripheral devices use IRQs to instruct the computer to stop the processor's current operation; this allows the devices to perform their operations.

Using COM1, 2, 3 or 4 depends on your equipment's configuration, as described below.  Remember which serial port you assign to the modem, because you must specify it to your
communications software later.  The table in the next section includes hexadecimal addresses, required by some software.

COM1:  You may use this port if the computer does not have an installed serial connector at COM1.  (This situation is unlikely.)  You will have to modify some switches on the modem, so be sure to read this entire section of the chapter.  However, you may prefer to leave the modem set to COM2 and proceed to DIP Switches, just before the installation instructions.

COM2:  If your computer is equipped with one serial connector at COM1, and if you are not already using COM2 for another device, use this port.  (This is the most common configuration.)  Since the modem is already set to operate at COM2, skip the rest of this section and proceed to DIP Switches, just before the installation instructions.

COM3, COM4:  If you have two serial devices that already use COM1 and COM2, you can set the modem for either COM3 or COM4.  Review your communications software documentation to be sure your program supports these additional serial ports.  (Most programs do.)  If your software doesn't, you will have to remove one of the other devices.

Select an Interrupt Request (IRQ)

IBM-compatible computers reserve IRQ4 for COM1 and IRQ3 for COM2, as shown below.  Some communication programs support reserved IRQs for two serial ports.

Serial	 	Serial Port
	Port	IRQ	Address (Hex)

COM1	IRQ4	3F8-3FF

COM2	IRQ3	2F8-2FF

COM3	 IRQ4*	3E8-3EF

COM4	 IRQ3*	2E8-2EF

*Select IRQ4 (COM3) or IRQ3 (COM4) only after reading the following guidelines:

If you need to use COM3 or COM4, keep the following in mind:

•	The Courier supports three additional IRQs:  IRQ2, IRQ5 and IRQ7.  Your software must support the one you use for COM3 or COM4.  However, if your computer is XT-compatible, IRQ5 is reserved for the hard disk, and it is not available for a serial port.  IRQ5 is available on 286-PCs and higher.

•	Two devices should not use the same IRQ at the same time, because there will be a conflict and you will probably lose data.  For example, if you use IRQ4 for a mouse installed at COM1 and for your modem installed at COM3 (as shown in the previous table) you cannot use the mouse and the modem at the same time.

Additionally, if an installed device doesn't use IRQs, you can use the IRQ normally associated with that COM port.  For example, if you have a serial printer at COM1 and you know it does not use an IRQ, you can use IRQ4 at COM3.  Similarly, you could use IRQ3 at COM4 if the device at COM2 doesn't use IRQ3.

If you cannot find the information you need in your software documentation, call your software's Technical Support Department for help in selecting an IRQ.  You may find it necessary to re-install your serial devices to correctly allocate the available serial ports and IRQs.

Reset the Jumpers

Figure 3.1  shows an enlarged view of the modem's jumper switches.  If you hold the modem so that the rear panel is at the bottom and the edge connector is at the left of the circuit board you can locate the COM and IRQ jumpers near the center left side of the board.

Each jumper has two upright contacts connected by a black plastic piece, called a shunt, placed over the contacts for the COM2 and IRQ3 settings.  This shunt selects the jumper by closing the circuit.

To change a jumper setting, gently lift off the black shunt and replace it over the contacts you want.

As we've said, your choice of an IRQ for COM3 or COM4 depends on your hardware/software configuration.  If the device at COM1 or COM2 isn't using IRQ4 or IRQ3, you may use one of them.  Otherwise, check your PC and software documentation to see if you may use IRQ2, IRQ5 or IRQ7.

WARNING:  Do not select an IRQ position until you've read the previous guidelines and reviewed your software documentation.

Figure 3.1¾Jumper Switch/Serial Port Settings

Set DIP Switches

A ten-position bank of Dual In-Line Package (DIP) switches is located at the rear of the modem.  A summary of the DIP switch functions and options is in Appendix B in this manual and on the Quick Reference Card.

WARNING:  Check your software documentation for its requirements, particularly for DIP switches 1, 4, 5 and 6.

NOTE:  Once the modem is installed, the DIP switches are accessible through the computer's rear panel.

## Installing the Modem

1.	Turn off the computer and peripheral devices.

2.	Remove the computer's cover.  Refer to the computer manual, if necessary, to see which rear panel screws to remove before sliding the cover off.

NOTE:  Our illustration shows expansion slots lined up vertically on the floor of the computer.  Some computers are configured so that the expansion slots are stacked horizontally, one on top of another.  The following instructions apply for both configurations.

Figure 3.2¾Removing the Computer Cover

3.	Unscrew the solid bracket at the back of any available expansion slot.  (The modem board requires at least a standard half-card slot.)

The bracket will pop out of the back, leaving an opening in the rear panel for access to the modem's phone jacks and DIP switches.

4.	Expansion slots have either a 3-inch-long groove, or a 3-inch and a 2-inch groove.  These grooves are lined on both sides with metal guides.  Insert the modem board into the slot you've chosen, as in Figure 3.3, with the leads on the modem board's edge connector firmly in the 3-inch groove.

Figure 3.3¾Inserting the Modem

5.	Screw the vertical bracket at the back of the modem firmly to the computer's rear panel, as shown in Figure 3.3.  This helps block radio frequency emissions from the computer and keeps the modem board firmly in place.

6.	Replace the cover of the computer and replace the screws.

7.	If you currently have a phone plugged into the wall jack, disconnect it.  Plug one end of the phone cable that came with the modem into the TELCO jack at the rear of the modem.  This allows the modem to switch into the telephone network, get a dial tone, and so on.  Plug the other end of the cable into the wall jack.

NOTE:  The phone cable is equipped with two standard, modular RJ11C phone connectors.  If you have an older type of wall jack, you can purchase an adapter and RJ11C connector from your phone company or computer dealer.

8.	If you wish, plug your phone's cord into the second jack on the modem, labeled PHONE.  This enables you to use your phone for conventional voice calls, and also allows you to switch between voice and data transmission in the same call.  See Voice/Data Communications in Appendix G for more information.

## Testing the Installation

Use the following procedures to verify that your modem is working properly.

1.	Turn on your computer.  Then turn the Courier's power switch ON.  The following front panel indicators, or LEDs, will light up on the modem.

CD	Carrier Detect, if you have set DIP switch 6 ON, enabling the CD override

TR	Data Terminal Ready, if you have set DIP switch 1 ON, enabling the DTR override

MR	Modem Ready/Power ON

CS	Clear to Send

2.	If you're using a personal computer, load your software to start the program.  Set your terminal or software to 19.2K bps or 9600 bps.  In addition, set a word length of either 7 bits plus 1 parity bit, or 8 bits with no parity—it doesn't matter which at this time—and 1 Stop bit.  Set the software to use the correct serial port (port through which the modem connects to the computer).

3.	Perform the function that lets you send AT commands to the modem, that is, puts the computer in Terminal mode.  Some communications programs do this automatically upon loading.  Others require you to display a communications or terminal screen, type a Function key, or perform some other operation.

Review your communications software documentation for instructions.

4.	Send the following attention command.  Type either upper or lower case letters, not a combination, and then press the Enter (Carriage Return) key, shown in the example below between angle brackets.  (Don't type the angle brackets.)

AT <Enter>

If everything is correct, the modem responds as follows:

OK

Go on to step 5.

Troubleshooting

If your entered command is not displayed, your local echo is OFF.  To turn the local echo ON, send the modem the following command:

ATE1 <Enter>

If double characters appear on the screen, both your modem and software are set to local echo ON.  Either set your software to local echo OFF, or turn the modem's echo OFF with the following command:

ATE0 <Enter>

If no OK appears on your screen after you completed Step 4, review the following checkpoints:

a.	Make sure you type all upper or lower case letters and press <Enter>.

b.	Check to see that you set your communications software to the correct serial port.  The correct serial port is the port to which the modem is connected to the computer.

c.	Make sure your software has put the computer in Terminal mode, so that you can send the modem commands.  Then review Step 4, on the previous page.

d.	Be sure that DIP switches 1 and 6 are set ON or OFF according to your terminal or software requirements.  The table in Appendix B explains each function, and you may also need to review your terminal or communications software documentation.

e.	If you set DIP switch 8 OFF, for Dumb mode, reset the modem to Smart mode:  set DIP switch 8 to ON.

f.	The modem is shipped with DIP switch 3 ON, enabling the result codes.  If DIP switch 3 is OFF, set it to the ON position.  Then initiate the new switch setting with the following reset command:

ATZ <Enter>

5.	As a final check, make sure the modem gets a dial tone.  Type the following Dial command:

ATD <Enter>

On receipt of the command, the modem goes off hook and waits for a dial tone.  The OH indicator lights up, and you'll hear the dial tone from the modem's speaker.  To cancel the operation, press any key.

Troubleshooting

If you don't hear the dial tone, first increase the volume by using the L command.  If that doesn't work, check to see that the phone cable from the wall jack is connected to the correct jack on the modem.  If necessary, reconnect the phone cable correctly.  Then try the Dial command again, ATD <Enter>.