******************************************************************************

���ୠ� ⠡��� ����� �訡�� BIOS

�� Phoenix AMI & Award

******************************************************************************

1. PHOENIX BIOS Trouble Shooting Error Codes

During power on self test (POST) a status port at adress 80H is used to

display the test in progress. If a fatal error is encountered during POST

the CPU will halt leaving the failing test number on the Hex display.

Also, a failure code will be sounded on the speaker.

Below is a list of the valid audible and Hex codes.

Error Codes

Code    Beep    Description

01H     none    80286 register test in progress

02H     1-1-3   CMOS write/read failure

03H     1-1-4   ROM BIOS checksum failure

04H     1-2-1   Programmable interval Timer failure

05H     1-2-2   DMA initialization failure

06H     1-2-3   DMA Page register write/read failure

08H     1-3-1   RAM refresh verification failure

09H     none    1st 64k RAM test in progress

0AH     1-3-3   1st 64k RAM chip or data line failure multi-BIT

0BH     1-3-4   1st RAM odd/even logic failure

0CH     1-4-1   Address line failure 1st 64k RAM

0DH     1-4-2   Parity failure 1st 64k RAM

10H     2-1-1   Bit 0 1st 64k RAM failure

11H     2-1-2   Bit 1 1st 64k RAM failure

12H     2-1-3   Bit 2 1st 64k RAM failure

13H     2-1-4   Bit 3 1st 64k RAM failure

14H     2-2-1   Bit 4 1st 64k RAM failure

15H     2-2-2   Bit 5 1st 64k RAM failure

16H     2-2-3   Bit 6 1st 64k RAM failure

17H     2-2-4   Bit 7 1st 64k RAM failure

18H     2-3-1   Bit 8 1st 64k RAM failure

19H     2-3-2   Bit 9 1st 64k RAM failure

1AH     2-3-3   Bit A 1st 64k RAM failure

1BH     2-3-4   Bit B 1st 64k RAM failure

1CH     2-4-1   Bit C 1st 64k RAM failure

1DH     2-4-2   Bit D 1st 64k RAM failure

1EH     2-4-3   Bit E 1st 64k RAM failure

1FH     2-4-4   Bit F 1st 64k RAM failure

20H     3-1-1   Slave DMA register failure

21H     3-1-2   Master DMA register failure

22H     3-1-3   Master interrupt mask register failure

23H     3-1-4   Slave interrupt mask register failure

25H     none    Interrupt vector loading in process

27H     3-2-4   Keyboard controller test failure

28H     none    CMOS power failure and checksum calculation in progress

29H     none    CMOS configuration validation in progress

2BH     3-3-4   Screen memory test failure

2CH     3-4-1   Screen initialization failure

2DH     3-4-2   Screen retrace test failure

2EH     none    Search for video ROM in progress

2FH     none    Screen believed operable

30H     none    Screen believed running with video ROM

31H     none    Monochrome monitor believed operable

32H     none    Color monitor (40 column) believed operable

33H     none    Color monitor (80 column) believed operable

Non-fatal System Board Errors

34H     4-2-1   No time tick

35H     4-2-2   Shutdown failure

36H     4-2-3   Gate A20 failure

37H     4-2-4   Unexpected interrupt in protected mode

38H     4-3-1   Memory high adress line failure at 01000H-0A000H

39H     4-3-2   Memory high adress line failure at 100000H-FFFFFFH

3AH     4-3-3   Timer chip counter 2 failed

3BH     4-3-4   Time-of-day clock stopped

This Hex display is also useful as an aid to application hardware

and software development.

PHOENIX is a tademark of PHOENIX TECHNOLOGIES LTD.

*******************************************************************************

2. AMI BIOS List of checkpoints

Check Point     Meaning

01          NMI Disabled and 286/386 register about to start

02          286 Register test over

03          ROM Checksum OK

04          8259 Initialization OK

05          CMOS Pending interrupt disabled

06          Video disabled and system timer counting OK

07          CH-2 of 8253 test OK

08          CH-2 delta count test OK

09          CH-1 delta count test OK

0A          CH-0 delta count test OK

0B          Parity status cleared

0C          Refresh and System timer OK

0D          Refresh link toggling OK

0E          Refresh period ON/OFF 50% OK

10          Confirmed Refresh ON and start 64K memory test

11          Address line test OK

12          64K Base memory test OK

13          Interrupt vectors initialized

14          8242 Keyboard controller test OK

15          CMOS Read/Write test OK

16          CMOS Checksum/battery OK

17          Monochrome mode set OK

18          Color mode set OK

19          About to look optional video ROM

1A          Optional video ROM control OK

1B          Display memory read/write test Ok

1C          Display memory r/w test for alternate display OK

1D          Video retrace check OK

1E          Global equipment byte set for video OK

1F          Mode set call for Mono/Color OK

20          Video test OK

21          Video display OK

22          Power on message display OK

30          Virtual mode memory test about to begin

31          Virtual mode memory test started

32          Processor in virtual mode

33          Memory address line in progress

34          Memory address line in progress

35          Memory below 1MB calculated

36          Memory size computation OK

37          Memory test in progress

38          Memory initialization below 1MB over

39          Memory initialization over 1MB over

3A          Display memory size

3B          About to start below 1MB memory test

3C          Memory test below 1MB OK

3D          Memory test below 1MB OK

3E          About to go to real mode (SHUTDOWN)

3F          Shutdown successful and entered in real mode

40          About to disable GATE A20 address line

41          GATE A20 line disabled successfully

42          About to start DMA Controller test

4E          Address line test OK

4F          Processor in real mode after shutdown

50          DMA Page register test OK

51          DMA unit-1 base register test about to start

52          DMA unit-1 channel Ok, about to begin CH-2

53          DMA channel 2 base register test OK

54          About to test F/F latch for Unit 1

55          F/F latch test for both units OK

56          DMA unit 1 and 2 programmes OK

57          8259 initialization over

58          8259 mask register check OK

59          Master 8259 mask register OK

5A          About to check timer and keyboard interrupt level

5B          Timer interrupt OK

5C          About to test keyboard interrupt

5D          Error Timer/Keyboard interrupt not in proper level

5E          8259 interrupt controller error

5F          8259 interrupt controller tset OK

70          Start of keyboard test

71          Keyboard BAT test OK

72          Keyboard test OK

73          Keyboard global data setup OK

74          Floppy setup about to begin

75          Floppy setup OK

76          Hard disk setup about to start

77          Hard disk setup OK

79          About to initialize timer data area

7A          Verify CMOS battery power

7B          CMOS battery verification done

7D          About to analyze diagnostics memory test results

7E          CMOS memory size update OK

7F          About to check optional ROM at C000:0

80          Keyboard sensed to enable setup

81          Optional ROM control OK

82          Printer global data initialization OK

83          RS-232 global data initialization OK

84          80287 check/test OK

85          About to display soft error messages

86          About to give control to system ROM at E000:0

87          System ROM E000:0 check over

00          Control given to INT 19, Boot loader

AMI is a trademark of American Megatrends INC.

*******************************************************************************

3. AWARD POST CODES

These test numbers are output to the defined port (normally port 80H

for the IBM BIOS and additionally 300H for the Award Modular POST BIOS)

during the POST. The following POSTal codes are valid for the Award

Modular BIOS with POST versions 3.00 and up.

POST Code   Description

HEX DEC PROCESSOR TEST 1: PROCESSOR STATUS VERIFICATION

INPIT: NONE

OUTPUT: INFINITE LOOP IF FAILED CONTINUE TEST IF OK

REGISTERS: AX, BP

2.  2.  determine type of post test

FAILED IF KEYBOARD INTERFACE

BUFFER FILLED WITH DATA

INPUT:NONE

OUTPUT: INFINITE LOOP IF FAILED CONTINUE TEST IF OK

REGISTERS: AX, BX, BP

3.  3.  CLEAR 8042 INTERFACE

INPUT: NONE

OUTPUT:INFINITE LOOP IF FAILED CONTINUE TEST IF OK

REGISTERS: AX, BX, BP

4.  4.  RESET 8042

INPUT: NONE

OUTPUT:INFINITE LOOP IF FAILED CONTINUE TEST IF OK

REGISTERS: AX, BX, BP

5.  5.  GET 8042 MANUFACTURING STATUS

READ INPUT PORT VIA KB CNTRL TO DETERMINE MANUF OR NORMAL MODE

OPERATION.

INPUT: NONE

OUTPUT:INFINITE LOOP IF FAILED CONTINUE TEST IF OK

REGISTERS: AX, BX, BP

6.  6.  INIT CHIPS (DMA, 8259, CMOS)

7.  7.  PROCESSOR TEST #2

8.  8.  INITIALIZE CMOS CHIP

9.  9.  EPROM CHECKSUM FOR 32 KBYTES

A.  10. INITIALIZE VIDEO INTERFACE

B.  11. TEST 8254 CHANNEL 0

C.  12. TEST 8254 CHANNEL 1

D.  13. TEST 8254 CHANNEL 2

E.  14. TEST CMOS DATE AND TIMER

F.  15. TEST CMOS SHUTDOWN BYTE

10. 16. TEST DMA CHANNEL 0

11. 17. TEST DMA CHANNEL 1

12. 18. TEST DMA PAGE REGISTERS

13. 19. TEST 8741 KEYBOARD CONTROLLER

14. 20. TEST MEMORY REFRESH TOGGLE CIRCUITS

15. 21. TEST 1ST 64K BYTES OF SYSTEM MEMORY

16. 22. SETUP INTERRUPT VECTOR TABLE

17. 23. SETUP VIDEO I/O MEMORY

18. 24. TEST VIDEO MEMORY

19. 25. TEST 8259 CHANNEL 1 MASK BITS

1A. 26. TEST 8259 CHANNEL 2 MASK BITS

1B. 27. TEST CMOS BATTERY LEVEL

1C. 28. TEST CMOS CHECKSUM

1D. 29. SETPU CONFIGURATION BYTE FROM CMOS

1E. 30. SIZING SYSTEM MEMORY & COMPARE W/CMOS

1F. 31. TEST FOUND SYSTEM MEMORY

20. 32. TEST STUCK 8259 INTERRUPT FUNCTIONALITY

21. 33. TEST STUCK NMI (PARITY/IO CHK) BITS

22. 34. TEST 8259 INTERRUPT FUNCTIONALITY

23. 35. TEST PROTECTED MODE AND A20 GATE

24. 36. SIZING EXTENDED MEMORY ABOVE 1MB

25. 37. TEST FOUND SYSTEM/EXTENDED MEMORY

26. 38. TEST EXCEPTIONS IN PROTECTED MODE

27. 39. RESERVED

28. 40. RESERVED

If a failure occures during Post test 6 thru 24 (unless the error causes

the computer to hang in the test), the system will keep outputing the

Post sequance to the defined POST PORT(8). A normal error message will

then be displayed on screen.

IBM is a registered trademark of International Business Machines Corporation,

Modular BIOS is a trademark of Award Software, Inc.

*****************************************************************************