M1429L : 32-Bit VESA Green Single Chip Solution·	Super integrated ASIC for 32-bit CPU Green chip, built-in PMU

·	Supports Intel, AMD, Cyrix, IBM, TI SMM features

·	Supports TTL buffer chip (M1431) to reduce TTL components

·	Four system states for power savings (On/ Doze/ Standby/ Suspend)

·	Full hardware power saving control without software limitation

·	Supports VESA & PCI local bus implementation (ready ALI M1435 PCI bridge interface)

FeaturesSUPPORTED PROCESSOR TYPES ·	386DX  486  P24C  P24D  P24T  TI486SXL  486DLC  M6  M7  Blue-Lightning·	Intel S-series, Cyrix, AMD, IBM, TI SMM featureSUPPORTED SMM & PMU FEATURES·	Supports CPU SMM mode, SMI feature·	Supports STPCLK control with clock scaling & clock throttling feature·	Supports I/O trap for I/O restart feature

·	Supports the APM control·	Supports the EXTSMI control ·	Supports 4 system states for power savings :1) ON      2)	DOZE 	3)	STANDBY 	4)	SUSPEND

·	Supports the programmable flash LED control for indicating the power saving state status·	Supports peripheral devices activity monitoring of each state for power saving: KBD, VGA, HDD, LPT, COM, FDD, IRQs, DRQs, VESA master & 2 programmable regions·	Supports 7 timers from 1s ~ 320 mins to individually monitor the STATE, VGA, HDD, LPT, programming region 1, programming region 2, and standard input devices' activity ·	Supports external switch, RTC alarm & MODEM ring-in wake-up control·	Supports 12 power control signals for peripheral devices SUPPORTED FAST LOCAL BUS ·	VESA BUS , n SLAVE and 2 MASTER:

- Allows ISA MASTER/DMA  access to LOCAL DEVICE	 - Allows LOCAL MASTER access to ISA DEVICE·	PCI BUS,  supports interfacing signals to ALI M1435 PCI bridgeOPERATING FREQUENCY ( 1X )

20MHz, 25MHz, 33MHz, 40MHz, 50MHz and 66MHz

SUPPORTED  WEITEK COPROCESSOR INTERFACE:Weitek 3167 for 386 systems;  Weitek 4167 for 486 systemsL1 CACHE FEATURES·	Supports L1 write-through feature of 486 systems·	Supports L1 write-through/write-back feature of P24T/ P24D/ M6/M7 systems·	Supports EN/DISable on L1 write-back for entire local memory space of P24T/P24D/M6/M7 systems·	Supports optional enable write-back in region >= 16M of P24T/P24D/M6/M7 ( for ISA compatibility issue )·	Cacheable to BIOS code R/ with default of write-through by P24T/P24D/M6/M7, non-cacheable to BIOS data R/W to protect or write-through to BIOS data R/WL2 CACHE FEATURES·	L2 write-back or write-through, but no write-buffered for one-bank or two-bank;   SRAM & TAG RAM testable cache size up to 1M byte;   cacheable region up to 128M, TAG bit with 1 Modify (Dirty) bit, TAG bit and Modify bit in one SRAM, 128 bit line size for both 386/486/P24D/P24T.DRAM FEATURES·	Supports 256K, 512K, 1M, 4M, 16M*1 with 80ns DRAM specifications·	Supports 2 banks/4 banks structure and any combination of each bank·	Page mode only·	HIDDEN refresh/SLOW refresh/NORMAL refresh·	Supports on-board memory size up to 128M byte·	Spilt memory remappingMISCELLANEOUS·	Built-in ISP controller & RTC·	Software AT clock programmable·	Supports Flash, HI-ROM ·	Supports I/O recovery ·	Fast RC, gate A20 emulation·	Supports 8/16 ROM

# Table of Contents

µSection1: Introduction

1.1	Features and Functions	3

1.2	Operating Speeds	3

1.3	Block Diagram	4

Section 2:  Pin Description

2.1	Pin Diagram	5

2.2	Pin Description Table	6

2.3	Numerical Pin List	14

2.4	M1429L Hardware Setting Control 	18

2.5	Hardware Setting Configuration	19

Section 3: Function Description

Section 4: Configuration Registers

Section 5: Programming Guide

Section 6: Design Issues

Section 7: Electrical Characteristics

7.1	Timing Diagrams	73

7.2	AC Characteristics	114

7.3	DC Characteristics	118

Section 8: Packaging Information

# Section1 : Introduction

## 1.1	Features and Functions

The M1429L provides a single-chip solution to support VESA standards, power management features, and the most advanced, currently available CPUs in the market today. It is a new generation, single-chip solution that offers the most cost-effective system integration for 386 to P24T/D systems.

With the M1431 TTL ASIC buffer, it allows the system to run on 386, 486 and P24T/Ds and supports local bus, SMM and PMU architecture. And with the M1435 PCI bridge, it supports PCI bus architecture.

## 1.2	Operating Speeds

20MHz, 25MHz, 33MHz,  40MHz,  50MHz and  66MHz

## 1.3	Block Diagram

Figure 1-1	M1429L Block Diagram

# Section 2:  Pin Description

## 2.1	Pin Diagram

Figure 2-1	M1429L Pin Diagram

## 2.2	Pin Description Table

Table 2-1	M1429L Pin Description Table

| Name | Number | Type | Driver (IOL/IOH) | Description |
| --- | --- | --- | --- | --- |
| HOST bus interface ( based on the upgradable 486 and P24T systems) |  |  |  |  |
| XGA31 | 74 | I/O- TTL-CMOS | 4/4mA | Address 31. This is the processor address line 31. It recognizes the coprocessor cycle. It drives low during ISA MST/DMA cycles. |
| XGA27 | 70 | I/O- TTL-CMOS | 4/4mA | Address 27. This is the processor address line 27. It distinguishes memory access cycles above 128M. It drives low during ISA MST/DMA cycles. |
| XGA[26:24] | 106, 104-103 | I/O- TTL-CMOS | 4/4mA | Address[26:24]. These addresses decode LOCMEM and ENBK. They are inputs for CPU cycles and are driven low-output for DMA,VESA/ISA MASTER and REFRESH cycles. |
| XGA[23:2] | 102-91, 89-80 | I/O- TTL-CMOS | 8/8mA | Address[23:2]. These addresses are connected to CPU addresses. They decode for LOCMEM and ENBK. They are inputs for CPU and VESA/ISA MASTER cycles,and are driven output for DMA and REFRESH cycles. |
| XBE[3:0]J | 76-79 | I/O-TTL/CMOS | 8/8mA | Byte Enable. These are the byte enable signals for the data bus. These signals are ignored during the local memory read access cycle, and the data of all bytes are treated as enable to access. In the ISA MASTER/DMA accesses, the VESA local slave cycle, these byte enable signals are driven by M1429L to host bus. |
| XADSJ | 67 | I/O- TTL-CMOS | 8/6mA | Address Strobe. The address status indicates that a new valid bus cycle is currently being driven by the CPU/VESA MASTER. In the ISA MASTER/DMA accesses, the VESA local slave cycle, this signal are driven by M1429L to host bus. |
| XM/IOJ,D/CJ,W/RJ | 54-56 | I/O- TTL-CMOS | 8/6mA | Memory or I/O, Data or Code, Write or Read. The memory/input-output, write/read, and data/code lines are the CPU bus cycle definition pins. In the ISA MASTER/DMA accesses the VESA local slave cycle, these signals are driven by M1429L to host bus. |
| XRDYOJ | 57 | O-CMOS | 8/6mA | Ready Out. The non-burst ready output indicates that the current bus cycle is complete. This signal is also the RDYRTNJ of the VESA devices. |
| XBRDYJ | 58 | O-CMOS | 12/8mA | Burst Ready Out. The burst ready output performs the same function during a burst cycle that RDYOJ performs during a non-burst cycle. |
| XPCD | 60 | I-TTL |  | Page Cache Disable. The page cache disable pin indicates that the current CPU bus cycle is non-cacheable. This signal is not available in the P24T system. |
| XCACHEJ | 60 | I-TTL |  | Cache. This cache cycle pin indicates the cacheability of a cycle for the P24T processor. This signal pin is only available in the P24T system. |
| XBUSY7J | 60 | I-TTL |  | Busy. This is the busy condition signal from an INTEL compatiable coprocessor. This signal pin is only available in 386 system. |
| XLOCKJ | 61 | I-TTL |  | Lock. The bus lock pin indicates that the current CPU bus cycle is locked. This pin is not available in 386 system. |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name | Number | Type | Driver (IOL/IOH) | Description |
| --- | --- | --- | --- | --- |
| HOST bus interface ( based on the upgradable 486 and P24T system ) |  |  |  |  |
| XPEREQ7 | 61 | I-TTL |  | Processor Extension Request. This is the date transfered request from an INTEL compatiable coprocessor. This signal pin is only available in 386 system. |
| XHITMJ | 69 | I-TTL |  | Hit Modify. The hit modified cache pin is asserted to indicate that a hit to a modified data cache occurred. This signal pin is only available in write-back processor systems such as P24T, M6, and M7. |
| XWINTR | 69 | I-TTL |  | Weitek interrupt. This is the Weitek coprocessor interrupt request signal. This signal pin is only available in 386 systems. |
| XBOFFJ DDDDDDDDDDDDDDDD | 68 | I/O- TTL-CMOS | 4/4mA | Backoff. This backoff signal pin forces the processor to float its bus in the next clock. This signal is not available in 386/M6 systems. |
| CALATCHJ | 68 | I/O- TTL-CMOS | 4/4mA | Cache Address Latch. This signal latches address for cache is address hold time is not enough. This signal is available in 386 systems. |
| BUSY7J | 68 | I/O- TTL-CMOS | 4/4mA | Busy. This is the busy condition signal from an Intel compatible coprocessor. This signal pin is only available in M6 system. |
| GD[31:0] | 15-30,32-39,41-48 | I/O- TTL-CMOS | 8/8mA | Data Bus [31:0]. These are the 32 data lines. |
| PD[3:0] | 49-52 | I/O- TTL-CMOS | 4/4mA | Parity Data Bus. These are the memory parity data lines. |
| PCHKJ | 67 | I-TTL |  | Parity Check Status. This input signal indicates the CPU parity check status. This signal is not available in 386 systems. |
| BS16J | 67 | I-TTL |  | 16-Bit Cycle. This is the signal intput from VESA MASTER that indicates that the current cycle is a 16-bit cycle. This signal is available in 386 systems. |
| HOLD | 65 | O-CMOS | 4/4mA | CPU Hold Request. This is the bus hold request signal. |
| HLDA | 66 | I-TTL |  | Hold Acknowledge. This is the bus hold acknowledge signal. |
| KENOJ | 62 | O-CMOS | 4/4mA | Cache Enable Output. This is the cache enable signal. This signal is not available in 386 systems. |
| FLUSHJ | 64 | O-CMOS | 4/4mA | Cache Flush Output. The cache flush signal forces the CPU to flush the entire internal cache. This signal is not available in 386 systems. |
| PEREQ6 | 64 | I/O- TTL-CMOS | 4/4mA | Processor Extension Request. This is the date transfered request to an Intel compatiable processor. This signal pin is only available in 386 systems. |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name |  | Number | Type |  | Driver (IOL/IOH) |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HOST bus interface ( based on the upgradable 486 and P24T system ) |  |  |  |  |  |  |  |  |
| XEADSJ | 63 |  |  | O-CMOS |  | 4/4mA |  | External Address Strobe. This signal indicates that a valid external address has been driven onto the CPU address pins to be use for a bus snooping cycle. It is not available in 386 systems. |
| XA20MJ | 4 |  |  | I/O- TTL-CMOS |  | 8/6mA |  | Address 20 Mask. This is the Address line 20 mask signal. It is not available in 386 system. |
| XSLA20 |  | 4 | I/O- TTL-CMOS |  | 8/6mA |  | Slot Address 20. This is the slot address line A20 signal. This signal pin is only available in 386 systems. |  |
| XFERRJ |  | 8 | I-TTL |  |  |  | Floating Point Error. This is the floating point error indicator signal. It is not available in M6/386 systems. |  |
| XERR7J |  | 8 | I-TTL |  |  |  | Coprocessor Error. This is the error condition signal from an Intel compatiable coprocessor. This signal pin is only available in 386/M6 systems. |  |
| XIGNNEJ |  | 6 | O-CMOS |  | 4/4mA |  | Ignore Numeric Error. This is the ignore numeric error indicator signal. It is not available in M6/386 system. |  |
| XBUSY6J |  | 6 | O-CMOS |  | 4/4mA |  | Busy. This is the busy condition signal to an Intel compatiable coprocessor. This signal pin is only avaiable in M6/386 systems. |  |
| XINTR |  | 10 | O-CMOS |  | 4/4mA |  | Interrupt Request. This is the interrupt request signal generated by the internal 8259. It is synchronalized with CLKI. |  |
| XNMI |  | 12 | O-CMOS |  | 4/4mA |  | Non-Maskable Interrupt. This is the non-maskable interrupt request signal. |  |
| VESA Interface |  |  |  |  |  |  |  |  |
| XLREQ0J |  | 74 | I/O-TTL-CMOS |  | 24/12mA |  | Local Request. This signal is used for the VESA bus master to request the ownership of the host bus. This signal pin is not available when hardware setting is setup to a 4-bank DRAM mode. |  |
| XRAS3J |  | 74 | I/O-TTL-CMOS |  | 24/12mA |  | Row Address Strobe 3. This is the memory module row address strobe signal. The signal is applied to bank 3 of DRAM. This signal is only available when hardware setting is setup to a 4-bank DRAM mode. |  |
| XLGNT0J |  | 73 | O-CMOS |  | 24/12mA |  | Local Grant. This signal grants the host bus ownership to the VESA bus master. This signal pin is not available when hardware setting is setup to 4-bank DRAM mode. |  |
| XRAS2J |  | 73 | O-CMOS |  | 24/12mA |  | Row Address Strobe 2. This is the memory module row address strobe signal. The signal is applied to bank 2 of DRAM. This signal is only available when hardware setting is setup to a 4-bank DRAM mode. |  |
| XLREQ1J |  | 154 | I/O-TTL-CMOS |  | 8/6mA |  | Local Request. This signal is used for the VESA bus master to request ownership of the host bus. This signal pin is available when hardware setting is setup to a 4-bank DRAM mode and RTCSJ mode or 2-VESA MASTER mode is disabled. |  |
| XRTCAS |  | 154 | I/O-TTL-CMOS |  | 8/6mA |  | RTC Address Strobe. This is the RTC address strobe signal. This signal pin is not available when hardware setting is setup to a 4-bank DRAM mode and RTCSJ mode or 2-VESA MASTER mode is disabled. |  |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name | Number |  | Type |  | Driver (IOL/IOH) |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VESA Interface |  |  |  |  |  |  |  |  |
| XLGNT1J | 155 |  | O-CMOS |  | 8/6mA |  | Local Grant. This signal is grants host bus ownership to the VESA bus master. This signal pin is available when hardware setting is setup to a 4-bank DRAM mode and RTCSJ mode or 2-VESA MASTER mode is disabled. |  |
| XADIR | 155 |  | O-COMS |  | 8/6mA |  | Address Direction Control. This is the slot address buffer direction control signal. This signal pin is not available when hardware setting is setup to a 4-bank DRAM and RTCSJ mode or 2-VESA MASTER mode is disabled. |  |
| XLNPRDYJ | 72 |  | I-TTL |  |  |  | Local Devices/Numeric Process Ready. The coprocessor/VESA slave ready indicator signal terminates the current coprocessor/VESA slave cycle. |  |
| XLDEVJ |  | 71 |  | I-TTL |  |  |  | Local Devices. This active low signal indicates that the current cycle is a VESA local device access cycle. |
| ON-BOARD and L2-CACHE controller interface |  |  |  |  |  |  |  |  |
| XRAS[1:0]J |  | 130,131 |  | O-CMOS |  | 24/12mA |  | Low Address Strobe. These are the memory module row address strobe signals. The RASJ0 applies to bank 0 memory module and RASJ1 applies to bank 1. |
| XCAS[3:0]J |  | 132-135 |  | O-CMOS |  | 24/12mA |  | Column Address Strobe. These are the memory module column address strobe signals. The CASJ0 applies to byte 0, CASJ1 applies to byte 1, CASJ2 applies to byte 2, and CASJ3 applies to byte 3. |
| XMA[11:0] |  | 138-149 |  | I/O-TTL-CMOS |  | 24/12mA |  | Memory Address. These are memory address signals. These signals change to PWCNTL0-PWCNTL11( power control signals ) during the power-control output cycle. These pins are input polarity during the power-on reset period for initializing the power-control default value of the internal ports. |
| XMWEJ |  | 136 |  | O-CMOS |  | 24/12mA |  | Memory Write Enable. This is the memory write enable signal. |
| XCOE[1:0]J |  | 109-110 |  | O-CMOS |  | 8/8mA |  | Cache Output Enable. These are the cache output enable signals with which the single chip controls the external 2 bank cache system. COEJ0 applies to bank 0 cache and COEJ1 applies to bank 1. |
| XCWE[1:0]J |  | 111,112 |  | O-CMOS |  | 8/8mA |  | Cache Write Enable. These are the cache write enable signals. CWEJ0 applies to bank 0 cache and CWEJ1 applies to bank 1 cache. |
| XCS[3:0]J |  | 114-117 |  | O-CMOS |  | 8/8mA |  | Cache Chip Select. These are the cache system chip select signals. |
| XTAGWEJ |  | 120 |  | O-CMOS |  | 8/8mA |  | Tag Write Enable. This is the tag RAM write enable signal. |
| XTAG[7:0] |  | 121-128 |  | I/O-TTL-CMOS |  | 8/6mA |  | Tag Data. These are the cache tag bits data. |
| XCA3IO |  | 118 |  | O-CMOS |  | 12/8mA |  | Cache Address 3. This is the cache address line 3 for bank 0 cache. |
| XCA3IIO |  | 119 |  | O-CMOS |  | 12/8mA |  | Cache Address 3. This is the cache address line 3 for bank 1 cache. |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name | Number | Type | Driver (IOL/IOH) | Description |
| --- | --- | --- | --- | --- |
| Clock & Reset Signals |  |  |  |  |
| XOSC | 208 | I-TTL |  | 14.318MHz Input. This is the 14.318MHz clock input signal. |
| XCLKI | 1 | I-TTL |  | Clock Input. This is the operating clock input. It generates the ISA clock ATCLK1. It also generates CLK2XO and half clock CLK1XO with the normal mode and power-down mode function. |
| XCLK2XO | 7 | O-CMOS | 12/12mA | Clock Output. This is the operating clock output for sourcing the 2X host system. In normal mode, this signal is the buffered CLKI output. In power-down mode, this signal is the power-down 2X clock output. |
| XCLK1XO | 11 | O-CMOS | 12/12mA | Clock Output. This is the operating clock output for sourcing the 1X host system. In normal mode, this signal is the buffered half CLKI output. In power-down mode, this signal is the power-down 1X clock output. |
| XFBCLK2 | 9 | I-CMOS |  | Feedback Clock. This clock input sources the M1429L internal CPU/L2 CACHE/MEMORY control interface and state machines. It connects to CLK2XO-like clock signal in real application. |
| XFBCLK1 | 13 | I-CMOS |  | Feedback Clock. This clock input is used by the M1429L internal CPU/L2 CACHE/MEMORY control interface and state machines to recognize the phase relation. It connects to CLK1XO-like clock signal in real application. |
| XRSTCPU | 5 | O-CMOS | 8/6mA | Reset Processor. This signal resets the processor, coprocessor, and all local devices. It may not be available for the P24T/M6/M7 CPU because of performance issue. |
| XINIT | 5 | O-CMOS | 8/6mA | Initial. This signal is the warm-reset for the P24T/M6/M7 CPU. |
| XSRESET | 5 | O-CMOS | 8/6mA | Soft Reset This signal forces the CPU to the SMAE state as RESET except that the SMRAM Relocation address (SMBASE) remains at its current value. |
| XPWGD | 207 | I-TTL |  | Power Good. This is the power good input signal and it is a schmitt trigger pad signal. It comes from the power supply to indicate that power is available and stable. |
| ISA SYSTEM interface |  |  |  |  |
| XATCLK | 3 | O-CMOS | 20/8mA | System Clock Output. This is the AT clock output signal. This clock can be set to CLKI divided by 2, 3, 4, 5, 6, 8 ,10 or 7.159 MHz. |
| XSYSRST | 2 | O-CMOS | 20/8mA | System Reset. This is the system reset signal for ISA devices. This signal pin is also available for P24T/M6/M7 CPU reset. |
| XSA0 | 179 | I/O-TTL-CMOS | 20/8mA | Slot Address 0. This is the system address line 0 signal. In CPU cycles, this signal is generated by BE3J-BE0J and the chip internal control circuit. In DMA cycle, it is generated by the internal 8237. In refresh cycle, it is generated by the internal refresh circuits. In master cycle, it is an input polarity signal and is driven by the master device. |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name | Number | Type | Driver (IOL/IOH) | Description |
| --- | --- | --- | --- | --- |
| ISA SYSTEM interface |  |  |  |  |
| XSA1 | 180 | I/O-TTL-CMOS | 20/8mA | Slot Address 1. This is the system address line 1 signal. In CPU cycle, this signal is generated by BE3J-BE0J and the chip internal control circuit. In DMA cycle, it is generated by the internal 8237. In refresh cycle, it is generated by the internal refresh circuits. In master cycle, it is an input polarity signal and is driven by the master device. |
| XSBHEJ | 181 | I/O-TTL-CMOS | 20/8mA | Slot Byte High Enable. This is the system byte high enable signal. In CPU cycle, this signal is generated by BE3J-BE0J and the chip internal control circuit. In DMA cycle, it is generated by the internal 8237. In refresh cycle, it is generated by the internal refresh circuits. In master cycle, it is an input polarity signal and is driven by the master device. |
| XSD[15:0] | 162-169,171-178 | I/O-TTL-CMOS | 24/8mA | Slot Data Bus. These are the system data lines. |
| XBALE | 182 | O-CMOS | 20/8mA | Buffer Address Latch Enable. This is the buffered address latch enable signal. A high pulse with half SYSCLK is generated at the beginning of CPU ISA cycle for the external devices to latch the system address. During the CPU bus releasing (HLDA) cycle, this signal is kept at a high level status. |
| XAEN | 183 | O-CMOS | 20/8mA | Address Enable. This is the address enable signal. In CPU or master cycle, this signal is driven low to enable the external devices to decode the I/O address. In other cycles, this signal is driven high to inhibit I/O address decoding. |
| XSMEMRJ | 188 | O-CMOS | 20/8mA | System Memory Read. This tri-state output signal is the system memory read signal. It is driven low only when the system executes a below 1 Mbyte ISA memory read cycle. Otherwise, it is a tri-state status. |
| XSMEMWJ | 189 | O-CMOS | 20/8mA | System Memory Write. This tri-state output signal is the system memory write signal. It is driven low only when the system executes a below 1 Mbyte ISA memory write cycle. Otherwise, it is a tri-state status. |
| XMRJ | 190 | I/O-TTL-CMOS | 20/8mA | Memory Read. This is the memory read signal. This signal is an input in master cycle but an output in other cycles. In CPU ISA cycles, this signal is driven by chip internal control circuits. In DMA cycles, this signal is driven by the internal DMA controllers. In refresh cycles, this signal is driven by the internal refresh circuits. In master cycles, this signal is driven by external master devices. |
| XMWJ | 191 | I/O-TTL-CMOS | 20/8mA | Memory Write. This is the memory write signal. This signal is an input in master cycles but is an output in other cycles. In CPU ISA cycles, this signal is driven by chip internal control circuits. In DMA cycles, this signal is driven by the internal DMA controllers. In master cycles, this signal is driven by the external master devices. |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name | Number | Type | Driver (IOL/IOH) | Description |
| --- | --- | --- | --- | --- |
| ISA System Interface |  |  |  |  |
| XIORJ | 187 | I/O-TTL-CMOS | 20/8mA | I/O Read. This is the I/O read signal. This signal is an input in master cycle but is output in other cycles. In CPU ISA cycles, this signal is driven by chip internal control circuits. In DMA cycles, this signal is driven by the internal DMA controllers. In master cycles, this signal is driven by the external master devices. |
| XIOWJ | 186 | I/O-TTL-CMOS | 20/8mA | I/O Write. This is the I/O write signal. This signal is an input in master cycle but is output in other cycles. In CPU ISA cycles, this signal is driven by chip internal control circuits. In DMA cycles, this signal is driven by the internal DMA controllers. In master cycles, this signal is driven by the external master devices. |
| XMEM16J | 160 | I-TTL |  | External 16-Bit Memory Chip Select. This is the 16 bit memory devices select indicating singal. |
| XIO16J | 159 | I-TTL |  | External 16-Bit I/O Chip Select. This is the 16 bit I/O devices select indicating signal. |
| XMASTERJ | 158 | I-TTL |  | Master. This is the ISA master access indicating signal. |
| XIOCHRDYJ | 157 | I/O-TTL-CMOS | 20/8mA | I/O Channel Ready. This is the I/O channel ready signal. This input is used to extend the ISA command width for the CPU and DMA cycles. In a write-back cache system, this signal is output polarity to pend the ISA MST/DMA cycle for a cache hit access. In the ISA MST/DMA accesses the VESA local slave cycle, this signal is also output polarity to pend the ISA MST/DMA cycle until the VESA slave finishes the accesed cycle. |
| XREFRESHJ | 184 | I/O-TTL-CMOS | 12/8mA | Refresh. This is the refresh signal. The signal is input polarity at master cycle, but it is output pin at other cycles. |
| XTC | 203 | I/O-TTL-CMOS | 20/8mA | Terminal Count. This is the DMA channel terminal count indicating signal. |
| XDIR | 156 | I/O-TTL-CMOS | 4/4mA | X-Data Direction. This is external data buffer direction control signal. |
| XBUSCSJ | 152 | I/O-TTL-CMOS | 8/6mA | X-Bus Chip Select. This is the X-BUS chip select indicating signal that includes ROM ,RTC, and keyboard. |
| XSPEAKER | 153 | I/O-TTL-CMOS | 8/6mA | Speaker Data. This is the speaker data. |
| XSPLED | 151 | I/O-TTL-CMOS | 4/4mA | CPU Speed. This is the speed LED indicating signal. |
| XSEL[2:0] | 193-195 | I/O-TTL-CMOS | 12/8mA | Multiplex Select. This are DRQ, IRQ multiplex select counter signals. |
| XDDRQ | 198 | I-TTL |  | DMA Request Input. This is the DRQ multiplex input signal. The multiplexed signals are DRQ0, DRQ1, DRQ2, DRQ3, DRQ5, DRQ6, and DRQ7 (and DRQ2). |
| XDK[2:0] | 200-202 | I/O-TTL-CMOS | 8/6mA | DMA Acknowledge Output. This are DACK de-multiplex select signals. |
| XENACKJ | 199 | O-CMOS | 8/6mA | Enable DMA Acknowledge. This is the DMA acknowledge enable indicating signal. |
| XMPXI | 196 | I-TTL |  | Multiplex One. This is the IRQ/TURBO/IOCHKJ multiplex input signal. The multiplexed signals are IOCHKJ, TURBO, IRQ1, IRQ4, IRQ5, IRQ6, IRQ7 (and IRQ3 ). |

Table 2-1 	M1429L Pin Description Table   (continued)

| Name |  | Number | Type |  | Driver (IOL/IOH) |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XMPXII |  | 197 | I-TTL |  |  |  | Multiplex Two. This is the IRQ multiplex input signal. The multiplexed signals are IRQ8J, IRQ9, IRQ10, IRQ11, IRQ12, IRQ14, and IRQ15 (and IRQ14). |  |
| XLOMEGJ |  | 188 | O-CMOS |  | 20/8mA |  | Below 1 Mega Cycle. This signal indicates the current cycle is operating a below 1 mega instruction. This signal generates the external SMEMRJ & SMEMWJ by combining with the MRJ & MWJ. |  |
| X32K |  | 205 | I-TTL |  |  |  | 32.768-KHz Input. This frequency is the time base of the RTC. |  |
| VBAT |  | 206 |  |  |  |  | V-Battery. This signals indicates supply battery power to internal RTC. |  |
| XPS | 204 |  |  |  |  |  |  | Power Sense. This signal is the status of V-Battery. |
| SMM & PMU signal |  |  |  |  |  |  |  |  |
| XSMIJ |  | 155 | I/O-TTL-CMOS |  | 8/6mA |  | System Management Interrupt This signal invokes the System Management Mode(SMM). |  |
| XSMIACTJ |  | 154 | I-TTL |  |  |  | System Management Interrupt Active This signal is an active low input indicating that the processor is operating in SMM. |  |
| XSTPCLKJ |  | 189 | O-CMOS |  | 20/8mA |  | Stop Clock Request This signal indicates that a request to Intel processor has been made to turn off its CLK input. |  |
| XSUSPJ |  | 189 | O-CMOS |  | 20/8mA |  | Suspend Request This signal indicates that a request to Cyrix processor has been made to enter suspend mode. |  |
| XSMIADSJ |  | 154 | I-TTL |  |  |  | SMM Address Strobe This signal from AMD/Cyrix/IBM processors is asserted instead of ADSJ during SMM bus cycles and indicates that SMM memory is being accessed. |  |
| XSMIRDYJ | 189 |  |  | O-CMOS |  | 20/8mA |  | SMM Ready This signal to AMD/IBM processors indicates that the current SMM bus cycle can be terminated. |
| Power |  |  |  |  |  |  |  |  |
| GND[9:0] |  | 14,40,59,90,108,129,150,161,185 |  |  |  |  | Ground. |  |
| VDD[0:6] |  | 31,75,113,137,170,192 |  |  |  |  | Power. +5V volt supply. |  |

2.3	Numerical Pin ListTable 2-2	Numerical Pin List for Different Systems

| Number | Type | 386/IBMsystem | 486/M7system | M6system | P24Tsystem |
| --- | --- | --- | --- | --- | --- |
| 1 | I | XCLKI | XCLKI | XCLKI | XCLKI |
| 2 | O | XSYSRST | XSYSRST | XRSTCPU | XRSTCPU |
| 3 | O | XATCLK | XATCLK | XATCLK | XATCLK |
| 4 | B | XSLA20 | XA20MJ | XA20MJ | XA20MJ |
| 5 | O | XRSTCPU | XRSTCPU | XWARN_RST | XINIT |
| 6 | O | XBUSY6J | XIGNNEJ | XBUSY6J | XIGNNEJ |
| 7 | O | XCLK2XO | XCLK2XO | XCLK2XO | XCLK2XO |
| 8 | I | XERR7J | XFERRJ | XERR7J | XFERRJ |
| 9 | I | XFBCLK2 | XFBCLK2 | XFBCLK2 | XFBCLK2 |
| 10 | O | XINTR | XINTR | XINTR | XINTR |
| 11 | O | XCLK1XO | XCLK1XO | XCLK1XO | XCLK1XO |
| 12 | O | XNMI | XNMI | XNMI | XNMI |
| 13 | I | XFBCLK1 | XFBCLK1 | XFBCLK1 | XFBCLK1 |
| 14 |  | GND1 | GND1 | GND1 | GND1 |
| 15 | B | XGD0 | XGD0 | XGD0 | XGD0 |
| 16 | B | XGD1 | XGD1 | XGD1 | XGD1 |
| 17 | B | XGD2 | XGD2 | XGD2 | XGD2 |
| 18 | B | XGD3 | XGD3 | XGD3 | XGD3 |
| 19 | B | XGD4 | XGD4 | XGD4 | XGD4 |
| 20 | B | XGD5 | XGD5 | XGD5 | XGD5 |
| 21 | B | XGD6 | XGD6 | XGD6 | XGD6 |
| 22 | B | XGD7 | XGD7 | XGD7 | XGD7 |
| 23 | B | XGD8 | XGD8 | XGD8 | XGD8 |
| 24 | B | XGD9 | XGD9 | XGD9 | XGD9 |
| 25 | B | XGD10 | XGD10 | XGD10 | XGD10 |
| 26 | B | XGD11 | XGD11 | XGD11 | XGD11 |
| 27 | B | XGD12 | XGD12 | XGD12 | XGD12 |
| 28 | B | XGD13 | XGD13 | XGD13 | XGD13 |
| 29 | B | XGD14 | XGD14 | XGD14 | XGD14 |
| 30 | B | XGD15 | XGD15 | XGD15 | XGD15 |
| 31 |  | VCC1 | VCC1 | VCC1 | VCC1 |
| 32 | B | XGD16 | XGD16 | XGD16 | XGD16 |
| 33 | B | XGD17 | XGD17 | XGD17 | XGD17 |
| 34 | B | XGD18 | XGD18 | XGD18 | XGD18 |
| 35 | B | XGD19 | XGD19 | XGD19 | XGD19 |
| 36 | B | XGD20 | XGD20 | XGD20 | XGD20 |
| 37 | B | XGD21 | XGD21 | XGD21 | XGD21 |
| 38 | B | XGD22 | XGD22 | XGD22 | XGD22 |
| 39 | B | XGD23 | XGD23 | XGD23 | XGD23 |
| 40 |  | GND2 | GND2 | GND2 | GND2 |
| 41 | B | XGD24 | XGD24 | XGD24 | XGD24 |
| 42 | B | XGD25 | XGD25 | XGD25 | XGD25 |
| 43 | B | XGD26 | XGD26 | XGD26 | XGD26 |
| 44 | B | XGD27 | XGD27 | XGD27 | XGD27 |
| 45 | B | XGD28 | XGD28 | XGD28 | XGD28 |
| 46 | B | XGD29 | XGD29 | XGD29 | XGD29 |
| 47 | B | XGD30 | XGD30 | XGD30 | XGD30 |
| 48 | B | XGD31 | XGD31 | XGD31 | XGD31 |
| 49 | B | XPD0 | XPD0 | XPD0 | XPD0 |
| 50 | B | XPD1 | XPD1 | XPD1 | XPD1 |
| 51 | B | XPD2 | XPD2 | XPD2 | XPD2 |
| 52 | B | XPD3 | XPD3 | XPD3 | XPD3 |

Table 2-2	Numerical Pin List  for Different Systems (continued)

| Number | Type | 386/IBMsystem | 486/M7system | M6system | P24Tsystem |
| --- | --- | --- | --- | --- | --- |
| 53 | B | XADSJ | XADSJ | XADSJ | XADSJ |
| 54 | B | XMIOJ | XMIOJ | XMIOJ | XMIOJ |
| 55 | B | XDCJ | XDCJ | XDCJ | XDCJ |
| 56 | B | XWRJ | XWRJ | XWRJ | XWRJ |
| 57 | O | XRDYOJ | XRDYOJ | XRDYOJ | XRDYOJ |
| 58 | O | XBRDYJ | XBRDYJ | XBRDYJ | XBRDYJ |
| 59 |  | GND3 | GND3 | GND3 | GND3 |
| 60 | I | XBUSY7J | XPCD | XPCD | XCACHEJ |
| 61 | I | XPEREQ7 | XLOCKJ | XLOCKJ | XLOCKJ |
| 62 | O | #nc/XDFSREQJ | XKENOJ | XKENOJ | XKENOJ |
| 63 | O | nc | XEADSJ | XEADSJ | XEADSJ |
| 64 | O | XPEREQ6 | XFLUSHJ | XFLUSHJ | XFLUSHJ |
| 65 | O | XHOLD | XHOLD | XHOLD | XHOLD |
| 66 | I | XHLDA | XHLDA | XHLDA | XHLDA |
| 67 | I | XBS16J | *@XPCHKJ | XPCHKJ | XPCHKJ |
| 68 | B | XCALATCHJ | XBOFFJ | XBUSY7J | XBOFFJ |
| 69 | I | XWINTR | XWINTR/XHITMJ | XHITMJ | XHITMJ |
| 70 | B | XIRQ3 | XIRQ3 | XPEREQ6 | XWB/XWTJ |
| 71 | B | XLDEVJ | XLDEVJ | XLDEVJ | XLDEVJ |
| 72 | B | XLNPRDYJ | XLNPRDYJ | XLNPRDYJ | XLNPRDYJ |
| 73 | O | *XLGNTJ1 | XLGNTJ1 | XLGNTJ1 | XLGNTJ1 |
| 74 | B | *XLREQJ1 | XLREQJ1 | XLREQJ1 | XLREQJ1 |
| 75 |  | VCC2 | VCC2 | VCC2 | VCC2 |
| 76 | B | XBE0J | XBE0J | XBE0J | XBE0J |
| 77 | B | XBE1J | XBE1J | XBE1J | XBE1J |
| 78 | B | XBE2J | XBE2J | XBE2J | XBE2J |
| 79 | B | XBE3J | XBE3J | XBE3J | XBE3J |
| 80 | B | XGA2 | XGA2 | XGA2 | XGA2 |
| 81 | B | XGA3 | XGA3 | XGA3 | XGA3 |
| 82 | B | XGA4 | XGA4 | XGA4 | XGA4 |
| 83 | B | XGA5 | XGA5 | XGA5 | XGA5 |
| 84 | B | XGA6 | XGA6 | XGA6 | XGA6 |
| 85 | B | XGA7 | XGA7 | XGA7 | XGA7 |
| 86 | B | XGA8 | XGA8 | XGA8 | XGA8 |
| 87 | B | XGA9 | XGA9 | XGA9 | XGA9 |
| 88 | B | XGA10 | XGA10 | XGA10 | XGA10 |
| 89 | B | XGA11 | XGA11 | XGA11 | XGA11 |
| 90 |  | GND4 | GND4 | GND4 | GND4 |
| 91 | B | XGA12 | XGA12 | XGA12 | XGA12 |
| 92 | B | XGA13 | XGA13 | XGA13 | XGA13 |
| 93 | B | XGA14 | XGA14 | XGA14 | XGA14 |
| 94 | B | XGA15 | XGA15 | XGA15 | XGA15 |
| 95 | B | XGA16 | XGA16 | XGA16 | XGA16 |
| 96 | B | XGA17 | XGA17 | XGA17 | XGA17 |
| 97 | B | XGA18 | XGA18 | XGA18 | XGA18 |
| 98 | B | XGA19 | XGA19 | XGA19 | XGA19 |
| 99 | B | XGA20 | XGA20 | XGA20 | XGA20 |
| 100 | B | XGA21 | XGA21 | XGA21 | XGA21 |
| 101 | B | XGA22 | XGA22 | XGA22 | XGA22 |
| 102 | B | XGA23 | XGA23 | XGA23 | XGA23 |
| 103 | B | XGA24 | XGA24 | XGA24 | XGA24 |
| 104 | B | XGA25 | XGA25 | XGA25 | XGA25 |
| 105 | B | XGA26 | XGA26 | XGA26 | XGA26 |
| 106 | B | XGA27 | XGA27 | XGA27 | XGA27 |

Table 2-2	Numerical Pin List  for Different Systems (continued)

| Number | Type | 386/IBMsystem | 486/M7system | M6system | P24Tsystem |
| --- | --- | --- | --- | --- | --- |
| 107 | B | XGA31 | XGA31 | XGA31 | XGA31 |
| 108 |  | GND5 | GND5 | GND5 | GND5 |
| 109 | O | XCOE0J | XCOE0J | XCOE0J | XCOE0J |
| 110 | O | XCOE1J | XCOE1J | XCOE1J | XCOE1J |
| 111 | O | XCWE0J | XCWE0J | XCWE0J | XCWE0J |
| 112 | O | XCWE1J | XCWE1J | XCWE1J | XCWE1J |
| 113 |  | VCC3 | VCC3 | VCC3 | VCC3 |
| 114 | O | XCS0J | XCS0J | XCS0J | XCS0J |
| 115 | O | XCS1J | XCS1J | XCS1J | XCS1J |
| 116 | O | XCS2J | XCS2J | XCS2J | XCS2J |
| 117 | O | XCS3J | XCS3J | XCS3J | XCS3J |
| 118 | O | XCA3IO | XCA3IO | XCA3IO | XCA3IO |
| 119 | O | XCA3IIO | XCA3IIO | XCA3IIO | XCA3IIO |
| 120 | O | XTAGWEJ | XTAGWEJ | XTAGWEJ | XTAGWEJ |
| 121 | B | XTAG0 | XTAG0 | XTAG0 | XTAG0 |
| 122 | B | XTAG1 | XTAG1 | XTAG1 | XTAG1 |
| 123 | B | XTAG2 | XTAG2 | XTAG2 | XTAG2 |
| 124 | B | XTAG3 | XTAG3 | XTAG3 | XTAG3 |
| 125 | B | XTAG4 | XTAG4 | XTAG4 | XTAG4 |
| 126 | B | XTAG5 | XTAG5 | XTAG5 | XTAG5 |
| 127 | B | XTAG6 | XTAG6 | XTAG6 | XTAG6 |
| 128 | B | XTAG7 | XTAG7 | XTAG7 | XTAG7 |
| 129 |  | GND6 | GND6 | GND6 | GND6 |
| 130 | O | XRAS0J | XRAS0J | XRAS0J | XRAS0J |
| 131 | O | XRAS1J | XRAS1J | XRAS1J | XRAS1J |
| 132 | O | XCAS0J | XCAS0J | XCAS0J | XCAS0J |
| 133 | O | XCAS1J | XCAS1J | XCAS1J | XCAS1J |
| 134 | O | XCAS2J | XCAS2J | XCAS2J | XCAS2J |
| 135 | O | XCAS3J | XCAS3J | XCAS3J | XCAS3J |
| 136 | O | XMWEJ | XMWEJ | XMWEJ | XMWEJ |
| 137 |  | VCC4 | VCC4 | VCC4 | VCC4 |
| 138 | O | XMA0 | XMA0 | XMA0 | XMA0 |
| 139 | O | XMA1 | XMA1 | XMA1 | XMA1 |
| 140 | O | XMA2 | XMA2 | XMA2 | XMA2 |
| 141 | O | XMA3 | XMA3 | XMA3 | XMA3 |
| 142 | O | XMA4 | XMA4 | XMA4 | XMA4 |
| 143 | O | XMA5 | XMA5 | XMA5 | XMA5 |
| 144 | O | XMA6 | XMA6 | XMA6 | XMA6 |
| 145 | O | XMA7 | XMA7 | XMA7 | XMA7 |
| 146 | O | XMA8 | XMA8 | XMA8 | XMA8 |
| 147 | O | XMA9 | XMA9 | XMA9 | XMA9 |
| 148 | O | XMA10 | XMA10 | XMA10 | XMA10 |
| 149 | O | XMA11 | XMA11 | XMA11 | XMA11 |
| 150 |  | GND7 | GND7 | GND7 | GND7 |
| 151 | B | XSPLED | XSPLED | XSPLED | XSPLED |
| 152 | B | XBUSCSJ | XBUSCSJ | XBUSCSJ | XBUSCSJ |
| 153 | B | XSPEAKER | XSPEAKER | XSPEAKER | XSPEAKER |
| 154 | B | @XLREQJ2 | XLREQJ2 | XLREQJ2 | XLREQJ2 |
| 155 | O | @XLGNTJ2 | XLGNTJ2 | XLGNTJ2 | XLGNTJ2 |
| 156 | B | XDIR | XDIR | XDIR | XDIR |
| 157 | B | XIOCHRDYJ | XIOCHRDYJ | XIOCHRDYJ | XIOCHRDYJ |
| 158 | I | XMASTERJ | XMASTERJ | XMASTERJ | XMASTERJ |
| 159 | I | XIO16J | XIO16J | XIO16J | XIO16J |
| 160 | I | XMEM16J | XMEM16J | XMEM16J | XMEM16J |

Table 2-2	Numerilcal Pin List for Different Systems (continued)

| Number | Type | 386/IBMsystem | 486/M7system | M6system | P24Tsystem |
| --- | --- | --- | --- | --- | --- |
| 161 |  | GND8 | GND8 | GND8 | GND8 |
| 162 | B | XSD0 | XSD0 | XSD0 | XSD0 |
| 163 | B | XSD1 | XSD1 | XSD1 | XSD1 |
| 164 | B | XSD2 | XSD2 | XSD2 | XSD2 |
| 165 | B | XSD3 | XSD3 | XSD3 | XSD3 |
| 166 | B | XSD4 | XSD4 | XSD4 | XSD4 |
| 167 | B | XSD5 | XSD5 | XSD5 | XSD5 |
| 168 | B | XSD6 | XSD6 | XSD6 | XSD6 |
| 169 | B | XSD7 | XSD7 | XSD7 | XSD7 |
| 170 |  | VCC5 | VCC5 | VCC5 | VCC5 |
| 171 | B | XSD8 | XSD8 | XSD8 | XSD8 |
| 172 | B | XSD9 | XSD9 | XSD9 | XSD9 |
| 173 | B | XSD10 | XSD10 | XSD10 | XSD10 |
| 174 | B | XSD11 | XSD11 | XSD11 | XSD11 |
| 175 | B | XSD12 | XSD12 | XSD12 | XSD12 |
| 176 | B | XSD13 | XSD13 | XSD13 | XSD13 |
| 177 | B | XSD14 | XSD14 | XSD14 | XSD14 |
| 178 | B | XSD15 | XSD15 | XSD15 | XSD15 |
| 179 | B | XSA0 | XSA0 | XSA0 | XSA0 |
| 180 | B | XSA1 | XSA1 | XSA1 | XSA1 |
| 181 | B | XSBHEJ | XSBHEJ | XSBHEJ | XSBHEJ |
| 182 | O | XBALE | XBALE | XBALE | XBALE |
| 183 | O | XAEN | XAEN | XAEN | XAEN |
| 184 | B | XREFRESHJ | XREFRESHJ | XREFRESHJ | XREFRESHJ |
| 185 |  | GND9 | GND9 | GND9 | GND9 |
| 186 | B | XIOWJ | XIOWJ | XIOWJ | XIOWJ |
| 187 | B | XIORJ | XIORJ | XIORJ | XIORJ |
| 188 | O | @XSMEMRJ | XSMEMRJ | XSMEMRJ | XSMEMRJ |
| 189 | O | @XSMEMWJ | XSMEMWJ | XSMEMWJ | XSMEMWJ |
| 190 | B | XMRJ | XMRJ | XMRJ | XMRJ |
| 191 | B | XMWJ | XMWJ | XMWJ | XMWJ |
| 192 |  | VCC6 | VCC6 | VCC6 | VCC6 |
| 193 | B | XSEL0 | XSEL0 | XSEL0 | XSEL0 |
| 194 | B | XSEL1 | XSEL1 | XSEL1 | XSEL1 |
| 195 | B | XSEL2 | XSEL2 | XSEL2 | XSEL2 |
| 196 | I | XMPXI | XMPXI | XMPXI | XMPXI |
| 197 | I | XMPXII | XMPXII | XMPXII | XMPXII |
| 198 | I | XDDRQ | XDDRQ | XDDRQ | XDDRQ |
| 199 | O | XENACKJ | XENACKJ | XENACKJ | XENACKJ |
| 200 | B | XDK0 | XDK0 | XDK0 | XDK0 |
| 201 | B | XDK1 | XDK1 | XDK1 | XDK1 |
| 202 | B | XDK2 | XDK2 | XDK2 | XDK2 |
| 203 | B | XTC | XTC | XTC | XTC |
| 204 | I | *@XPS | XPS | XPS | XPS |
| 205 | I | X32K | X32K | X32K | X32K |
| 206 | I | VBAT | VBAT | VBAT | VBAT |
| 207 | I | XPWGD | XPWGD | XPWGD | XPWGD |
| 208 | I | XOSC | XOSC | XOSC | XOSC |

*  Note:	If the system supports 4-bank DRAM, the XLGNTJ1 and XLREQJ1 pin map to XRASJ2 and XRASJ3, and XPS, XPCHKJ map to XLGNTJ1, XLREQJ1.  Thus XLGNTJ1---->XRASJ2.  XLREQJ1---->XRASJ3.  XPS---->XLREQJ1. XPCHKJ---->XLGNTJ1.  (*** Note : The index 45_D7 must be set high when XPCHKJ maps to XLGNTJ)@  Note:	This shows the PINOUT difference between systems using green function and those that do not.	If M1429L is not active, the green function is the same as M1429.

Table 2-3 lists the pin differences for systems using the green function.

Table 2-3	Pin Differences for Systems Using Green Function

|  |  | M1429 | M1429L |  |
| --- | --- | --- | --- | --- |
|  | CPU Type |  | Intel/AMD/Cyrix | IBM B.L. |
|  | Pin no. |  |  |  |
| *** | 67 | PCHKJ | LGNTJ | LGNTJ |
|  | 154 | RTCAS/LREQ2J | SMIACTJ/SMIADSJ/SMIADSJ | PWIADSJ |
|  | 155 | ADIR/LGNT2J | SMIJ | PWIJ |
|  | 188 | SMEMRJ | LOMEGJ | LOMEGJ |
|  | 189 | SMEMWJ | STPCLKJ/SMIRDYJ/SUSPJ | PWIRDYJ |
|  | 204 | PS | LREQJ | LREQJ |
|  | #62 | KENOJ | KENOJ | DFSREQJ |

2.4	M1429L Hardware Setting Control :	Original M1429 pin-out :	XTC pull-low, XSPEAK pull-high	pin 67 becomes a NC pin	PCI M1429 pin-out : ( interface to M1435 ) :	XTC pull-high   ( M1429L_LDEVJ = M1435_ISARQJ )	M1429L pin-out : ( M1429 pin -> M1429L pin )	XSPEAK pull-low	2 VM, 2BM  or	1 VM, 4BM NOTES:    	1.	If the SMRAM address needs to move to the split region of M1429L (i.e. the SMBASE descriptor relocates to differ with the CPU default address), the CPU SRESET pin connects to the M1429L_RSTCPU pin, and the CPU RESET pin connects to the M1429L_SYSRST pin shared with the ISA slot RSTDRV signal.	2.	To support the 0-Hz XSTPCLKJ, the system board needs extra logic to control the CPU clock.  And the M1429L uses Pin 6 (IGNNEJ ) to issue a 0-Hz starting &  ending event during the XSTPCLKJ active period.	3.	Under Green function, the XPS and XPCHKJ pins map to XLREQJ2 and XLGNTJ2.Table 2-4	PINOUT difference between M1431 Green Function and Normal Function

| Pin No. | M1431 (Normal) | M1431 (Green) |
| --- | --- | --- |
| 50 | GND | GREEN (pull-up) |
| 55 | RTCDS | EXTSW |
| 56 | RTCWR | RING-in |

## 2.5	 Hardware Setting Configuration

Table2-5		Hardware Setting Configuration Table

Function	Configuration

Pins	XSEL[0]	Pull-High/Pull-Low	8/16 Bit  ROM Select

XBUSCSJ	Pull-High/Pull-Low	Normal/Test Mode Operation

XCPUSPD	Pull-High/Pull-Low	Enable/Disable Internal RTC

XDIR	Pull-High/Pull-Low	2/4 Bank DRAM Architecture

XSEL[2]	Pull-High/Pull -Low	2X/1X Clock System

XTC	Pull-High/Pull-Low	Enable/Disable PCI Feature

XSPEAK	Pull-High/Pull-Low	Disable/Enable Green Function

Pins	XDK[2]	XDK[1]	XDK[0]	CPU Type Definition

Pull-Low	Pull-Low	Pull-Low	486DLC System

Pull-Low	Pull-Low	Pull-High	386DX System

Pull-Low	Pull-High	Pull-Low	Cyrix M6 System

Pull-Low	Pull-High	Pull-High	486 System

Pull-High	Pull-High	Pull-High	Upgradable 486 System

Pull-High	Pull-Low	Pull-High	Intel P24T System

Pull-High	Pull-High	Pull-Low	Cyrix M7 System

NOTE:	M1429L supports two VESA Master.  When the green and 4-bank DRAM features are enabled, it supports only one VESA master.  See Table 2-6.

Table 2-6	Supported VESA Masters

| Pin/Condition | 2-Bank and Non-Green | 4-Bank and Non-Green | 2-Bank and Green | 4-Bank and Green |
| --- | --- | --- | --- | --- |
| XLGNT1J | Pin73 | Pin67 | Pin73 | Pin67 |
| XLREQ1J | Pin74 | Pin204 | Pin74 | Pin204 |
| XLGNT2J | Pin155 | Pin155 | Pin67 |  |
| XLREQ2J | Pin154 | Pin154 | Pin204 |  |

NOTE:	The logical value of index 45_D7 must be one when Pin67 is mapped to LGNTJ.

|  |  | System Configuration |  |  |
| --- | --- | --- | --- | --- |
| Pin No. | 2-Bank and Non-Green | 4-Bank and Non-Green | 2-Bank and Green | 4-Bank and Green |
| Pin 73 | XLGNT1J | XRAS2J | XLGNT1J | XRAS2J |
| Pin 74 | XLREQ1J | XRAS3J | XLREQ1J | XRAS3J |
| Pin 67 | NC | XLGNT1J | XLGNT2J | XLGNT1J |
| Pin 204 | NC | XLREQ1J | XLREQ2J | XLREQ1J |
| Pin 155 | XLGNT2J | XLGNT2J | XSMIJ | XSMIJ |
| Pin 154 | XLREQ2J | XLREQ2J | XSMIACTJ | XSMIACTJ |

Section 3: Function Description

The M1429L & M1431 chipset are designed to perform in 386DX/387DX, TI486SXL/DLC systems with speeds of up to 40-MHz or in 486SX/487SX- or 486DX- or 486DX2- or Cyrix CX486S- (M6/M7) or Intel P24T/D-based systems with speeds of up to 66-MHz.  Using the green feature, the pair performs Intel S-series, Cyrix, AMD, IBM or TI CPU which support SMM feature.  The chipset offers the following blocks.For M1429L:

·	Bus control logic·	Power down mode logic & clock generator·	CPU interface logic·	Numeric coprocessor interface logic·	VESA VL-bus interface logic·	Control logic·	Memory logic·	Cache logic·	Address decode & memory mapping logic·	Configure register·	Address buffer·	Data buffer·	Keyboard & speaker logic·	ISP (two 82C37, two 82C59, one 82C54)·	Polling control logic·	Internal RTC & logic·	PCI interface logic (with M1435 pair, performs PCI system)·	SMM logic control·	STPCLK# control and clock throttling logic·	SMI control logic·	APM control logic·	PMU control logicFor M1431:

### ·	Includes three 74151 & four 74245·	RTC & buffer control logicThe following section describe the blocks.Bus Control LogicThe block includeds the ISA bus state machine, 16-bit or 8-bit commands justified wait-state and control logic.  These signals are compatible with PC/AT standards.

Power Down Mode Logic & Clock GeneratorThere are two clock inputs and two clock outputs in M1429L.  These are: XFBCLK1/XFBCLK2 and XCLK1O/XCLK2O input/output clocks which provide frequency operation for the system board and CPU depending on 1X or 2X system used.  The four CLKs (two inputs and two outputs) make it easy to tune the timing or clock skew under different timing delays for different system board layouts.  The other is XOSC crystal oscillator which is used as a clock for reset and time-out counters.  The ATCLK clock (XATCLK) output is generated from XCLK2 (divided by 4,5,6,8,10,12) or from XOSC (divided by 2).  It can be programmed at index 20.  The keyboard controller clock is the same as XATCLK.

Under power down mode, we can switch CPU clock to OSC clock (divided by 2, 4, 8) or AT clock (divided  by 1 and 2). For different CPU systems (386, 486 or others) we can choose to reset CPU or not to change CPU clock.  Index 21 is programmed for power down mode.

The switching power supply sends a POWERGOOD signal to M1429L to generate system-reset, and resets the chipsets to initial state.CPU Interface LogicThe CPU interface is different for 386-, TI486SXL, 486-, Cyrix CX486s and Intel P24T-system.

The chipset pinout definition is also different.  Having different CPUs, the interface logic decodes the status MIO, DC, WR with different equivalents.  The READY signal is also different because of the burst and write-burst cycle for different types of CPUs.Numerical Coprocessor Interface For 386 systems, M1429L monitors ERROR#, BUSY#, PEREQ and WINTR to support the 80387 or Weitek coprocessor.  When ERROR# is active, this indicates that an unmaskable coprocessor exception has occurred and M1429L generates an IRQ13 to the CPU.  (If it immediately follows a RESET signal, this indicates that a coprocessor is present in the system.)  The coprocessor asserts a BUSY# signal while executing, and asserts LNPRDY# to M1429L when it is finished.  The M1429L then bypasses or synchronizes (selected in index 27) the LNPRDY# signal to RDYO#.  The ready signal must meet the CPU requirement.  For 486 systems, M1429L receives FERR# from the CPU, if active, it internally generates IRQ13 to CPU and drives IGNNE# low to CPU.

### VESA VL-Bus Interface LogicM1429L supports two VESA MASTER interface logic and bus arbitration.  It supports VESA pre-empt feature when any VESA MASTER card asserts LREQ# too long to timeout other system requests (DMA, REFRESH, or other card accesses).  The optional pre-empt feature is for meeting the specification of  VL-bus.Control LogicThe control logic controls the internal data bus and address bus flow.  It also generates proper read-select to internal device and chooses the correct data output.  It selects the correct address bus for DMA and REFRESH cycles to send to system.Memory LogicThe memory logic is capable of accessing up to 128-MB of local memory, and supporting 4-banks of DRAM using 256K, 512K, 1M, 4M, and 16M single-side SIMM.  Each bank can be disabled through software.  Programmable DRAM timing is provided for RAS precharge time and RAS-to-CAS delay to achieve highest performance and reliability.  Programmable shadowing regions are divided into non-cacheable and optional non-cacheable groups.  If the optional non-cacheable regions are programmed as cacheable.  M1429L write-protects the second-level cache and uses FLUSH to write-protect for 486 systems internal cache.  The CPUs which support write-back forces the cacheable region to write-through mode and uses FLUSH to write-protect the region.Cache LogicThe cache control uses direct-map, write-through or write-back-update methods for maximum performance. Designers can use 8Kx8, 32kx8, 64Kx8 and 128Kx8 SRAMs to form 32K to 1M cache memory size with one-bank or two-bank types. When two-bank type is selected, bank interleave is used to increase system performance. 7-bit or 8-bit TAG can also be selected.  When write-back is selected, dirty bit SRAM is not needed.Address Decode and Memory Mapping LogicThe decode-circuit decodes the BIOS ROM, keyboard controller, internal ISP device, real-time clock, port61H, and configure register.  When remap is enabled, it decodes the remap memory to the end of DRAM.Configure RegisterThe configure register controls the whole system at different CPUs under different frequencies. It enables the system to set these configuration registers to meet the compatible, reliable performance and functional requirements.

Address BufferThe address buffer generated at address SA1, SA0, and BHE# for ISA bus, and A20M# for 486 CPUs initiates the byte-enable signal at DMA and MASTER cycles.Data BufferThe data control signal controls data-transfer between the CPU data bus, memory data bus and ISA data bus during CPU cycles, VESA bus cycles, DMA cycles and MASTER cycles.Keyboard and Speaker LogicThis block emulates the keyboard controller fast-RC and fast gate-A20 functions for maximum performance. It combines with port 61H at this block to generate speaker signal.ISP Devices (82C37x2, 82C59x2, 82C54, 74LS612)The ISP device is built-in, thus no 82C206 is required. There are two 82C37s, two 82C59s, one 82C54 and one 74LS612 built-in devices.Polling Control LogicThe IRQ, DRQ, and DACK polling logic minimize pin-count for the M1429L. A gray code polling counter is built-in to avoid polling glitch. The polling speed can be programmed by software. The M1431 and one 74LS138 (or three 74F151 and one 74LS138) is required to implement the polling logic as seen in the following table.

Table 2-5	Polling Logic

| XSEL[2] | XSEL[1] | XSEL[0] | XDDRQ | XMPXI | XMPXII |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | DRQ[0] | IOCHK# | IRQ[8]# |
| 0 | 0 | 1 | DRQ[1] | TURBO | IRQ[9] |
| 0 | 1 | 0 | DRQ[2] | IRQ[1] | IRQ[10] |
| 0 | 1 | 1 | DRQ[3] | IRQ[4] | IRQ[11] |
| 1 | 0 | 0 | DRQ[5] | IRQ[5] | IRQ[12] |
| 1 | 0 | 1 | DRQ[6] | IRQ[6] | IRQ[14] |
| 1 | 1 | 0 | DRQ[7] | IRQ[7] | IRQ[15] |
| 1 | 1 | 1 | Ring-In | IRQ[3] | EXTSW |

| XDK[2] | XDK[1] | XDK[0] | DACK |
| --- | --- | --- | --- |
| 0 | 0 | 0 | DACK[0] |
| 0 | 0 | 1 | DACK[1] |
| 0 | 1 | 0 | DACK[2] |
| 0 | 1 | 1 | DACK[3] |
| 1 | 0 | 0 | DACK[5] |
| 1 | 0 | 1 | DACK[6] |
| 1 | 1 | 0 | DACK[7] |
| 1 | 1 | 1 | ENPOWER |

NOTE:	RING-IN: The signal comes from the Modem.

EXTSW : External Switch

ENPOWER:This signal controls the 74LS373s which latch the 12 control power pins status.Parity Checking and Parity-Generation LogicDuring a local memory-read cycle, M1429L not only monitors bus steering, but also checks the parity bit for each data byte from DRAM to ensure that the correct data is read. If a parity error occurs, the M1429L generates an NMI-interrupt to CPU for the parity error. The parity-check can be disabled by programming index 17. During a local memory-write cycle, M1429L monitors bus steering and uses the accepted data to generate the parity bits sent to DRAM for each data byte.Data Conversion Logic Among 32, 16, and 8-BitsThe CPU data bus and memory data bus are 32-bit data buses. The ISA data bus is 16-bits. The external data bus is 8-bit. When data transfers between these buses at different cycles, byte- or word-conversion is required to keep the system working correctly. Internal RTCThe RTC device is built-in, thus no external RTC is required. If the user do not use the internal RTC for something else, then it can be disabled by hardware setting.  The M1431 generates RTC control signal .PCI Interface LogicIn PCI systems, M1429L recognizes the local bus cycle including the VESA and PCI devices, or issues an ISA cycle via pin 71 signal which connects to ALI M1435 PCI bridge XISARQJ pin. The M1429L provides two 8-bit R/W registers for supporting edge- or level-sensitive interrupts on a channel-by-channel basis. The two registers are 04D0h for master interrupt controller and 04D1h for slave interrupt controller. These registers are used to set the interrupts to be triggered by either the signal edge or the logic level. IRQ0, IRQ1, IRQ2, IRQ8, IRQ13 must be set to edge sensitive for AT traditional use. After the system resets, all IRQ signals are set to edge sensitive. The following description shows which bit numbers represent the various IRQ signals.( 0 = edge sensitive; 1 = level sensitive. )

Port 04D0h     (R/W)    00h        D0  :  IRQ0        D1  :  IRQ1        D2  :  IRQ2        D3  :  IRQ3        D4  :  IRQ4        D5  :  IRQ5        D6  :  IRQ6        D7  :  IRQ7     Port 04D1h     (R/W)    00h        D0  :  IRQ8        D1  :  IRQ9        D2  :  IRQ10        D3  :  IRQ11        D4  :  IRQ12        D5  :  IRQ13        D6  :  IRQ14        D7  :  IRQ15SMM Logic controlSMM (System Memory Management) Logic handles the memory mapping function when SMI event is active. It is different for Intel, Cyrix, AMD, TI CPU systems.  The SMRAM size is 64KB or 128KB.  SMM Logic determines whether SMRAM can or can not be redirected.  SMM Logic also controls the SMRAM cacheable issue, and the cacheable issue for the whole memory when SMI event is active.

If SMRAM is not redirected, SMRAM can be programmed to access directly without an active SMI event by setting index 43H~D6 to "1".  If SMRAM is not redirected, the SMRAM cannot be cacheable for both internal and external cache systems.  When SMRAM is redirected to a local memory non-continuous region, the system can access that region even without an SMI active event .

If the SMRAM address is moved to the split region of M1429L (i.e. the SMBASE descriptor is redirected to differ with the CPU default address), the CPU SRESET signal pin should be connected to M1429L_RSTCPU ( pin5 ) and the CPU RESET pin should be connected to the M1429L_SYSRST (pin2 ) pin that is shared with the ISA slot RSTDRV signal.

SMM Logic also supports the delay READY# function when the I/O trap is found whether or not the Local Device is present.

Stop Clock Control and Clock Throttle LogicThe STPCLKJ-like signal is asserted by software setting and is de-asserted by optional IRQ, DRQ, NMI & SMI events. The M1429L also provides a programmable timer for clock throttle feature.  So, when the INTR, DRQ, NMI or SMI have been idle for a pre-programmed period of time, the STPCLKJ-like signal asserts until a new INTR, DRQ, NMI or SMI event occurs.  The asserted STPCLKJ-like signal does not break the pre-asserted INTR, NMI, SMI, but it pends the later INTR, NMI and SMI until the STPCLKJ-like signal is de-asserted.  To support 0-Hz STPCLKJ, the system board needs extra logic to control the CPU clock.  The M1429L uses Pin 6 (XIGNNEJ) to issue a 0-Hz starting (level low) & ending (level high) signal during the STPCLKJ active period.  For IBM Blue Lightning, pin 62 of M1429L connects to IBM processor XDFSREQJ pin for supporting the Blue Lightning STPCLKJ-like feature.

SMI Control LogicThe SMI request is generated by the programmable events, and records its related causing-code in the local configuration port. The SMI request can be issued to the processor via SMIJ, NMI or IRQ15 signals. This block controls the SMI I/O drive timing to fit all processors including Intel, AMD, TI and Cyrix CPUs. The SMI request interlocks with the CPU reset signal. The SMI request also drives the A20MJ inactive before entering into SMM, and returns to its original level after the last cycle of the SMM state is restored.  The M1429L's SMIJ is asserted at least 7 CPU CLK1(cycle) periods prior to asserting the CPURDYJ signal in order to guarantee processor recognition on an I/O trap instruction boundary.APM Control LogicThe APM (Advanced Power Management Interface) creates an interface to allow the Operating System to communicate with the SMM code. The M1429L provides the Index Local Port 30h D4 to generate the software STPCLKJ signal, and the Index Local Port 30h D6 to generate the software SMIJ signal for the APM applications.

PMU Control Logic The PMU (Power Management Unit) strictly controls and dramatically reduces overall system power consumption.  This is accomplished via the activity monitors which detect the system inactivity timer timeout, and signals the power-saving device to slow-down the clock frequency or remove the power sources from various peripherals. The M1429L provides seven timers from one second to 320 minutes to individually monitor the system state (ON/DOZE/STANDBY/SUSPEND modes), VGA, HDD, LPT, two programmable regions, and the standard input devices (such as mouse, keyboard, etc.) activity.  The M1429L can select to monitor the keyboard, VGA, HDD, FDD, LPT, COM, IRQs, DRQs, VESA masters and two programmable regions of each system state.  The M1429L provides a LED flash control to indicate the system state status.  The M1429L also provides 12 programmable output control signals for peripheral devices to control power sourcing, or for the external clock synthesis to change its frequency setting.  The M1429L provides seven registers to store the last host IO command for those processors that do not support the I/O restart feature.  The M1429L also supports the external switch, RTC alarm and MODEM ring-in wakeup control.

Section 4 : Configuration RegistersI.  Unlock registers sequence:

Step 1 	 Write port 22H, data 03H

Step 2 	 Write port 23H, data C5H

II. Registers Description :

(a) Hardware setting & Lock register :

### INDEX-01H : ( Read-only )     2FH

D0 	16/8 bit ROM select (SEL[0] )

0 = 16 bit ROM select

1 =  8 bit ROM select

D1 	Reserved for Testing

D2 	Reserved for Testing

(XBUSCSJ) must be PULL-UP for normal 	operation

D3 	DIS/ENable internal RTC   (CPUSPD)

0 =  disable internal RTC	1 =  enable internal RTC

D7D6D5D4  Chip Version

### 0000 =  original M1429 	0001 =  M1429 with PCI interface	0010 =  M1429 with PCI interface & Green 
	              feature

### INDEX-02H : ( Read only )   7FH

D0 	DRAM bank type definition (XDIR)

1 =  2 bank DRAM architecture(pull-up)

0=  4 bank DRAM architecture(pull-down)

D1 	ENABLE PCI interface to M1435 PCI bridge 
	(TC )

0 =  non-PCI system

1 =  PCI system

D2 	Reserved

D3 	1X/2X clock system select (SEL[2])

0 =  1X clock system

1 =  2X clock system

D6D5D4(R/W) 	  CPU type definition (DK[2-0] )

0 0 0 =  486DLC/IBM Blue Lightning system

0 0 1 =  386DX system

0 1 0 =  M6 system

0 1 1 =  486 system

1 1 1 =  Upgradeable 486 system

1 0 1 =  P24T/P24D system

1 1 0 =  M7 system

Else  =  Reserved

NOTE:  The CPU type can also be changed through software programming.  It is  not guaranteed that the system can be changed to any CPU type without limitations. Because the M1429L pinout  is different depending on the CPU type. It may be too late to decide the CPU type when the system is active. Under the upgradable 486 system, the M1429L pinout is the same as P24T/P24D system but the pins which are different  from the 486DX/DX2 system are not functional. So,  the 486 system default setting of the upgradeable 486 type could be set through hardware, then using software programming for upgrading the system to P24T, P24D, or M7 type (this programming should be done before enabling the CPU internal cache).

D7 	Reserved

### INDEX-03H 	 ( R/W )   C5H

D7D6D5D4D3D2D1D0(11000101)

Configuration register ports protection

C5H 	configuration registers unlocking

else 	configuration registers locking

### INDEX-04H 	 ( Read only )   0FH

D0 	GREEN pinout setting (XSPEAK)

0 =  Green pinout setting enable

1 =  Non-green pinout setting

D7D6D5D4D3D2D1   (Reserved)

(b) DRAM & Cache Controllers feature

### INDEX-10H 	 ( R/W )   F0H

D3D2D1D0(0000) :  DRAM config. of BANK 0

D3	D2	D1	D0	DRAM	Configuration

1	1	1	1	None	None

0	0	0	0	1M	256K * 4

0	0	0	1	4M	1M * 4

0	0	1	0	2M	512K * 4

0	0	1	1	16M	4M * 4

0	1	0	0	64M	16M * 4

( else )				Reserved

D7D6D5D4(1111) :  DRAM configuration of BANK 1

Bits definition is same as BANK 0

### INDEX-11H 	 ( R/W )   FFH

D3D2D1D0(1111) : DRAM configuration of BANK 2

Bits definition is same as BANK 0

D7D6D5D4(1111) :  DRAM configuration of BANK 3

Bits definition is same as BANK 0

### INDEX-12H 	 ( R/W )   00H

D0(0) 	ROM size

0 : 64KB ROM/EPROM

(0F0000~0FFFFF/F.F0000~F.FFFFF)

1 : 128KB ROM/EPROM

(0E0000~0FFFFF)

D1(0) 	On-board 0F00000~0FFFFFF memory 
	usage

0 = on-board 15M range memory will be 
	       recognized

1 = on-board 15M range memory will not be 
	       recognized,and will be treated as the 	 
    	       ISA  range

D2(0) 	Memory split remapping

0 = disable memory split remapping feature

1 = enable memory split remapping feature

D3(0) 	RAS active timeout check

0 = disable RAS signal active timeout check 
	       feature

1 = enable RAS signal active timeout check 
	       feature

D4(0) 	Hidden refresh

0 = disable on-board memory hidden refresh

1 = enable on-board memory hidden refresh

D5(0) 	Flash ROM supporting

0 = disable active ROM chip select signal in 
	       the ROM region memory write cycle.

1 = enable active ROM chip select signal in 
	       the ROM region memory write cycle.

D6(0) 	High ROM supporting

0 = disable active ROM chip select signal in 
	       the 0C0000-0C7FFF video ROM region 
	       memory access cycle.

1 = enable active ROM chip select signal in 
	       the 0C0000-0C7FFF video ROM region 
	       memory access cycle.

D7(0) 	Enhanced high ROM chip select supporting

0 = disable active ROM chip select signal in the 0C8000-0CFFFF video ROM region memory access cycle.

1 = enable active ROM chip select signal in the 0C8000-0CFFFF video ROM region memory access cycle.

### INDEX-13H 	 ( R/W )   00H

D0(0) 	0C0000-0C7FFF region shadow control

0 = disable

1 = enable

D1(0) 	0C8000-0CFFFF region shadow control

0 = disable

1 = enable

D2(0) 	0D0000-0D7FFF region shadow control

0 = disable

1= enable

D3(0) 	0D8000-0DFFFF region shadow control

0 = disable

1 = enable

D4(0) 	0E0000-0E7FFF region shadow control

0 = disable

1 = enable

D5(0) 	0E8000-0EFFFF region shadow control

0 = disable

1 = enable

D6(0) 	0F0000-0F7FFF region shadow control

0 = disable

1 = enable

D7(0) 	0F8000-0FFFFF region shadow control

0 = disable

1 = enable

### INDEX-14H 	 ( R/W )   00H

D0(0) 	Shadow region read control

0 = disable

1 = enable

D1(0) 	Shadow region write control

0 = disable

1 = enable

D2(0) 	Weitek coprocessor present feature

0 = without Weitek coprocessor

1 = with Weitek coprocessor

D3(0) 	INTEL 80387 coprocessor present feature

0 = without 80387 coprocessor

1 = with 80387 coprocessor

D4(0) 	PS/2 mouse present feature

0 = without PS/2 mouse

1 = with PS/2 mouse

D5(0) 	reserved

D6(0) 	LDEVJ and On-board memory check priority 	setting.

0 = LDEVJ priority higher than on-board 
	       memory region 
	1 = LDEVJ priority lower than on-board 
	       memory region

D7(0) 	reserved

### INDEX-15H 	 ( R/W )   00H

D0(0) 	Remapping split address A20 definition

D1(0) 	Remapping split address A21 definition

D2(0) 	Remapping split address A22 definition

D3(0) 	Remapping split address A23 definition

D4(0) 	Remapping split address A24 definition

D5(0) 	Remapping split address A25 definition

D6(0) 	Remapping split address A26 definition

D7(0) 	Remapping split address A27 definition

### INDEX-16H 	 ( R/W )   00H

D1D0(00) 	On-board memory read access timing 	definition

0 0 = slow timing for on-board memory read

0 1 = normal timing for on-board memory 
	          read

1 0 = fast timing for on-board memory read 
	         access

1 1 = reserved (fastest if below 25MHz)

D3D2(00) 	On-board memory write access timing 
	definition

0 0 = slow timing for on-board memory write

0 1 = normal timing for on-board memory 
	          write

1 0 = fast timing for on-board memory write

1 1 = reserved(fastest if below 25MHz)

D4(0) 	Mask on-board bank 0 memory

0 = un-mask on-board bank 0 memory

(enable bank 0 memory usage )

1 = mask on-board bank 0 memory

(disable bank 0 memory usage )

D5(0) 	Mask on-board bank 1 memory

0 = un-mask on-board bank 1 memory

(enable bank 1 memory usage )

1 = mask on-board bank 1 memory

(disable bank 1 memory usage )

D6(0) 	Mask on-board bank 2 memory

0 = un-mask on-board bank 2 memory

( enable bank 2 memory usage )

1 = mask on-board bank 2 memory

( disable bank 2 memory usage )

D7(0) 	Mask on-board bank 3 memory

0 = un-mask on-board bank 3 memory

( enable bank 3 memory usage )

1 = mask on-board bank 3 memory

( disable bank 3 memory usage )

### INDEX-17H 	 ( R/W )   7EH

D1D0(10/11) 	Cycle check point select

0 0 = reserved

0 1 = fast

1 0 = normal

1 1 = slow (only available in 1X)

D2(1) 	On-board memory parity check

0 = disable on-board memory parity check

1 = enable on-board memory parity check

D3(1) 	reserved

D5D4(11) 	HITMJ sampling timing definition

D5 D4 =	timing

0 0 =	2t

0 1 = 	3t

1 0 = 	4t

1 1 = 	6t

D6(1) 	reserved

D7(0) 	L1KENJ check timing

0 = first T2 middle for L1KENJ checking

1 = T1 end for L1KENJ checking

### INDEX-18H 	 ( R/W )   00H

D0(0) 	Internal L1 cache ON/OFF

0 = disable L1 cache

1 = enable L1 cache

D1(0) 	External L2 cache ON/OFF

0 = disable L2 cache

1 = enable L2 cache

D2(0) 	Internal L1 cache FLUSH ON/OFF

0 = normal L1 cache operation

1 = flush L1 cache

D3(0) 	reserved

D5D4(00) 	L2 cache SRAM type configuration

0 0 = 8K * 8 SRAM

0 1 = 32K * 8 SRAM

1 0 = 64K * 8 SRAM

1 1 = 128K * 8 SRAM

D6(0) 	L2 cache structure

0 = 1 bank L2 cache SRAM

1 = 2 bank L2 cache SRAM

D7(0) 	Snoop write back CHOLD/BOFFJ operation 
	type

0 = Assert CHOLD after first ADSJ during 
	      HITMJ

1 = Only assert BOFFJ after HITMJ inactive

### INDEX-19H 	 ( R/W )   00H

D0(0) 	Force L2 cache miss feature

0 = normal operation

1 = force L2 cache MISS

D1(0) 	Force L2 cache non-dirty feature

0 = normal operation

1 = force L2 memory read cycles be NON-
	      DIRTY

D2(0) 	Internal L1 cache WB/WT feature

0 = L1 write through select

1 = L1 write back select

D3(0) 	External L2 cache WB/WT feature

0 = L2 write-through select

1 = L2 write-back select

D4(0) 	Internal L1 cache below 16M region WB/WT 
	feature

0 = L1 write-through select

1 = L1 write-back select

D5(0) 	L2 cache controller write timing setting

0 = L2 write-normal timing select

1 = L2 write-long timing select

D6(0) 	L2 cache controller read timing setting

0 = L2 read normal timing select

1 = L2 read long timing select

D7(0) 	Force L2 cache hit feature

0 = normal operation

1 = force L2 cache HIT

### INDEX-1AH 	 ( R/W )   00H

D0(0) 	Shadow region code L2 cacheable feature

0 = disable L2 cacheable shadow region 
	       code  access

1 = enable L2 cacheable shadow region 
	       code access & set to be write-protect

D1(0) 	Shadow region data L2 cacheable feature

0 = disable L2 cacheable shadow region 
	       data access

1 = enable L2 cacheable shadow region 
	       data access & set to be write-protect

D2(0) 	Shadow region code L1 cacheable feature

0 = disable L1 cacheable shadow region 
	       code access

1 = enable L1 cacheable shadow region 
	       code access & set to be write-through

D3(0) 	Shadow region data L1 cacheable feature

0 = disable L1 cacheable shadow region 
	       data access

1 = enable L1 cacheable shadow region 
	       data access & set to be write-through

D5D4(00) 	HITMJ inactive to IOCHRDYJ inactive delay

D5 D4  timing

0 0 = 2t

0 1 = 3t

1 0 = 4t

1 1 = 6t

D7D6(00) 	NON-CPU cycle RAS to CAS delay

0 1 = RAS to CAS delay timing is 2t

1 0 = RAS to CAS delay timing is 4t

1 1 = RAS to CAS delay timing is 6t

0 0 = RAS to CAS delay timing is 6t

### INDEX-1BH 	 ( Read-only )

D7D6D5D4D3D2D1D0 : Tag[7-0]

### INDEX-1CH ==> 1DH  Reserved

### INDEX-1EH  ( R/W )   00H

### IOCHRDYJ asserted at ISA Master command low cycle

D0(0) 	0 = disable

1 = enable

D1(0) 	Extend the internal ISA master command 
	high time	0 = disable	1 = enable

D2(0) 	L2 cache VESA MASTER cycle write timing 
	setting

0 = L2 VESA write normal timing select

1 = L2 VESA write long timing select

D3(0) 	L2 cache VESA cycle read timing setting

0 = L2 VESA read normal timing select

1 = L2 VESA read long timing select

D4(0) 	VESA master cycle delay ADSJ timing 
	setting

0 = VESA master non-delay ADSJ

1 = VESA master delay ADSJ

D5(0) 	Flush at SMM start

0 = disable flush

1 = Enable flush

D6(0) 	Flush at SMM end	0 = disable flush	1 = Enable flush

D7(0) 	SMM region L1 cacheable

0 = SMM L1 non-cacheable	1 = SMM L1 cacheable

### INDEX-1FH 	 reserved

(c) System feature

### INDEX-20H 	 ( R/W )   00H

D2D1D0(000) 	AT CLOCK select setting

0 0 0 = 7.19 MHz

0 0 1 = CLK2/4

0 1 0 = CLK2/5

0 1 1 = CLK2/6

1 0 0 = CLK2/8

1 0 1 = CLK2/10

1 1 0 = CLK2/12

1 1 1 = reserved

D3(0) 	Host address A31-A16 "1" ROM select 
	decoder feature

0 = disable

1 = enable

D6D5D4(000) 	POLLING CLOCK select setting

0 0 0 = 14.318 MHz

0 0 1 = CLK2

0 1 0 = CLK2/2

0 1 1 = CLK2/3

1 0 0 = CLK2/4

1 0 1 = 28.636 MHz

ELSE  = reserved

D7(0) 	DMA CLOCK select setting

0 = DMACLK = ATCLK/2

1 = DMACLK = ATCLK

### INDEX-21H 	 ( R/W )   00H

D2D1D0(000) :  CPU CLOCK2 power-down mode select 
	  setting

0 0 0 = OSC(14.318MHz)

0 0 1 = OSC/2

0 1 0 = OSC/4

0 1 1 = OSC/8

1 0 0 = OSC/16

1 0 1 = OSC/32

1 1 0 = ATCLK2

1 1 1 = ATCLK1

D3(0) 	CPU reset counter clock select

0 = OSC 14.318

1 = CLK1O

D4(0) 	reserved

D5(0) 	CPU immediately power-down mode feature

0 = immediately back to normal operating 
	       mode

1 = immediately enter CPU CLOCK power-
	       down mode

D6(0) 	reserved

D7(0) 	CPU CLOCK power-down in STPCLKJ 
	active period feature (Auto-CLOCK Scaling)

0 = disable output clock power-down in 
	       STPCLKJ active period

1 = enable output clock power-down in 
	       STPCLKJ active period

### Index-22H 	 ( R/W )  00H

D0(0) 	I/O recovery feature

0 = disable I/O recovery feature

1 = enable I/O recovery feature

D1(0) 	On-chip I/O recovery feature

0 = disable on-chip I/O recovery feature

1 = enable on-chip I/O recovery feature

D2(0) 	ISA write cycle end insert wait for slot data 
	hold timing issue

0 = disable ISA write end insert wait

1 = enable ISA write end insert wait
	      (1 ATCLK2)

D3(0) 	Timer counter read operation	(read the index 39H-3FH D7-D4 )	0 = read the timer counter setting	1 = read the timer counter current value

D7D6D5D4(0000) :  I/O recovery period setting

0 0 0 0 = 0 us ( 0/ATCLK )

0 0 0 1 = 0.25 us ( 2/ATCLK )

0 0 1 0 = 0.5 us ( 4/ATCLK )

0 0 1 1 = 0.75 us ( 6/ATCLK )

0 1 0 0 = 1 us ( 8/ATCLK )

0 1 0 1 = 1.25 us ( 10/ATCLK )

0 1 1 0 = 1.5 us ( 12/ATCLK )

0 1 1 1 = 1.75 us ( 14/ATCLK )

1 0 0 0 = 2 us ( 16/ATCLK )

1 0 0 1 = 2.25 us ( 18/ATCLK )

1 0 1 0 = 2.5 us ( 20/ATCLK )

1 0 1 1 = 2.75 us ( 22/ATCLK )

1 1 0 0 = 3 us ( 24/ATCLK )

1 1 0 1 = 3.25 us ( 26/ATCLK )

1 1 1 0 = 3.5 us ( 28/ATCLK )

1 1 1 1 = 3.75 us ( 30/ATCLK )

### Index-23H 	 ( R/W )  5BH

D0(1) 	TURBO mode HW/SW priority setting

0 = hardware HW (TURBO-switch) HIGH 
`	      priority first

1 = HW/SW equal priority

D1(1) 	SW TURBO setting control

0 = SW de-turbo setting

1 = SW turbo setting

D2(0) 	HW TURBO switch enable feature

0 = disable HW TURBO switch response

1 = enable HW TURBO switch response

D3(1) 	HW TURBO switch status(read-only and be 
	meaningful only while D2 is set to "1")

0 = HW TURBO is in de-turbo status (OFF)

1 = HW TURBO is in turbo status ( ON )

D4(1) 	TURBO performance status (read-only)

0 = system is in de-turbo mode status 
	      (TURBO LED = OFF )

1 = system is in turbo mode status 
	      (TURBO LED = ON )

D5(0) 	L1 cache control in de-turbo status

0 = cache off in de-turbo status

1 = cache on in de-turbo status

D6(1) 	Flush control when entering de-turbo status

0 = de-assert FLUSH when entering de-
	       turbo

1 = assert FLUSH when entering de-turbo 
	       status

D7(0) 	Transparent interrupt request in de-turbo 	status

0 = normal de-turbo hold request period 	 
	      setting

1 = cut-off de-turbo hold request while 
	      interrupt request is asserted

### INDEX-24H 	 ( R/W )  00H

D1D0  	SMI timing select

00 = SMI timing support

01 = NMI timing support

10 = IRQ15 timing support

11 = reserved

D3D2	SMI control select

00 = Intel SMI support

01 = Cyrix/TI SMI support

10 = AMD/IBM SMI support	11 = reserved

D7D6D5D4(0000) :  Hold request time setting in de-turbo 
	         status

0 0 0 0 = 0/AT CLOCK2

0 0 0 1 = 1/AT CLOCK2

0 0 1 0 = 2/AT CLOCK2

0 0 1 1 = 3/AT CLOCK2

0 1 0 0 = 4/AT CLOCK2

0 1 0 1 = 5/AT CLOCK2

0 1 1 0 = 6/AT CLOCK2

0 1 1 1 = 7/AT CLOCK2

1 0 0 0 = 8/AT CLOCK2

1 0 0 1 = 9/AT CLOCK2

1 0 1 0 = 10/AT CLOCK2

1 0 1 1 = 11/AT CLOCK2

1 1 0 0 = 12/AT CLOCK2

1 1 0 1 = 13/AT CLOCK2

1 1 1 0 = 14/AT CLOCK2

1 1 1 1 = 15/AT CLOCK2

### INDEX-25H 	 ( R/W )  C0H

D1D0(00) 	16-bit ISA I/O command insert wait count 
	control feature

0 0 = normal 16-bit ISA I/O access timing

0 1 = 16-bit ISA I/O access cycles insert 1 
	          wait

1 0 = 16-bit ISA I/O access cycles insert 2 
	          waits

1 1 = 16-bit ISA I/O access cycles insert 3 
	          waits

D3D2(00) 	16-bit ISA mem command insert waits 
	count control feature

0 0 = normal 16-bit mem I/O access timing

0 1 = 16-bit ISA mem access cycles insert 1 
	          wait

1 0 = 16-bit ISA mem access cycles insert 2 
	          waits

1 1 = 16-bit ISA mem access cycles insert 3 
	          waits

D5D4(00) 	Refresh period setting

0 0 = 15 us refresh period

0 1 = 30 us refresh period

1 0 = 60 us refresh period

1 1 = 120 us refresh period

D7D6(11) 	Second 16-bit access wait timing in 32-bit 
	ISA cycle

0 0 = 2t (1t=1/CPUCLK2)

0 1 = 4t

1 0 = 6t

1 1 = 8t

### INDEX-26H : ( R/W ) 00H

D1D0 	The time of 0 Hz/PDCLK wake up control 
	signal before de-asserting the STPCLKJ	 0 0 = 2 ms	 0 1 = 1 ms	 1 0 = 0.5 ms	 1 1 = 0.25 ms

D2 	0 Hz control signal feature	 0 = disable	 1 = enable

D4D3 	IBM snoop timing feature support	1 0 = enable IBM snoop timing feature	else = normal snoop feature

D5 	Specific pinout required feature	0 = normal pinout definition1 = if CPURDYJ and SMIRDYJ is executed in the same timing and function ( i.e. the SMIRDYJ is connected to CPURDYJ in the board design ) of the AMD/IBM system, the M1429L_SMIRDYJ pin changes to STPCLKJ_like function signal when this bit is set to '1'.

D6 	IBM Blue Lightning DFSREQJ signal feature	0 = disable	1 = enable

D7 	reserved

### INDEX-27H 	 ( R/W )  00H

D0(0) 	LOCAL device (NP) ready sync. mode setting

0 = sync. L(NP)RDY to CPURDY(RDYRTN)

1 = combination bypass L(NP)RDY to 
	      CPURDY(RDYRTN)

D1(0) 	Delay internal ADSJ feature setting

0 = disable delay internal ADSJ

1 = enable delay internal ADSJ

D2(0) 	L2 cache write hit CWEJ pulse setting in 
	386 system

0 = disable CWEJ delay tail rising edge

1 = enable CWEJ delay tail rising edge

D3(0) 	SMI acknowledge feature	0 = CPU with SMI feature	1 = software SMIACKJ

(CPU without SMI feature )

D5D4(00) 	Delay ISA cycle LDEVJ check point

0 0 = ISA check LDEVJ same as cycle check 
	        point

0 1 = ISA check LDEVJ delay 1 CLK2 after 
	        cycle check point

1 0 = ISA check LDEVJ delay 2 CLK2 after 
	        cycle check point

1 1 = ISA check LDEVJ delay 3 CLK2 after 
	        cycle check point

D7D6(00) 	TAGWEJ delay rising timing setting

0 0 = delay 0 ns

0 1 = delay 1 ns

1 0 = delay 2 ns

1 1 = delay 3 ns

Memory performance mapping

Miss read & write RAS precharge timing definition

read fastest or write fastest 	 4t

else                          	 5t

RAS to CAS delay timing definition

read slow and write slow      	 6t

else                          	 4t

Miss read CAS access timing definition

read slow                     	 5t

read else                     	 4t

Hit read CAS access timing definition

read slow                     	 5t

read normal                   	 4t

read fast                     	 3t

read fastest                  	 2t

Hit write CAS access delay timing definition

write slow                    	 3t

write normal                  	 2t

write fast                    	 1t

write fastest                 	 0t

Line-fill CAS access timing definition

read slow                     	 4t

read normal                   	 4t

read fast                     	 4t

read fastest                  	 2t

Write-back CAS precharge timing definition

write slow                    	 6t

write normal                  	 4t

write fast                    	 4t

write fastest                 	 4t

Line-fill CAS precharge timing definition

read slow                    	 6t

read normal                  	 4t

read fast                    	 2t

read fastest                 	 2t

M1429L PMU configuration register  I

### INDEX-1EH : ( R/W ) 00H

D0(0) 	IOCHRDYJ asserted in ISA Master 
	command low cycle	0 = disable	1 = enable

D1(0) 	Extend the internal ISA master command 
	high time	0 = disable	1 = enable

D4D3D2 	Ref. the previous definition

D5(0) 	Flush at SMM start	0 = disable flush	1 = Enable flush

D6(0) 	Flush at SMM end	0 = disable flush	1 = Enable flush

D7(0) 	SMM region L1 cacheable	0 = SMM L1 non-cacheable	1 = SMM L1 cacheable

### INDEX-22H 	 ( R/W ) 00H

D3(0) 	Timer counter read operation	( read the index 39H-3FH D7-D4 )	0 = read the timer counter setting	1 = read the timer counter current value

D7D6D5D4D2D1D0 : Ref. the previous definition

### INDEX-24H 	 ( R/W ) 00H

D1D0  	SMI timing select

00 = SMI timing support

01 = NMI timing support

10 = IRQ15 timing support	11 = Reserved

D3D2  	SMI control select

00 = Intel SMI support

01 = Cyrix SMI support

10 = AMD/IBM SMI support	11 = Reserved

D7D6D5D4 	Refer to the previous definition

### INDEX-26H 	 ( R/W ) 00H

D1D0 	The time of 0 Hz/PDCLK wake up control 	signal before de-asserting the STPCLKJ	0 0 = 2 ms	0 1 = 1 ms	1 0 = 0.5 ms	1 1 = 0.25 ms

D2 	0 Hz control signal feature	0 = disable	1 = enable

D4D3 	IBM snoop timing feature support	1 0 = enable IBM snoop timing feature	else = normal snoop feature

D5 	Specific pinout required feature

0 = normal pinout definition1 = if CPURDYJ and SMIRDYJ is executed in the same timing and function   ( i.e. the SMIRDYJ connects to CPURDYJ in the board design) of the AMD/IBM system, the M1429L_SMIRDYJ pin changes to STPCLKJ_like function signal when this bit is set to '1'

D6 	IBM Blue Lightning DFSREQJ signal feature	0 = disable	1 = enable

D7 	Reserved

### INDEX-28H==> 2FH Reserved

### INDEX-30H 	 ( R/W )

D1D0(00) 	System mode state

00 = ON

01 = DOZE	10 = STANDBY	11 = SUSPEND

D2(0) 	PMU feature control

0 = disable

1 = enable

D3(0) 	SMM region mapping control	0 = force fixed SMM region to split region	1 = normal split address region
	      (SMM in split address )

D4(0) 	STPCLKJ control 
	(optional ref. index-30h D5, index-32h D2 )	0 = disable STPCLKJ	1 = asserts software STPCLKJ

D5(0) 	DMA/Local Master request de-assert 	STPCLKJ	0 = DMA/Local Master request will de-assert 
	      STPCLKJ	1 = DMA/Local Master request will not de-
	      assert STPCLKJ

D6(0) 	Software SMI	0 = disable software SMI 	1 = asserts software SMI

D7(0) 	SMI event signal

0 = disable

1 = enable

SMIJ  control

### INDEX-31H 	 ( R/W ) 00H

Mode select the combination of Idle detection.  Mode event is active once selected items all reach their time-out.

( 0: unselect;  1: select )

D0 	KBD select  ( 60H, 64H)

D1 	VGA select  
	(Mem AB region write, 3B0-3BFH write)

D2 	HDD select  ( 1F0-1F7H )

D3 	IRQ select

D4 	LPT select  
	( 378-37FH, 278-27FH, 3BC-3BEH )

D5 	COM select  
	(3F8-3FFH, 2F8-2FFH, 3E8-3EFH, 2E8-
	2EEH)

D6 	FDD select  ( 3F0-3F7H )

D7 	DRQ select

0 = is not selected

1 = is selected

### INDEX-32H 	 ( R/W ) 00H

D0 	GP0 select

0 = is not selected for detection

1 = is selected for detection

D1 	GP1 select

0 = is not selected for detection

1 = is selected for detection

D2(0) 	INTR/NMI event de-assert STPCLKJ	0 = INTR/NMI event will de-assert STPCLKJ	1 = INTR/NMI event will not de-assert 	 
	      STPCLKJ

D5D4D3(000) 	Clock Throttling feature: Automatically asserts STPCLKJ for power saving when INTR /NMI /SMIJ /DMA request idle timer timeout condition is detected 
(optional ref. index-30h D5, index-32h D2 )	000 = disable	001 =250 us	010 = 500 us	011 = 1 ms	100 = 2 ms	101 = 5 ms	110 = 10 ms	111 = 20 ms

SMI register selects the source for generating SMI or

activating intelligent Powerdown mechanism.

( 0: unselect; 1: select )

D6(0) 	RING_IN active

0 = is not selected

1 = is selected

D7(0) 	EXTSW active (external suspend switch 
	input, level trigger)

0 = is not selected

1 = is selected

### INDEX-33H 	 ( R/W ) 00H

D0(0) 	VGA timer time-out

D1(0) 	HDD timer time-out

D2(0) 	LPT timer time-out

D3(0) 	GP0 timer time-out

D4(0) 	GP1 timer time-out

D5(0) 	Mode timer time-out

D6(0) 	RTC alarm

D7(0) 	IN time-out

0 = is not selected

1 = is selected

### INDEX-34H 	 ( R/W ) 00H

D(0) 	VGA access 
	(W A0000~BFFFFH, port 3B0~3BFH)

D1(0) 	HDD access (R/W port 1F0~1F7H)

D2(0) 	LPT access (R/W 378~37FH or 278~27Fh)

D3(0) 	GP0 access (R/W GP0 defined area)

D4(0) 	GP1 access (R/W GP1 defined area)

D5(0) 	IRQ active

D6(0) 	DRQ active

D7(0) 	IN access

0 = is not selected

1 = is selected

### INDEX-35H 	 ( Read-only ) 00H   SMIJ cause

D7(0)(R/W) 	IRQ/NMI de-asserts STPCLKJ setting recongnized only when index 32H D2 is set to '0'	0 = INTR/NMI event de-assert STPCLKJ	1 = IRQ de-asserts STPCLKJ depending on 
	        the index-36H, 37H setting

D6(0)(R/W) 	DRQ/LREQJ de-asserts STPCLKJ setting recongnized only when index 30H D5 is  set to '0'	0 = DRQ/LREQJ event de-assert STPCLKJ	1 = DRQ de-asserts STPCLKJ depending 
	       on index-38H setting

D5(0)(R/W) 	Ring-in extension polarity select	0 = only high-to-low is recognized	1 = both high-to-low and low-to-high are 	 
	       recognized

D4	D3	D2	D1	D0	Cause

0	0	0	0	0 	NONE

0	0	0	0	1	VGA timer time-out

0	0	0	1	0	HDD timer time-out

0	0	0	1	1	LPT timer time-out

0	0	1	0	0	GP0 timer time-out

0	0	1	0	1	GP1 timer time-out

0	0	1	1	0	Mode timer time-out

0	0	1	1	1	IN timer time-out

1	0	0	0	1	VGA access (W   A0000~BFFFFH,
					 port 3B0~3BFH)

1	 0	0	1	0	HDD access (R/W 1F0~1F7H)

1	0	0	1	1	LPT access (R/W 378~37FH or 
					278~27Fh)

1	0	1	0	0	GP0 access (R/W GP0 defined area)

1	0	1	0	1	GP1 access (R/W GP1 defined area)

0	1	0	0	0	IRQ active

0	1	0	0	1	DRQ active

0	1	0	1	0	IN access

0	1	0	1	1	RING-in active (MODEM)

0	 1	1	0	0	EXESW active 
					(external suspend switch input)0	1	1	0	1	RTC alarm0	1	1	1	1	Software SMI

other				Reserved

### INDEX-36H 	 ( R/W ) 00H

IRQ event chennal selected control

0 = non-selected in the IRQ group event	1 = selected in the IRQ group event

D0(0) 	IRQ0 selected

D1(0) 	IRQ1 selected

D2(0) 	NMI selected

D3(0) 	IRQ3 selected

D4(0) 	IRQ4 selected

D5(0) 	IRQ5 selected

D6(0) 	IRQ6 selected

D7(0) 	IRQ7 selected

### INDEX-37H 	 ( R/W ) 00H

IRQ event channel selected control

0 = non-selected in the IRQ group event1 = selected in the IRQ group event

D0(0) 	IRQ8 selected

D1(0) 	IRQ9 selected

D2(0) 	IRQ10 selected

D3(0) 	IRQ11 selected

D4(0) 	IRQ12 selected

D5(0) 	IRQ13 selected

D6(0) 	IRQ14 selected

D7(0) 	IRQ15 selected

### INDEX-38H 	 ( R/W ) 00H

DRQ event chennal selected control

0 = non-selected in the DRQ group event	1 = selected in the DRQ group event

D0(0) 	DRQ0 selected

D1(0) 	DRQ1 selected

D2(0) 	DRQ2 selected

D3(0) 	DRQ3 selected

D4(0) 	VESA master selected

D5(0) 	DRQ5 selected

D6(0) 	DRQ6 selected

D7(0) 	DRQ7 selected

### INDEX-39H 	 ( R/W ) 00H

The time based should associated with time count to choice

proper time. the relationship is below tome count

timebased	1s	10s	1min	10min

0	1	10	1	10

1	2	20	2	20

2	3	30	3	30

3	4	40	4	40

4	5	50	5	50

5	6	60	6	60

6	7	70	7	70

7	8	80	8	80

8	9	90	9	90

9	10	100	10	100

10	11	110	11	110

11	12	120	12	120

12	13	130	13	130

13	14	140	14	140

14	15	150	15	150

15	16	160	16	160

The minum interval is 1s. The maximum is 160 min.

D1D0(00) 	Time Base select for VGA timer

00  =1s

01  =10s

10  =1min

11  =10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3 	Reserved

D7D6D5D4 	Set the time-out period of VGA timer

VGA timer is reset by write accessing A0000-BFFFFH or port 3B0-3BFH

0-15 = time count

### INDEX-3AH 	 ( R/W ) 00H

D1D0(00) 	Time Base select for HDD timer

00  = 1s

01  = 10s

10  = 1min

11  = 10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3 	Reserved

D7D6D5D4 	set the time-out period of HDD timer which is reset by accessing port 1F0-1F7H

0 -15 = time count

### INDEX-3BH 	 ( R/W ) 00H

D1D0(00) 	Time Base select for LPT timer

00  = 1s

01  = 10s

10  = 1min

11  = 10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3 	Reserved

D7D6D5D4 	set the time-out period of LPT timer

LPT timer is reset by accessing port 378-37FH or 278-27Fh or 3BCH-3BEH

0 -15 = time count

### INDEX-3CH 	 ( R/W ) 00H

D1D0(00) 	 Time Base select for GP memory timer

00  = 1s

01  = 10s

10  = 1min

11  = 10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3 	Reserved

D7D6D5D4 	set the time-out period of GP memory timer

GP timer is reset by accessing a predfined 
	memory

0 -15 = time count

### INDEX-3DH 	 ( R/W ) 00H

D1D0(00) 	 Time Base select for GP IO timer

00  = 1s

01  = 10s

10  = 1min

11  = 10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3(0) 	General IO port decode for address A15-
	A10	0 = general IO address A15-A10 is '0'	1 = general IO address A15-A10 is ignored

D7D6D5D4 	set the time-out period of GP IO timer

GP IO timer is reset by accessing a 
	predfined I/O region

0 -15 = time count

### INDEX-3EH 	 ( R/W ) 00H

D1D0(00) 	Time Base select for Mode timer

00  = 1s

01  = 10s

10  = 1min

11  = 10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3 	Reserved

D7D6D5D4 	set the time delay from PRESENT mode to 
	NEXT mode

0 -15 = time count

### INDEX-3FH 	 ( R/W ) 00H

D1D0(00) 	Time Base select for IN timer

00  = 1s

01  = 10s

10  = 1min

11  = 10min

D2 	Timer count/reset

0 = timer reset	1 = timer count

D3 	Reserved

D7D6D5D4 	set the time-out period of IN timer

IN timer is reset by asserting IRQ1, IRQ12, 
	IRQ3, IRQ4

0 -15 = time count

### INDEX-40H 	 ( R/W ) 00H -

D0(0) 	VGA monitor memory write A0000H-
	BFFFFH

0 = disable	1 = enable

D1(0) 	VGA monitor IO write 3B0h-3BFH

0 = disable	1 = enable

D2(0) 	Software STPCLKJ active function in SMI 	routine	0 = disable	1 = enable

D3(0) 	Setup Switch Disable/Enable	0 = disable	1 = enable

The set up switch is enabled when the system needs two external switches: one for system status, another for setting up the system.  When the function is active, the mother board needs an extra circuit to distinguish set up from Ring-in. It needs a normal off switch and maintain level high 1 second at least when the switch active.

D4(0) 	Setup Switch Status ( Read-only)

0 = set up switch does not access

1 = set up switch access

IN monitor IRQ1 (KB) always

IRQ12 (ps2 mouse) select

IRQ4  (com1) select

IRQ3  (com2) select

D5(0) 	IN monitor IRQ12

0 = disable	1 = enable

D6(0) 	IN monitor IRQ4

0 = disable	1 = enable

D7(0) 	IN monitor IRQ3

0 = disable	1 = enable

### INDEX-41H 	 ( R/W ) 00H

D1D0 	EXTSW input polarity select

D0 	0 - disable low to high active

1 - enable low to high active

D1 	0 - disable high to low active	1 - enable high to low active

D2(0) 	Double counter time base	0 = disable	1 = enable

D3(read-only) 	  IRQ access

D4(read-only) 	  DRQ access

D5(Read-only) 	  IN time-out

D6(Read-only) 	  EXT SW status

D7(Read-only) 	  SMI event

### INDEX-42H 	 ( R/W ) 00H

D0(0) 	PWR0 control pin status

D1(0) 	PWR1 control pin status

D2(0) 	PWR2 control pin status

D3(0) 	PWR3 control pin status

D4(0) 	PWR4 control pin status

D5(0) 	PWR5 control pin status

D6(0) 	PWR6 control pin status

D7(0) 	PWR7 control pin status

### INDEX-43H 	 ( R/W ) 00H

D0(0) 	PWR8 control pin status

D1(0) 	PWR9 control pin status

D2(0) 	PWR10 control pin status

D3(0) 	PWR11 control pin status

D5D4  	ACCESS sense point

D5	D4   T2  begin delay	0	0	0 	0	1	1  	1	0	2	1	1	3

D6(0) 	Access SMM Memory MAP at normal cycle

0 = Disable SMM memory map at normal 	
	      cycle. Access SMM memory only at 
	      SMM mode. 	1 = Enable SMM memory map at normal 
	      cycle. This function initializes the 
	      SMM memory code for later SMI use.

D7(0) 	SMM memory size

0 = 64K byte   (A0000H-AFFFFH)

1 = 128K byte   (A0000H-BFFFFH)

### INDEX-44H 	 ( Read only ) 00H

D0(0) 	VGA timer time-out

D1(0) 	HDD timer time-out

D2(0) 	LPT timer time-out

D3(0) 	GP0 timer time-out

D4(0) 	GP1 timer time-out

D5(0) 	DOZE timer time-out

D6(0) 	STANDBY timer time-out

D7(0) 	SUSPEND timer time-out

0 = is not time-out

1 = is time-out

### INDEX-45H 	 ( R/W ) 00H

D0(0/1) 	Disable/Enable GP0 memory address
	( depends on setting of idices 46, 47, 48H)

D1(0/1) 	Disable/Enable GP1 I/O address ( depends
	on index 45-D2D3 and index 49 setting)

D3D2 	Define IO address mask A[3-2]

0 = is not masked

1 = is masked

D4(0/1)	Disable/Enable GP1 IO Address 100H~1FFH

( independs on any config)

D5(0/1)	Disable/Enable GP1 IO Address 200H~2FFH

( independs on any config)

D6(0/1)	Disable/Enable GP1 IO Address 300H~3FFH

( independs on any config)

D7(0/1)	Disable/Enable Pin67 to output drive LGNTJ

### INDEX-46H 	 ( R/W ) 00H

D[7-0] 	Define memory address A[23-16]

### INDEX-47H 	 ( R/W ) 00H

D[7-0] 	Define memory address mask A[23-16]

0 = is not masked

1 = is masked

### INDEX-48H 	 ( R/W ) 00H

D[3-0] 	Define memory address A[27-24]

D[7-4] 	Define memory address mask A[27-24]

0 = is not  masked

1 = is masked

### INDEX-49H 	 ( R/W ) 00H

D[7-0] 	define I/O port SA[9-2]

### INDEX-4AH  (R/W) 00H

D[1-0] 	MODE LED duty cycle

D1, D0 	0 0 = disable	0 1 = 25%

1 0 = 50%

1 1 = 75%

D[4-2]	MODE LED FREQ

D4	D3	D2	sec

0	0	0	0.4

0	0	1	0.8

0	1	0	1.6

0	1	1	3.2

1	0	0	4.0

1	0	1	8.0

1	1	0	16.0

1	1	1	32.0

D6	D5 (00) 	Ring Counter0	0	10	1 	41	0 	161	1 	64

D7(0) 	Last IO COMMAND R/W feature	0 = record only the WRITE command	1= record both READ and WRITE command

### INDEX-4BH 	 (R) 00H

D[7-0] 	shadow IO port for port 70H data

### INDEX-4CH 	 (R) 00H CPU LAST IO COMMAND

D[7-0] 	A[15-8]

### INDEX-4DH 	 (R) 00H CPU LAST IO COMMAND

D7D6   	Reserved

D[5-0] 	A[7-2]

### INDEX-4EH 	 (R) 00H CPU LAST IO COMMAND

D[3-0] 	BEJ[3-0]

D4	WRJ

### INDEX-4FH 	 (R) 00H CPU LAST IO COMMAND

D[7-0] 	D[31-24]

### INDEX-50H 	 (R) 00H CPU LAST IO COMMAND

D[7-0] 	D[23-16]

### INDEX-51H 	 (R) 00H CPU LAST IO COMMAND

D[7-0] 	D[15-8]

### INDEX-52H 	 (R) 00H CPU LAST IO COMMAND

D[7-0] 	D[7-0]

### INDEX-53~56H

D[7-0] 	Reserved

### INDEX-57H 	 (W) POWER CONTROL STATUS OUTPUT COMMAND

# Section 5:  Programming Guide

Example 1: Mult Time-out eventio_write(0x22,0x30);io_write(0x23,0x84);/* Enable PMU and Enable SMI */io_write(0x22,0x33);io_write(0x23,0x9F);/* Enable Time-out event is VGA,HDD,LPT,GP1,GP0,IN (standard input)*/io_write(0x22,0x39);io_write(0x23,0x77); /* Enable VGA Time Base is 80 min*/io_write(0x22,0x3A);io_write(0x23,0x77); /* Enable HDD Time Base is 80 min */io_write(0x22,0x3B);io_write(0x23,0x77); /* Enable LPT Time Base is 80 min*/io_write(0x22,0x3C);io_write(0x23,0x77); /* Enable GP0 Time Base is 80 min*/io_write(0x22,0x3D);io_write(0x23,0x77); /* Enable GP1 Time Base is 80 min*/io_write(0x22,0x40);io_write(0x23,0x83);/* Monitor VGA memory and IO region, and keyboard ,mouse access */io_write(0x22,0x45);io_write(0x23,0x03);/* Monitor GP0 & GP1 address *//* Program GP0 & GP1 address region */io_write(0x22,0x3F);io_write(0x23,0x77); /* Enable IN Time Base is 80 min*//* SMI is generated by anyone selected devices time-out.*/                	 /* Wait for SMIACT# */io_write(0x22,0x35); io_read(0x23);     /* To find the SMI event is caused by which one.	The example is VGA time-out.*/io_write(0x22,0x44);io_read(0x23);     /* Read the time-out status */io_write(0x22,0x30);io_write(0x23,0x04);/* de-assert SMI */ /* SMI routine (Turn-off the power of VGA or change the speed of VGA.   M1429L support 12 power control pins status to control the peripheral   devices */io_write(0x22,0x42);io_write(0x23,0x01);/* Set the power status */io_write(0x22,0x57);io_write(0x23,0x55);/* Output the power status to turn-off VGA power or change the speed of VGA */io_write(0x22,0x33);io_write(0x23,0x9E);/* Clear VGA time-out event let next time-out event can enter if other time-out event has been selected and		 time-out.io_write(0x22,0x34);io_write(0x23,0x01);/* Set wake-up event is VGA because the VGA is in power		   saving mode. */

Note: io_write(0x22,0x44)      io_read(0x23);/* Read Time-out Status, if the time-out event is not only one, we can go the SMI roution for the time-out                     events in the same SMI assert. For example, The VGA,HDD,LPT is time-out at the same time.*/      /* SMI routine for HDD power saving */      /* SMI routine for LPT power saving */      io_write(0x22,0x33);      io_write(0x23,0x98);/* Clear HDD and LPT time-out event */      io_write(0x22,0x34);      io_write(0x23,0x07);/* Set wake-up event VGA,HDD and LPT */io_write(0x22,0x30);io_write(0x23,0x84);/* Enable SMI */RSM;                /End of the mode time-out */

Example 2: MODE TRANSLATE     The DOZE, STANDBY, SUSPEND MODE is defined by the BIOS.io_write(0x22,0x30);/* Enable PMU System */io_write(0x23,0x84);/* Enable SMI and System Status at ON MODE. It can be in DOZE or STANDBY or SUSPEND MODE by set 
	D1D0. */io_write(0x22,0x31);io_write(0x23,0x01);/* Monitor Keyboard ---> VGA,HDD,IRQ,LPT,COM,FDD,DRQ,GP0,GP1 can be select also. When the 
		selected devices all  reach their time-out, the MODE SMI will be generated.
		The IRQ and DRQ are selected a group of IRQ and DRQ by index36,37,38.*/io_write(0x22,0x33);io_write(0x23,0x20);/* Enable Mode Time-out */io_write(0x22,0x3E);io_write(0x23,0x87);/* Enable Mode Time Base is 90 min*//* SMI generated when the keyboard is not any action reach 90 min. And  the system will be in DOZE mode as the SMI routine is finished.  If the system is in DOZE mode initially, the system will entry STANDBY MODE. Thus---> ON --> DOZE ---> STANDBY ---> SUSPEND */                    /* wait for SMIACT# */io_write(0x22,0x30); io_write(0x23,0x04);/* De-asserted SMI */io_write(0x22,0x35);io_read(0x23); /* Read SMI event is generated by which one.--> This example will be the mode time-out*/io_write(0x22,0x44);io_read(0x23);      /* Read time-out Status *//* SMI routine */io_write(0x22,0x42);io_write(0x23,0x55);/* Set power control pins status */io_write(0x22,0x43);io_write(0x23,0x0A);/* Set power control pins status */io_write(0x22,0x57);io_write(0x23,0xAA);/*Output power control pins status */

io_write  (0x22,0x33);

io_write 0x23,0x00);/* Clear-mode time out event let next time-out event can entry if other time-out event has been selected and 
	time-out.  (the example is only  mode time out is selected thus set "00")  M1429L has seven timers to monitor 
	VGA, HDD, LPT, GP0, GP1, MODE, and standard input.  We can select everyone at the same time with 
	different time base.*/

io_write (0x22,0x30);

## io_write (0x23,0x80)  ;/ Set wake-up event is standard input. It can be mouse or keyboard etc. The user can select ring-in,external
	switch(Index 32 D7D6), VGA,HDD,LPT,GP0,GP1,IRQ,DRQ, standard input(Index 34) to wake-up the 
	system.*/io_write(0x22,0x30); io_write(0x23,0x84);/* Enable SMI */RSM;                /* End of the mode time-out */

Example 3: General Purposeio_write(0x22,0x30);io_write(0x23,0x84);/* Enable PMU and Enable SMI */io_write(0x22,0x33);io_write(0x23,0x18);/* GP1 and GP0 time-out Select */io_write(0x22,0x45);io_write(0x23,0x03);/* Enable GP0 & GP1*/io_write(0x22,0x46);io_write(0x23,0xFF);/* Enable Memory Addr FF0000H */io_write(0x22,0x48);io_write(0x23,0x0F);/* Enable Memory Addr 27-24=F */io_write(0x22,0x49);io_write(0x23,0xFF);/* Enable GP1 IO Addr 9-2=FF */io_write(0x22,0x3C);io_write(0x23,0x77);/* Enable GP0 (GP memory) Time Base is 80 min*/io_write(0x22,0x3D);io_write(0x23,0x67);/* Enable GP1 (GP I/O) Time Base is 70 min*//* The GP0 and GP1 can be defined by the user for any special peripheral devices. The region is defined by index 45H to 49H and  
	the D3 of  index 3DH */ /* The same of time-out routine */

Example 4: External Switchio_write(0x22,0x30);io_write(0x23,0x84);/*Enable PMU & Enable SMI */io_write(0x22,0x41);io_write(0x23,0x03);/*Enable external switch low to high active & high to low active */io_write(0x22,0x32);io_write(0x23,0x00);/* Clear external switch status */io_write(0x23,0x80);/* Enable external switch active *//* SMI generate */ <-------- /* EXTSW HIGH to LOW */io_write(0x22,0x30);io_write(0x23,0x04);/* De-assert SMI */io_write(0x22,0x35);io_read(0x23);      /* To detect the SMI is caused by which one. */io_write(0x22,0x41);io_read(0x23);      /* To check the external switch status. *//* SMI routine */

io_write(0x22,0x32);io_write(0x23,0x00);/* Clear external switch status */io_write(0x23,0x80);/* Enable external switch active */io_write(0x22,0x34);io_write(0x23,0x80);/* Set wake-up event is keyboard.*/	Note: The IN access includes IRQ12,IRQ4,IRQ3 activity.  They can be selected by index 40 D7D6D5 but the	keyboard is always selected.io_write(0x22,0x30);io_write(0x23,0x84);/* Enable SMI */RSM;                /*End of the external switch */

Example 5:   IN timeout and access usage   IN group set is used to monitor the stardand INput devices activity.   IN group set : 	IRQ1 default for keyboard	IRQ12 optional for PS2 mouse (P40_D5)	IRQ3 optional for COM2 mouse (P40_D7)	IRQ4 optional for COM1 mouse (P40_D6)   IN group set idle timer timeout ==>       (1) power control signal to power OFF the MONITOR device       (2) IN access set to generate SMI   IN group set access ==>       (1) power control signal to power ON the MONITOR device       (2) IN timeout set to generate SMISo, the IN activity monitor can be used as the conventional SCREEN SAVER function and it will not impact the running program performance including  the VGA access instruction.  ==>But, the whole system power can be dramatically reduced because of the monitor power off.

Example 6:   DMA/ISA Master command wait control usageIn the power saving mode, the host clock frequency may be slow down to below 20 MHz(for example) that will make M1429L internal state machine sampling-lost the DMA/ISA MASTER command low pulse or command high pulse.So, the programmer can set P1E_D0 to "1" ==>       the M1429L will assert IOCHRDYJ active to extend the command active(low) time for keeping the DMA/ISA Master normal    
       operation.   And, the programmer also can set P1E_D1 to "1" ==>       the M1429L will extend the M1429L internal command high time for sampling the command signal high event.

Example 7:   I/O TRAP : Access SMI events/* When the peripherial device idle timer timeout and be set to its power saving mode that it needs a period of time to wakeup, the related access monitor bit should be set to "1" for generating the I/O trap SMI event.   */-- ex.  HDD idle time timeout and be set to power saving mode   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x84);/* enable PMU */   io_write(0x22,0x34);/* Index-30H */   io_write(0x23,0x02);/* enable HDD access to generate the SMI signal */   ......   io_write(0x01F2,0xAA55);/* SMI accerts ... HDD IO access */ --  I/O Trap-- SMI routine for wakeup HDD

io_write(0x22,0x35);/* Index-35H */   io_read(0x23);/* read SMI cause... "10010" = HDD access */   io_write(0x22,0x30);/* Index-35H */   io_write(0x23,0x04);/* clear SMI */   io_write(0x22,0x34);/* Index-30H */   io_write(0x23,0x00);/* disable HDD access to generate the SMI signal */   wakeup HDD routine   ......   io_write(0x22,0x33);/* Index-30H */   io_write(0x23,0x02);/* enable HDD idle timeout to generate the SMI signal */   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x84);/* enable PMU, enable SMI */   RSM

Example 8:   Software SMI event   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0xC4);/* enable PMU, bit 6 = "1" */   ......-- SMI routine (SMIACTJ active)   io_write(0x22,0x35);/* Index-35H */   io_read(0x23);/* read SMI cause... "01111" = Software SMI */   io_write(0x22,0x30);/* Index-35H */   io_write(0x23,0x04);/* clear SMI */   Software SMI routine   ......   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x84);/* enable PMU, enable SMI */   RSM

Example 9:   Software STPCLKJ active outside SMI routine   /*   P40_D2 = "0";	If the software STPCLKJ instruction is in the SMI routine, the real STPCLKJ signal will NOT active until the RSM 
	instruction and leaving the SMI routine. If the software STPCLKJ instruction is out the SMI routine, the real 
	STPCLKJ signal will immediately active to respond the software command.     */   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x94);/* enable PMU , software STPCLKJ active */   /*    Stop Clock control asserts, Host stop clock grant.   * /   /*    Waiting for the SMIJ, or optional INTR/NMI/DMA(VESA master) requests  events to de-assert the STPCLKJ.  	The optional INTR/NMI/HOLD(VESA master) requests selection are depended on the P32_D2 and P30_D5 
	setting :     a)  when P32_D2 = "1", then INTR/NMI or any IRQ events will NOT de-assert  STPCLKJ.     b)  when P32_D2 = "0",	(1) if P35_D7 = "1" then IRQ de-assert STPCLKJ are depended on the setting of P36 and P37.            (2) if P35_D7 = "0" then the system INTR or NMI signals active				    events will de-assert STPCLKJ.     c)  when P30_D5 = "1", then HOLD/LREQJ or any DRQ events will NOT de-assert STPCLKJ.     d)  when P30_D5 = "0"             (1) if P35_D6 = "1" then DRQ de-assert STPCLKJ are depended on the setting of P38.            (2) if P35_D6 = "0" then the system DMA or LREQJ requests signals active events will de-assert STPCLKJ.	                                                         */

Example 10:   Software STPCLKJ could active inside SMI routine   /*   P40_D2 = "1";	If the software STPCLKJ instruction is in/out the SMI routine, the real STPCLKJ signal will immediately active to 
	respond the software command.    */--  IN_SMI_ROUTINE ( SMIACTJ_like signal ACTIVE )     ......   label 1 :   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x94);/* enable PMU , software STPCLKJ active */   /*    Stop Clock control asserts, Host stop clock grant.   */	   /*    Waiting for the the new SMIJ events to de-assert the STPCLKJ.  	The optional INTR/NMI/HOLD(VESA master) requests selection should be DISABLE, i.e. ONLY the new SMI 
	event can de-assert STPCLKJ.         ==>   P32_D2 = "1" and  P30_D5 = "1"	                                                         */   /*    the NEW SMIJ event occurs ==>  the STPCLKJ will be de-asserted.	 But, the SMIJ will not assert again.                    */   io_write(0x22,0x35);/* Index-35H */   io_read(0x23);/* read SMI cause */   /* If the SMI is an idle timer timeout or NOT need to wakeup the system event, then the system can execute more power saving 
action and back to label  1 and wait the next SMI wakeup. If the SMI is a wakeup event, then the system enable the necessary power for normal operation.                                                  */   SMI wakeup    ......--  RSMExample 11:   Clock Throttling Feature   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x84);/* enable PMU */   io_write(0x22,0x32);/* Index-32H */   io_write(0x23,0x..);     /* programming the P32_D5-3 to enable clock throttling and setting the request timer */        Action 1 :   /*    Waiting for the SMIJ, or optional INTR/NMI/DMA(VESA master) requests events idle timer timeout.  	The optional INTR/NMI/HOLD(VESA master) requests selection are depended on the P32_D2 and P30_D5 
	setting :     a)  when P32_D2 = "1", then INTR/NMI or any IRQ events will NOT reset timer.     b)  when P32_D2 = "0",	    (1) if P35_D7 = "1" then IRQ reset timer are depended on the				    setting of P36 and P37.	    (2) if P35_D7 = "0" then the system INTR or NMI signals active				    events will reset timer.     c)  when P30_D5 = "1", then HOLD/LREQJ or any DRQ events will NOT reset timer.     d)  when P30_D5 = "0"             (1) if P35_D6 = "1" then DRQ reset timer are depended on the setting of P38.            (2) if P35_D6 = "0" then the system DMA or LREQJ requests signals active events will reset timer.	                                                         */--  Idle timer timeout   /*    Stop Clock control asserts, Host stop clock grant.   */	     Action 2 :   /*    Waiting for the SMIJ, or optional INTR/NMI/DMA(VESA master) requests         events to de-assert the STPCLKJ.  	 The optional INTR/NMI/HOLD(VESA master) requests selection are depended on the P32_D2 and P30_D5 
	setting :     a)  when P32_D2 = "1", then INTR/NMI or any IRQ events will NOT de-assert 	 STPCLKJ.     b)  when P32_D2 = "0",	    (1) if P35_D7 = "1" then IRQ de-assert STPCLKJ are depended on the setting of P36 and P37.    	(2) if P35_D7 = "0" then the system INTR or NMI signals active				    events will de-assert STPCLKJ.     c)  when P30_D5 = "1", then HOLD/LREQJ or any DRQ events will NOT de-assert STPCLKJ.     d)  when P30_D5 = "0"             (1) if P35_D6 = "1" then DRQ de-assert STPCLKJ are depended on the setting of P38.            (2) if P35_D6 = "0" then the system DMA or LREQJ requests signals active events will de-assert STPCLKJ.	                                                         */--  The STPCLKJ will be asserted or de-asserted looping between the Action 1 &  Action 2 if the clock throttling feature be enable.    The SMI events and the SMI routine execution will TEMPORARY DISABLE the clock throttling feature.

Example 12:   CPU CLOCK power-down in STPCLKJ active period feature   io_write(0x22,0x21);/ enable Index-21H */   io_write(0x23,0x84);/* enable stpclk pdown , Pdown CLK2=OSC/16 */   io_write(0x22,0x30);/* Index-30H */   io_write(0x23,0x94);/* enable PMU , software STPCLKJ active */   /*    Stop Clock control asserts,	 Host stop clock grant,	 Pin 7 & Pin 11 Clock output changed to Pdown CLK2 & CLK1	 those are defined in P21 D2-0                            */   /*    Waiting for the SMIJ, or optional INTR/NMI/Hold(VESA master) requests events to de-assert the STPCLKJ.  When  
    deasserting event occurs, the output clock changes to full-on frequency and waits the specified time in the P26 D1-D0 then de-
    asserts the STPCLKJ.           */

Example 13:   Power saving level   M1429L provides 2 types of power saving level: one is the whole system level, and the other is the peripheral level.     1) For the whole system level:        The M1429L provides 2 bits ( P30_D1-0 ) for the programmer to distinguish the system state : ON, DOZE, STANBY, and 
        SUSPEND.        The P31 & P32_D1-0 be defined the monitoring selection of the state events activity.  The P3E be defined the idle timer 
        delay time from the present state to the next state.        The P31_D3 IRQ channel selected be defined in the P36 & P37.        The P31_D7 DRQ channel selected be defined in the P38.        The P32_D0 GP0 which is a memory space be defined in the P45_D1, P46, P47 & P48.  The memory address line will be 
        ignored or set to "don't care" while the related address mask bit be set to "1".        The P32_D1 GP1 which is an I/O space be defined in the P45_D0, P45_D6-2 & P49. The I/O address line will be ignored or 
        set to "don't care" while the related address mask bit be set to "1".  The P45_D3-2, P49 means one group of the GP I/O.  The 
        P45_D4, P45_D5, and P45_D6 meanthe group of I/O space 100h~1FFh, 200h~2FFh, and 300~3FFh respectively.So, the 
       GP I/O has 4 group of I/O space.  Once enabled I/O group event will represents the whole GP I/O event.  The P3D_D3 be 
      defined the GP I/O  address line A15-A10 will be recognized or not.       ON -> DOZE         io_write(0x22,0x30);/* Index-30H */        io_write(0x23,0x84);/* enable PMU, state = "00" */       *load the monitoring event selected and timer time of ON-to-DOZE from the RTC.        io_write(0x22,0x31);/* Index-31H */        ......        io_write(0x22,0x32);/* Index-32H */        ......( bit 1 & 0 )        io_write(0x22,0x3E);/* Index-3EH */        ......   ...... Mode Timeout ==> SMI	read SMI cause and Index P40_D5 Doze timeout ="1"        SMI routine       DOZE -> STANDBY         io_write(0x22,0x30);/* Index-30H */        io_write(0x23,0x85);/* enable PMU, state = "01" */       *load the monitoring event selected and timer time of DOZE-to-STANDBY from the RTC.        io_write(0x22,0x31);/* Index-31H */        ......        io_write(0x22,0x32);/* Index-32H */        ......( bit 1 & 0 )        io_write(0x22,0x3E);/* Index-3EH */        ......

...... Mode Timeout ==> SMI	read SMI cause and Index P40_D6 Standby timeout ="1"        SMI routine	...	...	...              The M1429L provides the P30_D1-0 2 bits for the programmer to recognize the upto 4 system state.   The M1429L has no 
      absolute difference between  the 4 state in the hardware implementation.  The 4 system state differences are the software-
      oriented issues.  The software programmer can implement 2, 3, or 4 system state in any sequence which are dependent on 
      the requirement of the system maker.   2) The peripheral level:       The M1429L provides 6 timers for VGA, HDD, LPT, GP memory, GP I/O, and the standard INPUT devices.  So, the M1429L 
       can individually monitor the  peripheral devices activity and control its power saving effect.

Example 14: Setup Switchio_write(0x22,0x30);io_write(0x23,0x84);/*Enable PMU & Enable SMI */io_write(0x22,0x40);io_write(0x23,0x08);/* Enable setup switch */io_write(0x22,0x32);io_write(0x23,0x00);/* Clear setup switch and Ring-in status */io_write(0x23,0x40);/* Enable setup switch and Ring-in SMIJ active *//* SMI generate */ <-------- /* Setup switch active */io_write(0x22,0x30);io_write(0x23,0x04);/* De-assert SMI */io_write(0x22,0x35);io_read(0x23);      /* To detect the SMI is caused by which one. */io_write(0x22,0x40);io_read(0x23);      /* To check the setup switch status. */io_write(0x22,0x32);io_write(0x23,0x00);/* Clear setup switch and Ring-in status */io_write(0x23,0x40);/* Enable setup switch and Ring-in active *//* SMI routine */io_write(0x22,0x34);io_write(0x23,0x80);/* Set wake-up event is keyboard.*/	Note: The IN access includes IRQ12,IRQ4,IRQ3 activity  They can be selected by index 40 D7D6D5 but the  
	keyboard is always selected.io_write(0x22,0x30);io_write(0x23,0x84);/* Enable SMI */RSM;                /*End of the setup switch */

# Section 6:  Design Issues

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

1

9

20

21

22

23

24

25

26

27

# Section 7:  Electrical Characteristics

### 7.1	Timing Diagrams

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

### 7.2	AC Characteristics

Table 7-1	AC Characteristics

Symbol	Parameter	Minimum	Maximum	Unit	See Figure 	Notes

CLK2I Frequency		80	MHz		2XCPU clock

CLK1O Frequency		40	MHz		CPU clock

CLK2O Frequency		80	MHz		CPU clock

t1	XGA31, XGA[27-2], XBEJ[3-0], XSMIADSJ	11	-	ns	A	CPU timing
	XADSJ, XMIO, XDC, XWR, FERRJ,
	HITMJ, CHLDA, PCD, LOCKJ setup time

t2	XGA31, XGA[27-2], XBEJ[3-0], XSMIADSJ	11	-	ns	A	CPU timing
	XADSJ, XMIO, XDC, XWR, FERRJ,
	HITMJ, CHLDA, PCD, LOCKJ hold time

t3	XPCHKJ setup time	8	-	ns	A	CPU timing

t4	XPCHKJ hold time	2	-	ns	A	CPU timing

t5	XPD[3-0] setup time	20	-	ns	A	CPU timing

t6	XPD[3-0] hold time	5	-	ns	A	CPU timing

t7	XPD[3-0] valid time		20	ns	B	CPU timing

t8	XPD[3-0] float time	-	6	ns	B 	CPU timing

t9	XLDEVJ, XLREQJ setup time	8	-	ns	A	CPU timing

t10	XLDEVJ, XLREQJ setup time	2	-	ns	A	CPU timing

t11	XA20MJ, XINTR, XHOLD, XIGNNEJ	-	7	ns	B	CPU timing
	XSMIJ,XSMIACTJ, XSTPCLKJ, XSUSPA,
	XNMIJ, XBOFFJ, XLGNTJ valid delay

t12	XA20MJ, XINTR, XHOLD, XIGNNEJ	-	7	ns	B	CPU timing
	XSMIJ,XSMIACTJ, XSTPCLKJ, XSUSPA,
	XNMIJ, XBOFFJ, XLGNTJ float delay

t13	XRDYOJ, XSMIRDYJ valid delay	-	9	ns	B 	CPU timing

t14	XRDYOJ, XSMIRDYJ float delay	-	9	ns	B	CPU timing

t15	XBRDYJ valid delay	-	8	ns	B	CPU timing

t16	XBRDYJ float delay	-	8	ns	B	CPU timing

t17	XLNPRDYJ setup time	8	-	ns	A	CPU timing

t18	XLNPRDYJ hold time	5	-	ns	A	CPU timing

t19	XKENOJ valid delay	-	5	ns	B	CPU timing

t20	XKENOJ float delay	-	4	ns	B	CPU timing

t21	XEADSJ valid delay	-	6	ns	B	CPU timing

t22	XEADSJ float delay	-	6	ns	B	CPU timing

t23	XFLUSHJ valid delay	-	6	ns	B	CPU timing

t24	XFLUSHJ float delay	-	7	ns	B	CPU timing

t25	XWB_WTJ valid delay	-	6	ns	B	CPU timing

t26	XWB_WTJ float delay	-	6	ns	B	CPU timing

t27	XCOEJ[1-0] valid delay	-	7	ns	B	CPU timing

t28	XCOEJ[1-0] float delay	-	7	ns	B	CPU timing

t29	XCWEJ[1-0] valid delay	-	8	ns	B	CPU timing

t30	XCWEJ[1-0] float delay	-	8	ns	B	CPU timing

t31	XCSJ[3-0] valid delay	-	4	ns	B	CPU timing

t32	XCSJ[3-0] float delay	-	4	ns	B	CPU timing

t33	XA3IIO, XA3IO valid delay	-	8	ns	B	CPU timing

Table7-1	AC Characteristics  (continued)

Symbol	Parameter	Minimum	Maximum	Unit	See Figure 	Notes

t34	XA3IIO, XA3IO float delay	-	8	ns	B	CPU timing

t35	XTAGWEJ valid delay	-	6	ns	B	CPU timing

t36	XTAGWEJ float delay	-	6	ns	B	CPU timing

t37	XTAGD[7-0] valid delay	-	8	ns	B	CPU timing

t38	XTAGD[7-0] float delay	-	8	ns	B	CPU timing

t39	XRASJ[3-0] valid delay	-	7	ns	B	CPU timing

t40	XRASJ[3-0] float delay	-	7	ns	B	CPU timing

t41	XCASJ[3-0] valid delay	-	6	ns	B	CPU timing

t42	XCASJ[3-0] float delay	-	6	ns	B	CPU timing

t43	XMWEJ valid delay	-	6	ns	B	CPU timing

t44	XMWEJ float delay	-	6	ns	B	CPU timing

t45	XMA[11-0] valid delay	-	7	ns	B	CPU timing

t46	XMA[11-0] float delay	-	6	ns	B	CPU timing

t47	XMWJ, XMRJ valid delay	-	15	ns	B	AT timing

t48	XMWJ, XMRJ float delay	-	20	ns	B	AT timing

t49	XIOWJ, XIORJ, SMEMRJ,	-	20	ns	B	AT timing
	XMEMWJ valid delay

t50	XIOWJ, XIORJ, SMEMRJ,	-	20	ns	B	AT timing
	XMEMWJ float delay

t51	XIO16J, XMEM16J, XIOCHRDYJ,	10	-	ns	A	AT timing
	XMASTERJ, XREFRESHJ, XIOCHKJ
	setup time

t52	XIO16J, XMEM16J, XIOCHRDYJ,	6	-	ns	A	AT timing
	XMASTERJ, XREFRESHJ, XIOCHKJ
	hold time

t53	XIOCHRDYJ, XREFRESHJ, valid delay	-	15	ns	B	AT timing

t54	XIOCHRDYJ, XREFRESHJ, float delay	-	15	ns	B	AT timing

t55	XBALE, XSBHEJ, XAEN valid delay	-	20	ns	B	AT timing

t56	XBALE, XSBHEJ, XAEN float delay	-	20	ns	B	AT timing

t57	XBUSCSJ valid delay	-	15	ns	B	AT timing

t58	XBUSCSJ float delay	-	15	ns	B	AT timing

t59	XRTCAS, XADIR, XDIR valid delay	-	15	ns	B	AT timing

t60	XRTCAS, XADIR, XDIR float delay	-	15	ns	B	AT timing

### Figure A

### Figure B

## 7.3	DC Characteristics

Table 7-2	DC Characteristics

| Symbol | Parameter | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| V-IN-Hi | Input low voltage | -0.3 | +0.8 | V |
| V-IN-Hi | Input High voltage | 2.0 | Vcc+0.3 | V |
| V-OUT-Low | Output low voltage |  | 0.45 | V |
| V-OUT-Hi | Output high voltage | 2.4 |  | V |
| ICC | Power supply current |  | 200 | mA1 |
| IIi | Input leakage current |  | +-15 | uA2 |
| Iih | Input leakage current |  | 200 | uA3 |
| IiI | Input leakage current |  | -200 | uA4 |
| IoO | Output leakage current |  | +-15 | uA |
| Cin | Input capacitance |  | 20 | pF5 |
| Co | I/O or output capacitance |  | 20 | pF5 |
| Cclk | CLK capacitance |  | 20 | pF5 |

Table 7-3	Absolute Maximum Ratings

| Symbol | Parameter | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Vcc | Supply voltage | 4.5 | 5.5 | V6 |
| V-IN | Input voltage | -0.5 | 5.5 | V |
| V-OUT | Output voltage | -0.5 | 5.5 | V |
| Top | Operating temperature | 0 | 70 | C |
| Testing | Storage temperature | -40 | 125 | C |

# Section 8:	Packaging Information

1 Typical supply current at 80-MHz

2 For input without pull-ups or pull-downs, and if 0 =< V-IN =< Vcc

3 For inputs with pull-downs and V-IN-Hi = 2.4V

4 For inputs with pull-ups and V-IN-Low = 0.45V

5  FC = 1-MHz and not 100% test

6  This chip will not be damaged if the voltage is under the absolute minimum rating, but timing function is not guaranteed.