OPTi82C895 Power Management Registers

I/O Ports: 22h Index, 24h Data

[Reg E0h]

bit 7   *SMI (1)/AUTO (0) freen mode. SMI is for SL CPUs.

bit 6   *Use PPEN# for green mode

bit 5   *Use SMI# for green mode

bit 4   *Use IRQ15 for SMI green (set to 0)

bit 3   *<R/O> System is in normal mode (green if 0)

bit 2   *Write: Enable timer timeout to trigger green mode

Read: check if timer caused green mode

bit 1   *Same for EPMI# instead of timer

bit 0   *Same for software forced Green mode

Set bits 7..5 as follows:

Auto green: 010

82C601 SMI: 111

82C602 SMI: 101

No green:   000

[Reg E1h] {off:00}

bit 7   *Wake up on any access to interrupt vectors

bit 6   *On EPMI#, return to NORMAL if GREEN and reload timer if NORMAL (?)

It seems to mean just "Enable SLEEP button function"

bit 5   *EPMI# pulse width. 0: 100ns, 1: 5ms (dwitch debouncing)

bit 4   *EPMI# pulse level. 0: Active low, 1: high

bit 3   *Force green mode (if enabled, see above)

bit 2..0 System Timeout: 15"/2'/5'/15'/30'/45'/1h/4h

[Reg E2h]

bit 7..0 IRQ7..IRQ0 Monitor (*IRQ2 and IRQ0 ignored)

[Reg E3h]

bit 7..0 IRQ15..IRQ8 Monitor

[Reg E4h]

bit 7..0 DRQ7..DRQ0 Monitor

[Reg E5h] {off:00}

bit 7   *Wake up on port access (see reg. E6h/E7h)

bit 6    Video RAM Monitor (A000..BFFF)

bit 5    Video Ports Monitor (3B0..3DF)

bit 4    HDD Ports Monitor (1F0..1F7, 3F6)

bit 3    FDD Ports Monitor (3F5)

bit 2    Keyboard Ports Monitor (60, 64)

bit 1    Local Bus Device Monitor (Wake up on LDEV#)

bit 0   *Wake up on LREQ# if EEh.7 is set

[Reg E6h]

bit 7..0*I/O port address A0..A7 (see E5h.7)

[Reg E7h]

bit 7   *0

bit 6..4*Number of lowest bits to mask in port range

bit 3,2 *00

bit 1,0 *I/O port address A8,A9

[Reg E8h,E9h] (2 registers)

bit 7..0*Interrupt # to mask out even if E1h.7 is set

By default, both are set to 8

[Reg EAh] {off:00/on:F8}

82C602 power management values for GREEN mode:

bit 7..3*00000

bit 2   *HSYNC (0)

bit 1   *VSYNC (0)

bit 0   *CLKCNT (0)

[Reg EBh] {FF}

82C602 power management values for NORMAL mode:

bit 7..3*11111

bit 2   *HSYNC (1)

bit 1   *VSYNC (1)

bit 0   *CLKCNT (1)

[Reg ECh] Scratch register 1 {00}

[Reg EDh] Scratch register 2

bit 7    Screen Sleep (maybe bit 6?)

bit 6..0 ? {1010000}

[Reg EEh]

bit 7   *Read: a wakeup has occurred (cleared when read once)

*Write: enable local bus master to generate PPEN#

bit 6   *Write: enable DRQ to generate PPEN#

bit 5   *0 (debug)

bit 4   *Enable nested green events

bit 3..1*000 (debug)

bit 0   *Gerenate SMI# on wakeup

[Reg EFh] {off:C0/on:40}

bit 7..4*0100

bit 3,2 *00: 82C602 mode, 11L 82C601 mode

bit 1,0:*01 (debug)