OPTi82C895 General Purpose Registers

I/O Ports: 22h Index, 24h Data

[Reg 20h]

bit 7,6  <R/O> Revision =00

bit 5    Cache Read Cycle X-2-2-2 if 1, X-1-1-1 if 0

bit 4   *TAG7 bit sampling

bit 3    Single ALE

bit 2    AT Cycle W/S (0 or 1)

bit 1   *Keyboard Reset Control (0: wait for HLT)

bit 0   *Fast Reset: reset on any HLT

[Reg 21h]

bit 7    Master Mode Byte Swap

bit 6   *0 (debug)

bit 5    DRAM Parity Check Disable

bit 4    Ext. Cache Enable

bit 3,2  Cache Size (00=64K 01=128K 10=256K 11=512K)

bit 1    Cache Write 0 W/S (1 W/S if 0, see next line)

bit 0    Cache Read Cycle 2-X-X-X if 1, 3-X-X-X if 0

[Reg 22h]

bit 7    F000:64 R/O Shadow if 0, ROM to DRAM Copying if 1

bit 6,5 *11 (debug)

bit 4    D000:64 R/O Mode

bit 3    E000:64 R/O Mode

bit 2    Hidden Refresh Disable

bit 1   *Fast A20 (A20 always enabled if 1)

bit 0    Slow Refresh

[Reg 23h]

bit 7    EC00:16 Shadow

bit 6    E800:16 Shadow

bit 5    E400:16 Shadow

bit 4    E000:16 Shadow

bit 3    DC00:16 Shadow

bit 2    D800:16 Shadow

bit 1    D400:16 Shadow

bit 0    D000:16 Shadow

[Reg 24h]

bit 7   *Remap 3xxxx to Bxxxx for SMI handler upload

bit 6..4*DRAM configuration

bit 4   *0 (debug)

bit 2..0*DRAN configuration

[Reg 25h]

bit 7   *0: pin 62=MDIR#, 1: pin 62=LMEM#

bit 6,5  DRAM Burst Mode (00=3222 01=4333*(+1WS on page miss)

10=4333 11=5444)

bit 4    DRAM Write W/S (3222/4333)

bit 3,2 *11 (debug)

bit 1,0  ATCLK (00=CLKI/6 01=CLKI/5 10=CLKI/4 11=CLKI/3)

[Reg 26h]

bit 7   *Generate ROMCS# for write cycles (flash ROM)

bit 6    C000-EFFF: 0=R/W AT Bus, 1=Read AT Bus, Write Shadow RAM

bit 5    C000:64 R/O Mode

bit 4   *1

bit 3    CC00:16 Shadow

bit 2    C800:16 Shadow

bit 1    C400:16 Shadow

bit 0    C000:16 Shadow

[Reg 27h]

bit 7   *Global cache disable

bit 6    Fast AT Cycle

bit 5    AT Cycle Between I/O Cycles (0=3, 1=0)

bit 4   *F000:64 _L2 CACHE_ (NOT DRAM!!!) R/O Mode

bit 3    Turbo mode (Ctrl-Alt-[+])

bit 2   *CPU write-back mode

bit 1    ATCLK Mode (0=14.318/2, 1=Synchro)

bit 0   *0

[Reg 28h]

Non-Cacheable Block 1 (part 1)

bit 7..5*Size (000-64K, 001-128K, 010-256K, 011-1M, 1xx-Disabled)

bit 4..3*11

bit 2..0 A26..A24 of block base

[Reg 29h]

Non-Cacheable Block 1 (part 2)

bit 7..0 A23..A16 of block base

[Reg 2Ah,2Bh]

Non-Cacheable Block 2 (same as [28h,29h])

[Reg 2Ch]

R/O, {2C/3C} *This register does not exist in 82C895

[Reg 2Dh]

Related to shadowable segments

bit 7   *if 0: IRQ12 latch cleared on reads from port 60h

bit 6   *if 0: IRQ1 --""--

bit 5..0*Enable ROMCS# on reads from E800:32..C000:32

[Reg 2Eh]

bit 7    F000:64 Cacheable L1

bit 6    E000:64 Cacheable L1

bit 5    D000:64 Cacheable L1

bit 4    C000:64 Cacheable L1

bit 3    EC00:64 Cacheable L2

bit 2    E800:64 Cacheable L2

bit 1    E400:64 Cacheable L2

bit 0    E000:64 Cacheable L2

[Reg 2Fh]

bit 7    DC00:64 Cacheable L2

bit 6    D800:64 Cacheable L2

bit 5    D400:64 Cacheable L2

bit 4    D000:64 Cacheable L2

bit 3    CC00:64 Cacheable L2

bit 2    C800:64 Cacheable L2

bit 1    C400:64 Cacheable L2

bit 0    C000:64 Cacheable L2