OPTi82C895 Internal Peripherals Controller Configuration Register

I/O Ports: 22h Index, 23h Data

[Reg 01h]

bit 7,6 *Number of W/S on access to IPC (DMA, timer, etc.) minus 1

Default=11 (4 W/S)

bit 5,4 *Number of DMA16 W/S minus 1 (Default=00)

bit 3,2 *Number of DMA8 W/S minus 1 (Default=00)

bit 1   *Disable 1 W/S between SMEMR# and DMAMEMR# (set to 0)

bit 0   *DMACLK=SYSCLK/2 if 0, SYSCLK if 1 (set to 0)

Note: This register is set to 03 (fastest) on my system and nothing crashes.