TITLE : 			ADDITIONAL PRODUCT FEATURES FOR 						8XC251SA/SB/SP/SQ B-STEPPING

PRODUCT COVERED : 	8XC251SA/SB/SP/SQ B-STEPPING

DATE/VERSION : 		SEPTEMBER/25/95 VER. 0.3

RELATED INFO : 		8XC251SB A-1 STEPPING PRODUCT PREVIEW 					(272616-001) AND ADVANCED INFORMATION 					(272616-002)  DATASHEETS, 8XC251SB USER’S 					MANUAL (272617-001)

Keywords : 			8XC251SB, 8XC251SA/SB/SPSQ, B-STEPPING, FEATURES

ADDITIONAL PRODUCT FEATURES FOR 8XC251SA/SB/SP/SQ B-STEPPING

1.0 INTRODUCTION

This document contents of Intel Confidential information. The 8XC251SA/SB/SP/SQ B-stepping products are the next stepping devices for 8XC251SB A-1 stepping. The additional product features for 8XC251SA/SB/SP/SQ B-stepping products are listed below:

1) 8K-Byte or 16K-Byte of internal OTPROM or MASK ROM

2) 512 Bytes or 1 K-Byte of internal Data RAM

3) External Bus Timing Improvement

4) Configuration Bytes Enhancement

5) Extended Memory Addressing

6) 40lds PDIP Package and 44lds PLCC Package

This document describe the additional product features information for 8XC251SA/SB/SP/SQ B-stepping products except for the External Bus Timing information.  The External Bus Timing information can be obtained from a separate document. The title for that document is “Changes For 8XC251SB AC Timing Spec From A-1 Stepping To B-stepping” and it is available at the Technical Document section of Lotus Notes Database.

2.0 MEMORY SIZES

There are a few memory sizes products availble for the 8XC251SA/SB/SP/SQ which are listed in the Table 1.0.

Table 1.0 Memory sizes for the 8XC251SA/SB/SP/SQ products

|  | 512 Bytes internal Data RAM | 1 KBytes internal Data RAM |
| --- | --- | --- |
| 8K internal OTPROM or MASK ROM | 8XC251SP | 8XC251SA |
| 16K internal OTPROM or MASK ROM | 8XC251SQ | 8XC251SB |

3.0 CONFIGURATION BYTES FOR 8XC251SB A-1 STEPPING

There are two configuration bytes available for the first 8XC251SB product (A-1 stepping). These two configuration bytes are located at nonvolatile memory outside of the memory address space and are inaccessible by the user code. The configuration bytes mentioned above are Config0 (location 80H) and Config1 (location 81H). These two configuration bytes are arranged in the format as described in Table 2.0. As for 8XC251SA/SB/SP/SQ products, these two configuration bytes are not available to the users, they have been replaced by two new configuration bytes as described in the Section 4.0.

Table 2.0 Bits assignment for A-stepping  Configuration Bytes

Config0 (located at location 80H)

| Symbol | - | - | WSA | XALE | RD1 | RD0 | PAGE | SRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Config1 (located at location 81H)

| Symbol | - | - | - | INTR | WSB | - | - | EMAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Notes: Individual bit definition are explained in the 8XC251SB Datasheet

4.0 CONFIGURATION BYTES ENHANCEMENT (FOR B-STEPPING)

For the 8XC251SA/SB/SP/SQ products (B-stepping), the configuration bytes are located at different locations depending on where the user code is located. The user code can be divided into:-

1)Internal code only

2)Internal and external codes

3)External code only

1)INTERNAL CODE ONLY

For the user code which is located at internal code memory only, the configuration bytes will be programmed at location  FF:FFF8  and  FF:FFF9. The upper 8 bytes of the internal code memory (either 8K-Byte or 16K-Byte)  available to the user are not  to be used.  The reason is  the prefetch  due to the pipeline. If there is any code at these locations the processor will be prefetching information off chip thus corrupting any eventual I/O information on P0 and P2.  Note that   this restriction  due the prefetch does not have anything to do with the configuration bytes.

2)INTERNAL CODE WITH AUTOMATIC CODE ROLLOVER TO EXTERNAL MEMORY:

For the user code which is located at both internal and external memories, the configuration bytes will be programmed at location  FF:FFF8  and  FF:FFF9. In this case the upper 8 bytes of the internal code memory (8k or 16k) available to the user should be used in order to allow the automatic code rollover to external memory to be done smoothly.

When the code rolls over to external memory at some level (if the code is long) it will reach location  FFF8  and  FFF9.  This will conflict with the configuration bytes which are programmed internally. So, user are not allow to use the last eight bytes of the external EPROM.

3)EXTERNAL CODE ONLY:

In this case the configuration bytes will be programmed at the upper bytes of the external EPROM. For example,  they will be physically programmed at  location 3FF8 and 3FF9 of  a 16K  EPROM.

Table 3.0 shows the configuration bytes location at internal and external memory.

Table 3.0 Configuration Bytes location for Internal and External Memory

| user code lcoated at | address for UConfig0 | address for UConfig1 |
| --- | --- | --- |
| 8K internal OTPROM or MASK ROM | FF:FFF8H | FF:FFF9H |
| 16K internal OTPROM or MASK ROM | FF:FFF8H | FF:FFF9H |
| 16K external EPROM | FF:3FF8H | FF:3FF9H |
| 32K external EPROM | FF:7FF8H | FF:7FF9H |
| 64K, 128K or 256K external EPROM | FF:FFF8H | FF:FFF9H |

For the new configuration bytes,  the bits definition are arranged in the format shown in Table 4.0.

Table 4.0 Bits assignment for B-stepping Configuration Bytes

UConfig0 (either located at internal code memory address space or at external user code memory address space)

| Symbol | - | WSA1 | WSA0 | XALE | RD1 | RD0 | PAGE | SRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note: WSA0 is the same as WSA in the A-1 stepping

WSA1	WSA0	Wait State for PSEN#/RD#/WR# signal

1	1	0 wait state for all regions except region 01:

1	0	Extended to 1 wait state for all regions except region 01:

0	1	Extended to 2 wait state for all regions except region 01:

0	0	Extended to 3 wait state for all regions except region 01:

RD# and PSEN# Function Select

RD1	RD0	RD# Range	PSEN# Range	P1.7 Function	Features

1      	1	<=7F:FFFFH	>=80:0000H	P1.7/CEX4	Compatible with MCS 51

1      	0	P3.7 only	All address	P1.7/CEX4	One additional port pin

0      	1	RD#=A16	All address	P1.7/CEX4	128K External Address space

0      	0	RD#=A16	All address	A17		256K External Address space

UConfig1 (either located at location internal user code memory address space or at external user code memory address space)

| Symbol | - | - | - | INTR | WSB | WSB1 | WSB0 | EMAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Note: WSB is the same function as WSB1, it is use for backward compatible with A-1 stepping only. For the new application, it is recommended to use WSB1 and WSB0 together for new feature.

WSB1	WSB0	Wait State for PSEN#/RD#/WR# signal

1	1	0 wait state for region 01:

1	0	Extended to 1 wait state for region 01:

0	1	Extended to 2 wait state for region 01:

0	0	Extended to 3 wait state for region 01:

WSB	1	0 wait state for region 01:

0	1 wait state for region 01:

Note:  the bits assignment that are not explained here are the same as described in the 8XC251SB Datasheet

5.0 EXTENDED MEMORY ADDRESSING

The 8XC251SA/SB/SP/SQ B-stepping products are available with 256K-Byte addressing space. There are 18-bit of address line available for the users.  The address pins A16 and A17 are multiplexed with P3.7/READ# and P1.7/CEX4 respectively. To use this feature, users are required to set the configuration bits RD1 and RD0 of the configuration byte UConfig0 (shown in Table 4.0).

When the 8XC251SA/SB/SP/SQ products are configured for the 256K-Byte address space mode, the total external memory space available to the users is 256K-Byte. Each regions consist of 64K-Byte and users are allow to choose any  four of the eight memory regions shown in the Figure 1.0. But at any one time, any two  of the four regions are not allow to have the same A16 and A17 setting. This is due to the address pins A16 and A17 are two of the address pins to determine which external memory regions to be decoded. For an example, if user choose to have regions FF:, FE:, 01: and 00: for 256K-Byte external memory addressing, he is not allow to have the regions FD:, FC:, 03: and 02:  at the same time.

Figure 1.0 Extended Memory Space for 8XC251SA/SB/SP/SQ products

| FF:0000H-FF:FFFFH | 64K-Byte for code or data memory | A17=1, A16=1 |
| --- | --- | --- |
| FE:0000H-FE:FFFFH | 64K-Byte for code or data memory | A17=1, A16=0 |
| FD:0000H-FD:FFFFH | 64K-Byte for code or data memory | A17=0, A16=1 |
| FC:0000H-FC:FFFFH | 64K-Byte for code or data memory | A17=0, A16=0 |
| RESERVED |  |  |
| 03:0000H-03:FFFFH | 64K-Byte for code or data memory | A17=1, A16=1 |
| 02:0000H-02:FFFFH | 64K-Byte for code or data memory | A17=1, A16=0 |
| 01:0000H-01:FFFFH | 64K-Byte for code or data memory | A17=0, A16=1 |
| 00:0000H-00:FFFFH | 64K-Byte for code or data memory | A17=0, A16=0 |

6.0 PDIP PACKAGE

The pin assignment for 40 lds PDIP package is shown in the following diagram:

7.0 PLCC PACKAGE

The pin assignment for 44 lds PLCC package is shown in the following diagram: