EXAMPLE

A set of example files have been included which show how this disassembler

might be used to incrementally disassemble some unknown code.

SOURCE

The file 'example.asm' is some source asm code which has been taken from one    of the 8051 FTP sites. It has been assembled using a suitable assembler

(not CAS or AS31 though) into an intelhex format in file 'example.hex'.

1ST PASS CONTROL FILE

The file 'example1.ctl' contains a few comments and 6 'En' (Entry Point)

directives only. These correspond to the reset vector location of 0000,

and the 5 other standard interrupt vectors. We can usually make a good

guess at which of these vectors are used by viewing the code file.

1ST PASS DISASSEMBLY

Using the command 'gd8051 example.hex example1.ctl example1.asm -i' (-i to

indicate that the code file is in intelhex format), we get the first pass

disassembled listing file 'example1.asm'.

At this point we use our common sense and understanding of 8051 code to

start to determine directives. The file 'example1.ed' is an edited version

of 'example1.asm' and has had directives added as notes to the source

code. My personal perference (and this is purely arbitary) is to put each

directive on a line of its own preceded by ':::' which serves as a

marker, and allows the additional edits (i.e. the directives) to be

'grepped' for. As can be seen a variety of directives have been determined,

including 'Hd' (Headers) to document subroutine entry points, 'Cc'

(Comments) to be added to the end of a line, 'Az' (Ascii Zero terminated

string) which is very obvious at the end of the file, amongst others.

2ND PASS CONTROL FILE

The user determined directives are extracted (I used grep followed by vi)

from the edited first pass disassembly file and added (merged) with the

previous version of the control file to give the second generation

control file 'example2.ctl'.

2ND PASS DISASSEMBLY

Using the command 'gd8051 example.hex example2.ctl example2.asm -i', we get

the second pass disassembly in the file 'example2.asm'. Examination of

this file shows that we are one step closer to the original source file

(which is normally assumed to be totally unknown!).

If we were to use the command 'gd8051 example.hex example2.ctl example2.asm

-i -f0 -a', we would instead obtain an asm file which could then be edited,

perhaps to add a new feature into the code, or perhaps to fix a bug and

which can then be assembled (using AS31 in this case) back into a binary

or intelhex file. This then allows a new EPROM to be blown to replace the

original and make the new feature permanent.