NAME

gd8051 -- Gary's Disassembler for the 8051 family of microprocessors.

VERSION

Beta1.00 - beta release version.

V2.02    - authors internal version number.

COPYRIGHT

gd8051 is Copyright (C) 1994 Gary Morton

SYNOPSYS

gd8051 code_file control_file output_asm_file options

DESCRIPTION

Gd8051 disassembles 8051 binary files into human readable assembler

source code. Gd8051 intelligently disassembles the code (by use of the

'control_file', leaving data structures and embedded text strings intact,

and is capable of writing the 'output_asm_file' in a useful native

format, or in a re-assemblerable format for either the AS31 or CAS

public domain 8051 assemblers.

Gd8051 currently supports pure binary or intelhex format files for the

'code_file'. The control file both directs the disassembly and contains

user defined comments which are used to annotate the disassembled text.

This allows code to be "incrementally" disassembled, by adding more

directives to the control file, as the code is better understood, without

losing any of the original disassembly information.

Gd8051 is written in C code and should be portable to a wide variety of

platforms, including PC, Workstation, MAC, and any other similar platform.

This beta release contains binaries for both PC (or compatible) and Sun4

architecture machines.

Gd8051 has been improved (before release) to run as efficiently as possible,

and is hoped will result in fast execution times even on the slowest of

PC's.

No artificial restrictions either in speed or funtionality have been

imposed on the beta release binaries, excepting those which result from

poor programming practices and the PC's archaic architecture (and 640k

limit of DOS).

USAGE INFORMATION

Gd8051 executed with no parameters at all displays basic usage information,

as follows:-

---------------------------

Usage: GD8051 [-h|?]/['codefile' 'controlfile' 'output_asm_file' [-c][-a][-f0..2][-b][-i] ]

Release Beta1.00 is Copyright (C) 1994 Gary Morton

Compiled for PC architecture using Turbo C++ V1.01 (Huge Model)

: -c  generate CAS  compatible code.

: -a  generate AS31 compatible code.

: -f0 generate re-assemblerable code.

: -f1 generate standard code output format (append address to each line).

: -f2 generate standard code output format (address + data bytes).

: -b  input code file is in binary format.

: -i  input code file is in intelhex format.

: V2.02 modified by Gary Morton (18-Apr-94)

---------------------------

Note: I thought it useful to clearly indicate the target architecture,

compiler used, release version, and author/internal version. If

feeding back bug reports to the author please clearly indicate

this information.

FILES

Gd8051 processes two input files, and generates a single output file.

The names of all 3 files MUST be provided on the command line as the

first three parameters AND in the expected order (except if only the

-h or -? parameter is given). Presently no verification is made to ensure

that the two input files exist (other than the failure of the "fopen()"

function). The name given for the output asm file will be created if it

doesn't exist already, or will overwrite the contents of any identically

named file which might already exist in the current directory.

'code_file' - this file is expected to contain 8051 microprocessor (or

similiar family part) executable code, stored in either a binary or

intelhex format. The type is defined by either the '-b' or '-i' command

line option.

'control_file' - this file contains disassembly directives, which control

the format of the disassembly. See the documentation file 'ctl.doc' for

full explainations of the supported directives.

'output_asm_file' - the resultant disassembled code is written to this

file in ascii format.

OPTIONS

Any options must be supplied following the 3 filenames only.

Options are introduced by the dash ('-') character.

-h  Print extended help, with brief listing and explaination of the

allowable control file directives. It is suggested that the output

of this option is piped through 'more' as more than 24 lines of

text are printed.

Overrides all other parameters, and can be used without specification

of the first three file parameters.

-?  Same as -h. On a Un*x system, the '?' may need escaping to avoid

interpretation by the shell.

If no '-f' parameter is supplied '-f1' is the default.

-f2 Affects the "address and data" field of the asm output file.

A four digit hexadecimal address field terminated by a ':',

followed by upto three 2 digit hexadecimal numbers seperated by

spaces is printed before each disassembled opcode. As a result labels

start in column 15, and opcodes in column 18. The two digit

hexadecimal numbers represent the opcode and parameters fo the

instruction (where applicable).

Note that in the beta program, none opcode fields are improperly

handled (although this should not be much of a problem).

-f1 Affects the "address and data" field of the asm output file.

A four digit hexadecimal address field terminated by a ':' is

printed before each disassembled opcode. As a result labels

start in column 6, and opcodes in column 9 (seperated from the

address field by three spaces).

This format allows any specific address to be located quickly

with the asm file loaded into an editor.

-f0 Affects the "address and data" field of the asm output file.

No address or data field is printed. Labels start in column 1,

and opcodes start in column 4.

If neither '-a' or '-c' is supplied as a parameter, the output format

defaults to a native ("rich"?) format.

-a Generate AS31 style output format.

The asm file is written in an AS31 assembler compatible format,

and if used with the '-f0' directive can be used as source to the

AS31 assembler directly.

Information from the AS31 documentation file:-

as31 - An 8031/8051 assembler written by Ken  Stauffer

(stauffer@cpsc.ucalgary.ca).

as31 is a full featured assembler but it does lack facilities

for linking several modules together. As31 can be configured

to produced a wide variety of object output formats.

One of the supported output formats is S-records.

Standard assembler syntax is accepted.

This [as31] code is public domain.

-c Generate CAS style output format.

The asm file is written in an CAS assembler compatible format,

and if used with the '-f0' directive can be used as source to the

CAS assembler directly.

Information from the CAS documentation file:-

CAS: The Free Full-Featured 8051 Assembler

Located at the FTP site csd4.csd.uwm.edu in /pub/8051/assem.

This is a 1-pass assembler for the MCS-51 family of

microprocessors with the following features:

(1) Seperately assembleable files.

There are two stages of assembly:

(a) Pass 1: Creation of object files

(b) Pass 1 1/3: Linking of object files

(2) Relative addressing

(3) Unnamed segments

(4) Conditional assembly

With a C-like syntax, e.g.

if (Condition) {

Assembly instructions...

} else {

Assembly instructions...

}

(5) Multiple statements per line with C-like syntax.

(6) C-like expression syntax.

(7) Command-line options similar to those of the UNIX C compiler.

Compatible with UNIX's make.

(8) An extensive archive of real-life assembly language programs,

including a multi-tasking library.

This is probably the most elaborate 8051 assembler you will find

anywhere in the public domain. It's yours with the source for free.

If neither '-b' or '-i' is supplied as a parameter, the output format

defaults to '-b' (binary).

-i The code file is in intelhex format.

-b The code file is in binary format.

LEGAL RESTRICTIONS

Local laws or copyright restrictions on the source binaries to be

disassembled may prevent use of this or other disassemblers to reverse

engineer certain code.

It is expected that users use the program responsability, respecting local

laws, and seeking legal advice where unsure of the legality of the use of

this program.

LIMITATIONS

The imposition of arbitary program limitations has been avoided as much

as possible, but unfortunately some limitations do exist in this

program. It is hoped that they will be removed or their impact lessened

in the full release. If program restrictions get in the way of the use

of this program, the author would be keen to understand which

limitations are most in need of improving (see FEEDBACK).

Any lines of text read from the control file, will be truncated to the

first 512 characters. Care must be taken where the '\' continuation

character is used in 'CC' or 'CB' directives to generate multi-line

comments in the asm output file.

A 'CC' and 'CB' directive cannot be used with the same address.

This program restricts the user to 1024 'NW' (Named Word) directives.

An unavoidable limitation is imposed by the archaic PC architecture and the

640k limit imposed by DOS. The data contained in the control file is read

into memory where it is stored in data structures which are generated using

'malloc()'. If the data in the control file is too large to be stored in

the PC's memory, the usual "out of memory" error message will be generated.

The source code can be compiled with DJGPP port of the 'gcc' compiler and

gets around memory limitations through use of the 'go32' DOS extender.

Unfortunately I am unsure of the restrictions of distributing gcc compiled

code with the go32 extender, so the gcc compiled PC binaries are not

included.

TESTING

Gd8051 has being sucessfully used to disassemble a 64k binary file

(the operating system from a satellite receiver) into AS31 compatible

code using a 135k control file. The resultant asm file (995k in length)

was subsequently assembled by AS31 into a binary obj file, which matched

the original 64k binary file. This code is believed to contain a large

cross section of the 8051 instruction set. [ This was done using the

DJGPP gnu C++ compiled version of the binaries and run on a 386 PC ]

Gd8051 has also been successfully used with other smaller pieces of 8051

code, which again re-assemble successfully to match the original binary

code file.

PUBLIC DOMAIN

The binaries of the beta release of Gd8051 and the associated documentation

files are released into the public domain, providing the following

conditions of use are followed.

The author allows Gd8051 to be copied and freely given to friends or

colleagues, provided all parts of the package are kept together.

The author retains all copyright on this work.

CONDITIONS OF USE

Gd8051 may be used for personal use, by private individuals only.

Gd8051 may not be used in any way by any company, or be associated in

any way with any company, or be supplied by any company with its own

products.

Gd8051 may not be given away or sold by any companies who supply shareware.

Gd8051 must not be uploaded to any BBS, FTP or FSP site without the

authors permission.

[ Sorry for the above list of restictions - I sort of got carried away.

Basically I want to limit the spread of the beta version, as it is

hoped to replace it with a proper release within a couple of months of

this release. ]

AUTHOR

Gary Morton of Bristol England. Pinball enthusiast and

York City FC supporter.

FEEDBACK

In order to improve this software, and especially rid it of any bugs, I

am keen to receive feedback (contructive preferably) on how it might

be improved. Until a program is used in anger by people other than the

author some bugs remain hidden for some time.

If you discover a bug, please email the author with details, including a

description of the bug, where possible an extract from the disassembly

listing, any associated directives from the control file, and version and

architecture information (gd8051 -h).

Internet email address (as of 13-Apr-94): gary@inmos.co.uk

DISCLAIMER

This software is provided AS IS without warrently of any kind, either

expressed or implied, including but not limited to the implied warrenties

of fitness for a particular purpose. The author assumes NO liability

for any alleged or actual damages arising from the use (or misuse) of

this program. [Local laws or by-laws may exclude some of the above

disclaimers]

FUTURE CHANGES

It is hoped to support Motorola S format input files. If you would like to

see any other file formats supported, please contact the author (see

FEEDBACK).

The restriction on the number of 'NW' directives will be removed.

Better "saneness" checking will be made of the control file directives.

Explicit support for 8051 family variants, especially Signetics or

Philips parts which support extended SFRs (Special Function Registers).

Remove the artificial restriction on not allowing a 'CC' and 'CB'

directive to be associated with the same address.

Quiet output option.

Allow a user definable warning count, rather than the rather unfriendly

bail out at the first problem implementation.

Support other 8051 assembler asm format.

If you would like to see any other changes, please contact the author

(see FEEDBACK).