README.DOC

8051 V1.07 C-Compiler & Library Update

======================================

This package contains an updated version of the 8051 C-Compiler package.

Important additions/changes versus the current manual (IPM-00043B/Mars 1986):

1. MEMORY MODELS

================

The compiler and library now supports two "expanded" memory

models called "large" and "medium".  The "medium" memory model

is equivalent to the one called "expanded" in the current manual.

The "large" memory model is now default (-m0 command line option).

In the "medium" model /PSEN and /RD are supposed to be ANDed to-

gether to form a system with a uniform 64K address space.

----------------------------------------------------------------------------

I    The new memory model called "large" makes it possible to generate     I

I    code for a system with 64K of code and 64K of data (variable space).  I

I    That is: /PSEN and /RD ANDing is not required in this mode.           I

----------------------------------------------------------------------------

Code density (i.e. size of generated code) is the same for both

memory models but variables will in the "large" memory model also

occupy some space from the code section according to the following:

Uninitialized variables will only be allocated in [external]

DATA memory.

Initialized variables will use the same amount of space in both the

CODE and [external] DATA memory.  I.e. the initializer

constants are stored in CODE memory and copied into the

DATA memory at start-up (completely transparent to the user).

Examples (applies to all non-auto variables):

int i = 10;		   /* 2 bytes CODE and 2 bytes DATA */

char *p = "ABC"; 	   /* 2+4 bytes in both CODE and DATA

2 bytes for *p, 4 for "ABC"  */

int j;                    /* 2 bytes in DATA memory only */

char terror_buf[10000];   /* 10000 bytes in DATA area */

Note that static and extern variables without explicit initializers

are automatically set to zero at start-up according to the C standard.

Also note that code is always PROMable in the "large" memory

model (i.e. the -P command line option is redundant). Code

generated by the "large" memory model will also be usable in

a 64K (PSEN+RD ANDed) system if linked in such a way that code

segments and data segments do not overlap.

The same library is used for both expanded models and is fully

reentrant and PROMable and is now called "cl8051.r03".  An exception

from the above is that "sprintf" currently cannot be called from more

than one place at a time (i.e. it is not interruptable).  This will

be changed in a future release of this package.  The "small" memory

model library is called "cl8051s.r03".   In analogy with the library

names, the linker command files has been renamed to "lnk8051.xlk" and

"lnk8051s.xlk" respectively.

2. PORT INPUT/OUTPUT

====================

The library routines "input" and "output" have been changed to

become fully PROMable and interruptable as well as being compatible

with the new "large" memory model.  The source code is also supplied

(in "input.s03" and "output.s03").

3. COMPILER UPGRADE

===================

If a statically allocated object is prefixed with "const" this object

will be put into the ROM section only (CONST segment).  This is

valuable when a lot of static data structures must be created as it

conserves RAM space.  This is an ANSI supported feature.

Example:

typedef struct

{

declarators.....

} s;

const s table[]=

{

initializers.....

};

Please note that the "const" attribute will only have this effect in

the "medium" and "small" memory models.

*************************************************************************

********** Modules compiled with the "large" and "medium"   *************

********** models CAN be mixed but that SHOULD BE AVOIDED!  *************

*************************************************************************

4. CUSTOMIZING THE LIBRARY

==========================

Since the "small" and expanded memory models are incompatible, the user-

configurable as well as user-written assembly language routines must

know what model to use.  In order to ease things a bit the routines

supplied in this package reference an include file "defmm.inc".  This

file is supposed to be created by copying "defsc.inc" (small) or

"defem.inc" (expanded models) to "defmm.inc".  Conditional assembly

does the rest.   Study "cstartup.s03" for more info.

The "printf" and "sprintf" routines share a common formatter

which is pre-installed in the library as module _frm_wri.  This module

contains a version of the formatter that does not support floating point

numbers.  It will complain about %e, %E or %f specifiers.  If you need

floating point formatting you should replace the original module with a

(much bulkier) version which resides on file "_frm_wri.r03".  The

command sequence is as follows:

xlib				% The library manager program

def-cpu 8051			% Must know the CPU

rep-mod _frm_wri cl8051		% This is the replacement

exit				% Return to system

Note that "printf" and "malloc" are not supplied in the "small" library

(cl8051s.r03) due to the rather limited use of such routines when only

120 or 248 bytes are available for stack and variables.

5. INTERRUPT ROUTINES

=====================

Note that the "small" memory model deviates from the current manual

in one important aspect.  Register R0 is used as parameter to C functions

rather than R3.  This makes the correct "small" interrupt handler look

like this:

handler:PUSH	ACC

PUSH	B

PUSH	PSW

PUSH	DPH

PUSH	DPL

PUSH	0

PUSH	1

PUSH	2

PUSH	3

PUSH	4

PUSH	5

PUSH	6

PUSH	7

MOV	R0,#0	; No parameters

LCALL	heater_overflow

POP	7

POP	6

POP	5

POP	4

POP	3

POP	2

POP	1

POP	0

POP	DPL

POP	DPH

POP	PSW

POP	B

POP	ACC

RETI

Note that the "small" memory model also needs saving of R6 and R7 while

the expanded models require that R6 and R7 are left 'untouched'.

6. MATHEMATICAL FUNCTIONS

=========================

The updated library now contains the most popular mathematical functions

including: atan, asin, acos, tan, cos, sin, sqrt, exp, pow, exp10, log,

and log10.  To ease the use of the mathematical functions a header file

"math.h" is also featured in the package.

Differences to previous versions

V1.04:

Single-chip (small) memory model and math support added.

A number of nasty bugs fixed.

V1.06:

Error in do-while (exp && exp) fixed.

Internal error in function call generator corrected.

NOTE: NEVER WORK ON THE DISTRIBUTION DISKETTES!

README.DOC