The IBM PC 6800/01/04/05/09/11 cross assemblers

GENERAL

The assemblers are named as*.exe where '*' is any of 0,

1, h1, 4, 5, 9, or 11 depending on which one you're

using.  Command line arguments specify the filenames to

assemble.

The assemblers accept options from the command line to be

included in the assembly.  These options are the

following:

l      enable output listing.

nol    disable output listing (default).

cre    generate cross reference table.

s      generate a symbol table.

c      enable cycle count.

noc    disable cycle count.

The command line looks like this :

as* file1 file2 ... [ - option1 option2 ...]

If this method of passing commands to the assembler is

used rather than the OPT pseudo op code, a space should

separate the minus sign from the last file name and the

first option.  Example:

as5 program - l cre

This command assembles file 'program' with an output

listing and a cross reference table.

The `S1' formatted object file is placed in file

`filename.S19' (m.out on older versions).  The listing

and error messages are written to the standard output.

If multiple files are assembled, the 'S1' file will be

placed under the first file's name.S19.

The listing file contains the address and bytes assembled

for each line of input followed by the original input

line (unchanged, but moved over to the right some).  If

an input line causes more than 6 bytes to be output (e.g.

a long FCC directive), additional bytes (up to 64) are

listed on succeding lines with no address preceding them.

Equates cause the value of the expression to replace the

address field in the listing.

Equates that have forward references cause Phasing Errors

in Pass 2.

Expressions may consist of symbols, constants or the

character '*' (denoting the current value of the program

counter) joined together by one of the operators: +-

*/%&|^.  The operators are the same as in C:

+      add

-      subtract

*      multiply

/      divide

%      remainder after division

&      bitwise and

|      bitwise or

^      bitwise exclusive-or

Expressions are evaluated left to right and there is no

provision for parenthesized expressions.  Arithmetic is

carried out in signed twos-complement integer precision

(16 bits on the IBM PC)

Constants are constructed with the same syntax as the

Motorola MDOS assembler:

'      followed by ASCII character

$      followed by hexadecimal constant

@      followed by octal constant

%      followed by binary constant

digit  decimal constant

ERRORS

Error diagnostics are placed in the listing file just

before the line containing the error.  Format of the

error line is:

File_name,Line_number: Error -> Description of error

or

File_name,Line_number: Warning --- Description of error

Errors of the first type in pass one cause cancellation

of pass two.  Warnings do not cause cancellation of pass

two but should cause you to wonder where they came from.

Error messages are meant to be self-explanatory.

Finally, some errors are classed as fatal and cause an

immediate termination of the assembly.  Generally these

errors occur when a temporary file cannot be created or

is lost during the assembly. Consult your local guru if

this happens.

DIFFERENCES

For indexed addressing, the comma is required before the

register; `inc x' and `inc ,x' are not the same.

Macros are not supported.  (try M4 or M6)

The force size operators ('>' and '<') are implemented

for all assemblers.

The only pseudo-ops supported are:

ORG, FCC, FDB, FCB, EQU, RMB, BSZ, ZMB, FILL,

PAGE, END, and OPT.

The OPT pseudo-op allows the following operands:

nol     Turn off output listing

l       Turn on output listing (default)

noc     Disable cycle counts in listing (default)

c       Enable cycle counts in listing (clear

total cycles)

contc   Re-enable cycle counts (don't clear total

cycles)

cre     Enable printing of a cross reference

table

s       generate a symbol table

Some of the more common pseudo-ops are not present:

SPC     Use blank lines instead

TTL     use `pr' to get headings and page numbers

NAM[E]  Did you ever use this one anyway?

The above pseudo-ops are recognized, but ignored.

ZMB (Zero Memory Bytes) is equivalent to BSZ (Block Store

Zeroes). FILL can be used to initialize memory to

something other than zero:

FILL val,nbytes.

TARGET MACHINE SPECIFICS

(as0) 6800:  Use for 6802 and 6808 too.

(as1) 6801:  You could use this one for the 6800 and avoid LSRD,

ASLD, PULX, ABX, PSHX, MUL, SUBD, ADDD, LDD and

STD.

6301:  Called ash1 (Hitachi 6801).  Same as 6801 except

cycle times are reduced and a few new instructions

are added: XGDX, SLP, AIM, OIM, EIM, TIM.

Convenience mnemonics BSET, BCLR, BTGL, BCHG and

BTST are also present which generate appropriate

mask values:

BSET 4,dog --> OIM #10,dog

Bit manipulation using indexed addressing is

specified with a trailing ",x": aim #mask,index,x

or btgl 5,index,x.

(as4) 6804:  The symbols 'a', 'x' and 'y' are predefined as $FF,

$80 and $81 respectively.  Also defined as 'A', 'X'

and 'Y'.  Because of the 6804 architecture, this

means that 'clr x' will work since the x register

is just a memory location. To use short-direct

addressing, the symbol involved must not be a

forward reference (i.e. undefined) and must be in

the range $80-$83.

Remember that bytes assembled in the range $10-$7F

will go into the data space; There is no program

space ROM for these locations.

The syntax for Register indirect addressing is as

follows:

mnemonic [<x>or<y>]

an example is:

lda [x]

The MVI instruction (move immediate) has its own

format :

mvi address,#data

where address is an 8-bit address in page zero,

and data is the value to be written to specified

location.

(as5) 6805:  There is no 'opt cmos' pseudo, so be careful not to

use STOP or WAIT in a program that is destined for

an NMOS version of the 6805.  The MUL instruction

should also be avoided on all versions of the 6805

except the C4.  Cycle times are for the NMOS

versions.

(as9) 6809:  The SETDP pseudo-op is not implemented.  Use the

'>' and '<' operators to force the size of

operands.

For compatibility, CPX is equal to CMPX.

(as11) 68HC11:  Bit manipulation operands are separated by

blanks instead of commas since the 'HC11 has bit

manipulation instructions that operate on indexed

addresses.

DETAILS

Symbol:  A string of characters with an initial non-

digit. The string of characters may be from the

set:

[a-z][A-Z]_.[0-9]$

( . and _ count as non-digits ).  The `$' counts as

a digit to avoid confusion with hexadecimal

constants.  All characters of a symbol are

significant, with upper and lower case characters

being distinct.  The maximum number of characters

in a symbol is currently set at 15.

The symbol table has room for at least 2000 symbols

of length 8 characters or less.

Label:  A symbol starting in the first column is a label

and may optionally be ended with a ':'.  A label

may appear on a line by itself and is then

interpreted as:

Label  EQU    *

Mnemonic:  A symbol preceded by at least one whitespace

character. Upper case characters in this field are

converted to lower case before being checked as a

legal mnemonic.  Thus `nop', `NOP' and even `NoP'

are recognized as the same mnemonic.

Note that register names that sometimes appear at

the end of a mnemonic (e.g. nega or stu) must not

be separated by any whitespace characters.  Thus

`clra' means clear accumulator A, but that `clr a'

means clear memory location `a'.

Operand:  Follows mnemonic, separated by at least one

whitespace character.  The contents of the operand

field is interpreted by each instruction.

Whitespace:  A blank or a tab

Comment:  Any text after all operands for a given

mnemonic have been processed or, a line beginning

with '*' or ';' up to the end of line or, an empty

line or, the remainder of a line after ';' (at

least one space or tab must precede the semicolon

if it is not the first character on the line)

FILES

filename.S19  S-record output file

STDOUT        listing and errors (use redirection for

listing file)

Fwd_refs      Temporary file for forward references.

IMPLEMENTATION NOTES

This is a classic 2-pass assembler.  Pass 1 establishes

the  symbol table and pass 2 generates the code.

12/11/84 E.J.Rupp

This version of the cross assemblers updated by Greg

Thoman in July, 1990 and includes the correction and

enhancements made by Bruce Olney.

Previous version of the cross assemblers ported to the

IBM PC on 4/13/87.

The "older versions" mentioned above were ported to the

IBM PC on 7/25/85.