SIM_R3.DOC

I. Introduction

A. Who wrote sim_r3 and why

B. Disclaimer

II. Using sim_r3.exe

A. Items on the display

B. Explanation of commands

III. Recompiling sim_r3

A. For the IBM-PC

1. Using Microsoft C 5.0 or higher

2. Using another compiler

B. For non-IBM compatible machines

I. Introduction

I. A. Who wrote sim_r3 and why

All files in this archive were written entirely by myself,

Joseph Schachner.  I can be contacted via Compuserve

(73637,3437).

/* I, Amir Bernstein, have made several improvements and fixed

minor bugs, I can be contacted on FREEWARE BBS */

sim_r3 was written to support a project where I work which

is planning to use a 68705R3.  Because the 68705 program and

data memory is compeletly internal, debugging a running 68705

is nearly impossible.  Of course, the 68705 can be emulated

by hardware, at a cost of several thousand dollars.  This is

an excellent debugging solution.  However, simulation with

software provides a very good debugging environment for

almost no cost.  sim_r3 is in use by another engineer at my

workplace, and has already found several bugs.

I. B. Disclaimer

Although I am an experienced 6800 family programmer, and I

took great care to insure the accuracy of this emulation, and

several bugs have been found and fixed as the program has

been used, I make no representation that this program in its

current form is either bug free or will precisely meet your

needs.  The source code is distributed in this archive for

you to add (or change) any feature.  If you change and

recompile sim_r3.exe, PLEASE do not distribute it without

noting your changes in this file!

Changes:

V1.21 - added clear of latched external interrupt to

simulated reset routine in instr.c.  This change is

present in the source code in this archive.  This was an

omission in 1.2 and earlier, but the only symptom is, if

you issue 'I' and then reset before simulating servicing

the interrupt the interrupt persists. 8-July-1989 Joe

Schachner

/*   V2.00 - The major changes are the symbol table support (5 chars/label)

and the requirement(!) for VGA in order to support 50 line mode,

the source code can always be changed to allow for different

numbers of line per screen.

Additional changes to files are noted:

INSTR.C  -  Add MUL (actually implements MUL).

Add STOP, WAIT (do not actually STOP or WAIT).

Update timings for HC05 devices.

Add symbol table reference by "linked list".

INSTR2.C -  Add symbol table reference to JMP, JSR, BRANCH

these do not appear as cleanly in "assignment"

column but it is clear enough.

Add STOP, WAIT (CCR modify).

Fix TAX,TXA by masking off the upper byte.

Fix ROR,ROL by masking off the upper byte.

V  I         SIM_MEM.C - sym_nmbr[], sym_tbl[] memory reference.

E  M                     load_sym() is identical to load() but it is very

R  P                     important(!!!) to specify the correct file with

Y  O                     the symbol table since otherwise the program will

R                     get lost (it searches for the \f charcater the

T                     assembler inserts in the output listing in

A                     the load_sym_data() routine).

N                     sym_writef_init() initialized the memory "image"

T                     sym_writef() actually sets the "linked list".

SIM_R3.C -  sym_nmbr[], sym_tbl[] memory allocation.

add 'y' condition for keyboard input.

*/

II. Using sim_r3.exe

Normally, one develops for a 6805 family processor using an

assembler and possibly a linker.  The output from the

development tools is a file containing Motorola hex format

records (S0, S1 and S9).

The simulator loads in the Motorola hex file in response to

the 'L' command (see below).  By default trace display and

log file capture are on.  The simulator responds to your

commands input in either upper or lower case.

Note: the simulator's trace log file is named "sim_r3.log"

and is created in the current directory when the simulator

is started.  If a file with the same name existed, it is

replaced by the new (empty) file.

sim_r3 is a very small program.  When compiled with Microsoft

C 5.0, it is about 55,000 bytes.  /* QuickC yeilds 45K */

Therefore, even 256K of memory is more than enough.  If you

plan to turn on the log file (to capture the trace on disk),

it helps to have a hard disk to keep execution speed up.

II. A. Items on the display

When sim_r3 is started, it puts up the following on the top

seven lines of the display:

68705R3 Simulator V1.2  6-July-1989              Disp+log ON    Halted

R - disp/chg regs       L file - load Sx & reset        E - toggle TIE  G - Go

T - trace on/off        B addr - Break at address (go)  I - /INT      S - Step

X - Reset       M addr - Mem view/mod   Z - Disassem    H - Halt      Q - Quit

/*                                                      Y - symbol table    */

______________________________________________________________________________

The top line (line 1) shows program status.  The next three

lines list commands, which will be explained below. The fifth

line is always blank.  The sixth line is where all commands

appear as you type them, and where prompts and messages

appear; this line will be referred to as the "prompt line".

The next line (line 7) is a solid bar separating the fixed

top portion of the screen from the trace display, which

scrolls up from line 24 to line 8 and vanishes.  Disassembly

also appears on lines 8 to 24.

The status line shows the program revision, whether trace

results will be displayed (and optionally captured to a

file), and whether simulation is in progress.  The next

section, "Explanation of commands", provides more information

on these fields.

II. B. Explanation of commands

R - display / change registers

Text similar to the following appears on the prompt line:

PC= 100, A= 00, X= 00, SP= 07F, CCR=08 (I)  Change?(reg) _

The user may enter (at least the first character of) the name

of any register shown, sim_r3 will prompt for a new value.

Press return without entering a register name to end this

command without changing a register.  Note: the values of the

registers are displayed and entered in hex.  Bits that are on

in the condition code register are also shown using the

characters HINZC.

L file - Load Sx and reset

sim_r3 reads motorola hex records.  Only S1 and S9 records

are significant.  Checksums are ignored.  Loading is

extremely fast.  When the "S9" record is encountered the load

terminates.  End of file without an "S9" record causes an

error message to be displayed.

If the load completes without error, a message similar to the

following is displayed:

File test.sx loaded

When a hex file has been loaded, a simulated reset is

automatically performed (see 'X' command), below.

E - Toggle TIE in

sim_r3 simulates all four timer modes.  The 'E' command

changes the state of the timer's external input.  The current

state is displayed on the prompt line.  The effect of

changing the state of the timer's external input depends on

the mode set by bits 4 and 5 (TIE and TIN) in the Timer

Control Register.  See section 5.3 of the MC68(7)05 R/U

Series technical data book, Motorola ADI-977.

G - Go

The 'Halted' label changes to 'Running' and simulation

commences at the current PC.  The breakpoint address is

unchanged.  Simulation will halt when the PC equals the

breakpoint address on an opcode fetch, or if the 'H' command

is entered (see below).

T - Trace display (and logfile) on/off

If display is on when this command is entered, it is turned

off.  Logfile capture is always off when display is off.  If

display is off when this command is entered, the prompt

"Enable logfile also?(Y/N) " will appear.  After the prompt

is answered, the label on line one will say either "Display

ON" or "Disp+Log ON".  The format of the log file is

identical to that of the trace display.

Note: simulation proceeds MUCH faster if the trace display is

turned off.

An example trace display (part of a log file) appears below.

It was generated by the commands 'L test.sx', 'B 10b'.

100  A6 55         LDA   #$55           101=>55 ACCUM<=55                     2

102  48            LSLA                 ACCUM=>55 ACCUM<=AA                   6

103  47            ASRA                 ACCUM=>AA ACCUM<=D5                  10

104  97            TAX                  X_REG<=D5                            12

105  BF 50         STX   $50            050<=D5                              17

107  3C 50         INC   $50            050=>D5 050<=D6                      23

109  26 FC         BNE   $107                                                27

107  3C 50         INC   $50            050=>D6 050<=D7                      33

109  26 FC         BNE   $107                                                37

^    ^             ^                     ^                                   ^

|    |             |                     addr=>data (read)                   |

|    |             |                     addr<=data (write)        Cycle count

|    |             |                     All values in hex.         in decimal

|    |             Assembly              Register names ACCUM,

|    |             Mnemonics             X_REG, SP may appear.

|    |

|    Opcode and operand bytes for this instruction, in hex.

|

PC for opcode, in hex

The example trace continues until finally location $50

becomes zero and the BNE is not taken:

109  26 FC         BNE   $107                                               437

107  3C 50         INC   $50            050=>FF 050<=00                     443

109  26 FC         BNE   $107                                               447

At this point the prompt line shows "Breakpoint reached" and

the simulation is halted.

B addr - Go and break at addr

The breakpoint address is set from the address (in hex)

following the 'B'. After that, operation is the same as 'G',

above.  See 'T', above,  for an example of this command's

usage.

I - /INT

Latches an external interupt.  If the interrupt mask is not

set sim_r3 will simulate interrupt response prior to

beginning the next simulated instruction.

S - Step

Simulate one instruction at the current PC.  Similar to 'G'

except simulation is automatically halted after one

instruction.  The breakpoint address is unchanged.

X - Reset

sim_r3 simulates a reset, loading the PC from the reset

vector, initializing the data direction registers and the

timer, and the timer control register from the Mask Options

Register, and setting the interrupt mask and resetting the

stack pointer to 7F, and (version 1.21 and higher) also

clears internal latched interrupt if any.

M addr - memory view/change

The address (in hex) is read as if read by a program running

in the 68705, and the address and value are displayed on the

prompt line.  If 'Enter' is pressed, the next location is

displayed.  If a hex value is entered, the value is written

to the displayed location and then the next location is

displayed.  To get out of this routine, enter a non-hex-digit

ascii character, for example 'q'.

Example: M 100

100 A6

101 55

102 48  q

Z - disassemble

Z displays assembly mnemonics for the next 15 instructions in

memory starting at the current program counter, or, if an

address is entered, for the next 15 instructions starting at

that address.  The display looks similar to a trace display

except no read/write data is shown and machine cycles are not

shown.  Disassembly has no effect on simulation state.

H - halt

If simulation is in progress, it is stopped.  The 'Running'

label on the first line changes to 'Halted'.

Q - quit

Leave sim_r3.  The following appears on the prompt line:

Logfile closed - normal exit

Y - load symbol table.  As stipulated above it is necessary to

specify the correct file with the symbol table (generated by

the FREEWARE assembler on the BBS) to prevent the program from

locking up.

Enter -

Just pressing 'Enter' does nothing but cause the line under

the prompt line to be redrawn.  This is useful in case you

accidentally press "Escape" and DOS blows away the line.

III. Recompiling sim_r3

III. A. For the IBM-PC

III. A. 1. Using Microsoft C 5.0 or higher

This supplied code may be compiled without changes.  The

supplied make file, makefile.mak, is for Polytron's POLYMAKE

to invoke the Microsoft C compiler.  The makefile is written

in a manner which makes it very easy to convert for

Microsoft's make, which is supplied with Microsoft C.  Only a

few lines need to be moved.  Exact direction are in a comment

in makefile.mak.

III. A. 2.Using another compiler

Most of this program is "plain vanilla" C, and will port

unchanged.  However, the functions which clear the screen and

position the text cursor are not portable.  clearscreen and

settextposition call routines from Microsoft C 5.0's

graphics library.  Most C compilers will supply library

routines for these functions, which may be substituted.

Delete_line uses the Microsoft C function "int86" to invoke

one of the DOS BIOS services.  Again, most C compiles for the

PC will have some way to invoke the BIOS services, which may

be substituted.

If your compiler does NOT have cursor position and screen

functions, you may use ANSI.SYS to supply these functions.

ANSI.SYS is a file supplied with IBM DOS.  It may be built

into the DOS of some clones.  ANSI.SYS must be memory

resident.  This requires the statement "DEVICE=ANSI.SYS" in

your config.sys file when you boot.  Then, simply writing a

special sequence of characters to stdout will cause ANSI.SYS

to perform these functions.  See the IBM DOS technical

reference for documentation on control codes for ANSI.SYS.

The code for using ANSI.SYS will look a lot like the code for

driving a VT-52 (a Digital Equipment Corp terminal), which is

included.  However, the control codes are not the same:

ANSI.SYS uses DEC VT-100 control codes.

sim.h includes a #define for the type of graphics routines,

at the moment either MSC or VT52.  I suggest you add another

#define and add your new C code right after each occurance of

"#ifdef VT52 ... #endif" in the routines settextposition and

clearscreen, both in sim_r3.c; and in the routine delete_line

in instr.c.

III. B. For non-IBM compatible machines

As long as your machine has an 80x25 character text display,

the only changes which need to be made are to support the

functions settextposition, clearscreen, and delete_line.

sim.h includes a #define for the type of graphics routines,

at the moment either MSC or VT52.  I suggest you add another

#define and add your new C code right after each occurance of

"#ifdef VT52 ... #endif" in the routines settextposition and

clearscreen, both in sim_r3.c; and in the routine delete_line

in instr.c.

Note that if your computer emulates a DEC VT 52 terminal, or

if you will use this on a computer that actually drives a

terminal that can handle VT 52 control codes, then all you

have to do is uncomment "#define VT52" instead of "#define

MSC" in sim.h and recompile.

It is interesting to note that exactly the same parts of the

program need to be changed for a non-IBM compatible as for a

true IBM compatible but not Microsoft C 5.0 or higher.  This

is because of the lack of standardization of even the most

rudimentary graphics functions; each compiler is different.

Perhaps X Windows will finally change this sad situation.

One further reflection on graphics standardization: I

mentioned above that sim_r3 was about 55,000 bytes when

compiled by Microsoft C 5.0.  When compiled by Mark Williams

C for the Atari ST it is only about 36,000 bytes.  This

difference is due mostly to the fact that support for the

needed graphics functions and VT52 emulation is part of the

ST's operating system, but requires routines from Microsoft C

5.0's graphics library in the IBM version.  Consider the

amount of memory wasted in a multitasking environment if each

application includes the graphics library and ten or twenty

applications are running...  This has nothing to do with

sim_r3, but it's one of my favorite subjects!  My apologies.

I hope sim_r3 is useful to you.

--- Joe Schachner 8-July-1989

/* Amir Bernstein 27-November-1991, Happy Thanksgiving Joe in case you

get this before Thanksgiving */