This is a non-invasive debugger.  When the entry point is called

all registers are asved into the internal memory image, and become the

defaults for the debugger.  The PC register in particular will

point directly after the call to the debugger start routine.

The constant 'rambase' must be defined at link time.  This constant

defines where the debugger will put its scratch ram.  All code is PIC

code so the debugger will run at ROM or RAM address.

This version has no support for 68020 code; although the disassembler

will recognize some 68020 addressing modes incorrectly.  A later

version may resolve the problems.

MAIN.SRC shows a sample startup sequence.  The code in this module

is sufficient on my target to get the debugger to run out of ROM.

However, this module may be left out and the debugger will initialize

itself when you call the dbg_start entry point.

IO.SRC has the two basic routines that have to be written to do I/O

to a terminal.  This is the only hardware-dependent portion of the

debugger.