To:     Users

From:	The Author

Subj:	Simulator Usage

Date:	15-Oct-94

This memorandum documents the PDP-8, PDP-11, and Nova simulators.  These

simulators are freeware; commercial use is prohibited, and support is

not available.  The best way to fix problems or add features is to read

and modify the sources yourself.  Alternately, you can send Internet mail

to dsmaint@pa.dec.com; but responses are NOT guaranteed.  [Note: the

Nova simulator is not included in this release.]

The simulators use a common command interface.  The memorandum first

describes the common features of the command interface and then provides

information on each of the individual simulators.

1. Compiling And Running A Simulator

The simulators have been tested on VAX/VMS, Alpha/VMS, and Alpha/OSF.

Porting to other environments will require changes to the operating

system dependent code in scp_tty.c.

To compile the simulators on Alpha/OSF, use these commands:

% cc pdp8*.c scp*.c -lm -o pdp8

% cc pdp11*.c scp*.c -o pdp11

% cc nova*.c scp*.c -lm -o nova

[Or use the makefile and issue the command 'make pdp8' to

build the pdp8 emulator, 'make pdp11' to build the pdp11

emulator, or simply 'make' to build both - mbg]

To start the simulator, simply type its name.  The simulator takes

one optional argument, a startup command file.  If specified, this

file should contain a series of non-interactive simulator commands,

one per line.  These command can be used to set up standard parameters,

for example, disk sizes.

% pdp8 <startup file>(cr)	or

% pdp11 <startup file>(cr)	or

% nova <startup file>(cr)

The simulator types out its name and version, executes the commands

in the startup file, if any, and then prompts for input with

sim>

2. Simulator Conventions

A simulator consists of a series of devices, the first of which is always

the CPU.  A device consists of named registers and one or more numbered

units.  Registers correspond to device state, units to device address

spaces.  Thus, the CPU device might have registers like PC, ION, etc,

and a unit corresponding to main memory; a disk device might have

registers like BUSY, DONE, etc, and units corresponding to individual

disk drives.  Except for main memory, device address spaces are simulated

as disk files in the host file system.  The SHOW CONFIGURATION command

displays the simulator configuration.

A simulator keeps time in terms of arbitrary units, usually one time unit

per instruction executed.  Simulated events (such as completion of I/O)

are scheduled at some number of time units in the future.  The simulator

executes synchronously, invoking event processors when simulated events

are scheduled to occur.  Even asynchronous events, like keyboard input,

are handled by polling at synchronous intervals.  The SHOW QUEUE command

displays the simulator event queue.

3. Commands

3.1 Loading Programs

The LOAD command (abbreviation LO) loads a file in binary paper-tape

loader format:

sim> load <filename>(cr)

3.2 Saving and Restoring State

The SAVE command (abbreviation SA) save the complete state of the

simulator to a file.  This includes the contents of main memory and

all registers, and the I/O connections of devices:

sim> save <filename>(cr)

The RESTORE command (abbreviation REST, alternately GET) restores a

previously saved simulator state:

sim> restore <filename>(cr)

Note: SAVE file format compresses zeroes to minimize file size.

3.3 Resetting Devices

The RESET command (abbreviation RE) resets a device or the entire

simulator to a predefined condition:

sim> RESET(cr)			-- reset all devices

sim> RESET ALL(cr)		-- reset all devices

sim> RESET <device>(cr)		-- reset specified device

Typically, RESET stops any in-progress I/O operation, clears any

interrupt request, and returns the device to a quiescent state.  It

does not clear main memory or affect I/O connections.

3.4 Connecting and Disconnecting Devices

Except for main memory, simulated unit address spaces are simulated as

disk files in the host file file system.  Before using a simulated unit

the user must specify the file to be used by a particular unit.  The

ATTACH (abbreviation AT) command associates a unit and a disk file:

sim> ATTACH <device><unit number> <filename>(cr)

If the file does not exist, it is created, and an appropriate message

is printed.

The DETACH (abbreviation DET) command breaks the association between a

unit and a file and closes the file:

sim> DETACH ALL(cr)			-- detach all units

sim> DETACH <device><unit number>(cr)	-- detach specified unit

The EXIT command performs an automatic DETACH ALL.

3.5 Examining and Changing State

There are four commands to examine and change state:

EXAMINE (abbreviated E) examines state

DEPOSIT (abbreviated D) changes state

IEXAMINE (interactive examine, abbreviated IE) examines state

and allows the user to interactively change it

IDEPOSIT (interactive deposit, abbreviated ID) allows the user

to interactively change state

All four commands take the form

command {-switches} {<device>{<unit number>}} <state list>

Deposit must also include a deposit value at the end of the command.

The examine and deposit commands can work on the address space of any

unit.  If no device is specified, the CPU (main memory) is selected; if

a device but no unit is specified, unit 0 of the specified device is

selected.  The "state list" consists of one or more of the following,

separated by commas:

register		the specified register

register1-register2	all the registers starting at register1

up to and including register2

address			the specified location

address1-address2	all locations starting at address1 up to

and including address2

STATE			all registers in the device

ALL			all locations in the unit

Examples:

sim> ex 1000-1100		-- examine 1100:1100

sim> de PC 1040			-- set PC to 1040

sim> ie 40-50			-- interactively examine 40:50

sim> ex rx0 50060		-- examine 50060, RX unit 0

sim> de all 0			-- set main memory to 0

Note: to terminate an interactive command, simply type a bad value (eg,

XXX) when input is requested.

3.6 Running Programs

The RUN command (abbreviated RU) resets all devices, deposits its argument

(if given) in the PC, and starts execution.  If no argument is given,

execution starts at the current PC.

The GO command does NOT reset devices, deposits its argument (if given) in

the PC, and starts execution.  If no argument is given, execution starts at

the current PC.

The CONT command (abbreviated CO) does NOT reset devices and resumes

execution at the current PC.

The STEP command (abbreviated S) resumes execution at the current PC for

the number of instructions given by its argument.  If no argument is

supplied, one instruction is executed.

The BOOT command (abbreviated BO) bootstraps the random access device

and unit given by its argument.  If no unit is supplied, unit 0 is

bootstrapped.  The specified unit must be attached to a disk file.

3.7 Stopping Programs

Programs run until the simulator detects an error or stop condition, or

until the user forces a stop condition.

3.7.1 Simulator Detected Stop Conditions

These simulator-detected conditions stop simulation:

o HALT instruction.  If a HALT instruction is decoded, simulation stops.

o Breakpoint.  The IBKPT register provides a single virtual address

breakpoint.  If the PC matches the contents of the IBKPT register,

simulation stops.  The breakpoint is automatically disabled for

the next instruction execution.

o I/O error.  If an I/O error occurs during simulation of an I/O operation,

and the device stop-on-I/O-error flag is set, simulation usually stops.

o Processor condition.  Certain processor conditions can stop simulation;

these are described with the individual simulators.

3.7.2 User Specified Stop Conditions

Typing the interrupt character stops simulation.  The interrupt character

is defined by the WRU (where are you) register and is initially set to

005 (^E).

3.8 Setting Device Parameters

The SET command (abbreviated SE) changes the status of a device parameter:

sim> SET <unit> <parameter>

The parameters are simulator and device specific.  Disk drives, for

example, can usually be set ONLINE or OFFLINE, write ENABLED or write

LOCKED; if a device supports multiple drive types, the SET command

can be used to specify the drive type.

3.8 Displaying Parameters and Status

The SHOW CONFIGURATION command shows the simulator configuration and the

status of all simulated I/O devices.

The SHOW <device> command shows the status of the named simulated I/O

device.

The SHOW QUEUE command shows the state of the simulator event queue.  Times

are in "simulation units", typically one unit per instruction execution,

relative to the current simulation time.

The SHOW TIME command shows the number of time units elapsed since

the last RUN command.

3.9 Exiting The Simulator

EXIT (synonym QUIT) returns control to the operating system.

5. PDP-11 Features

The PDP-11 simulator is configured as follows:

device	simulates

name(s)

CPU	J-11 CPU with 256KB of memory

-	FP-11 floating point unit (FPA)

PTR,PTP	PC-11 paper tape reader/punch

TTI,TTO	DL-11 console terminal

LPT	LP-11 line printer

CLK	line frequency clock

RK	RK11/RK05 cartridge disk controller with eight drives

RL	RLV12/RL01(2) cartridge disk controller with four drives

RX	RX-11/RX01 floppy disk controller with two drives

The PDP-11 simulator implements several unique stop conditions:

- abort during exception vector fetch, and register STOP_VEC is set

- abort during exception stack push, and register STOP_SPA is set

- trap condition 'n' occurs, and register STOP_TRAP<n> is set

- wait state entered, and no I/O operations outstanding

(ie, no interrupt can ever occur)

5.1 CPU

Memory size is a compile time parameter in pdp11_defs.h.  The standard

memory size is 248KB.

These switches are recognized when examining or depositing in CPU memory:

-v			interpret address as virtual

-d			if mem mgt enabled, force data space

-k			if mem mgt enabled, force kernel mode

-s			if mem mgt enabled, force supervisor mode

-u			if mem mgt enabled, force user mode

-p			if mem mgt enabled, force previous mode

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		16	program counter

R0..R5		16	R0..R5, first register set

R10..R15	16	R0..R5, second register set

KSP		16	kernel stack pointer

SSP		16	supervisor stack pointer

USP		16	user stack pointer

PSW		16	processor status word

CM		2	  current mode, PSW<15:14>

PM		2	  previous mode, PSW<13:12>

RS		2	  register set, PSW<11>

IPL		3	  interrupt priority level, PSW<7:5>

T		1	  trace bit, PSW<4>

N		1	  negative flag, PSW<3>

Z		1	  zero flag, PSW<2>

V		1	  overflow flag, PSW<1>

C		1	  carry flag, PSW<0>

SR		16	front panel switches

DR		16	front panel display

MEMERR		16	memory error register

CCR		16	cache control register

MAINT		16	maintenance register

HITMISS		16	hit/miss register

CPUERR		16	CPU error register

PIRQ		16	programmed interrupt requests

FAC0H..FAC5H	32	FAC0..FAC5, high 32 bits

FAC0L..FAC5L	32	FAC0..FAC5, low 32 bits

FPS		16	floating point status

FEA		16	floating exception address

FEC		4	floating exception code

MMR0..3		16	memory management registers 0..3

{K/S/U}{I/D}{PAR/PDR}{0..7}

16	memory management registers

INT		32	interrupt pending flags

TRAP		18	trap pending flags

WAIT		0	wait state flag

WAIT_ENABLE	0	wait state enable flag

STOP_TRAPS	18	stop on trap flags

STOP_VECA	1	stop on read abort in trap or interrupt

STOP_SPA	1	stop on stack push abort in trap or interrupt

OLDPC		16	PC prior to last JMP, JMS, or interrupt

BREAK		16	breakpoint address (1 to disable)

WRU		8	interrupt character

5.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

CSR		16	control/status register

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		32	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

5.3 Line Time Clock

The clock implements these registers:

name		size	comments

CSR		16	control/status register

INT		1	interrupt pending flag

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

TIME		24	time from I/O initiation to interrupt

5.4 RK11/RK05

RK11 options include the ability to place units online or offline,

write enabled or write locked:

sim> SET RKn OFFLINE(cr)	-- set unit n offline

sim> SET RKn ONLINE(cr)		-- set unit n online

sim> SET RKn LOCKED(cr)		-- write lock unit n

sim> SET RKn ENABLED(cr)	-- write enable unit n

The RK11 implements these registers:

name		size	comments

RKCS		16	control/status

RKDA		16	disk address

RKBA		16	memory address

RKWC		16	word count

RKDS		16	drive status

RKER		16	error status

INTQ		9	interrupt queue

DRVN		3	number of last selected drive

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

5.5 RX11/RX01

RX11 options include the ability to make units write enabled or write locked:

sim> SET RXn LOCKED(cr)		-- write lock unit n

sim> SET RXn ENABLED(cr)	-- write enable unit n

The RX11 implements these registers:

name		size	comments

RXCS		12	status

RXDB		8	data buffer

RXES		8	error status

RXERR		8	error code

RXTA		8	current track

RXSA		8	current sector

STAPTR		3	controller state

BUFPTR		3	buffer pointer

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

TR		1	transfer ready flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

DONE		1	device done flag (CSR<5>)

CTIME		24	command completion time

STIME		24	seek time, per track

XTIME		24	transfer ready delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

5.6 RL11(V12)/RL01,RL02

RL11 options include the ability to set units write enabled or write locked,

and to set the drive size to RL01, RL02, or autosize:

sim> SET RLn LOCKED(cr)		-- write lock unit n

sim> SET RLn ENABLED(cr)	-- write enable unit n

sim> SET RLn RL01(cr)		-- set size to RL01

sim> SET RLn RL02(cr)		-- set size to RL02

sim> SET RLn AUTOSIZE(cr)	-- set size based on

file size at attach

The size options can be used only when a unit is not attached to a file.

The RL11 implements these registers:

name		size	comments

RLCS		16	control/status

RLDA		16	disk address

RLBA		16	memory address

RLBAE		6	memory address extension (RLV12)

RLMP..RLMP2	16	multipurpose register queue

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop