PAGE 1

README.DOC

Introduction

This document describes the usage of the PIC Simulator software (PS.EXE)

developed by Central Services. The scope of the PS is to give a base help

during writing and debugging the PIC family 16C5x software, and to reduce

software development time.

PS was created to simplify first steps during software development: first

time program execution, signal acquisition, debug procedure and time

measurement. PS is NOT a real-time simulator that can substitute development

system (specially not PICMASTER In-Circuit Emulator system). It is a

user-friendly, easy to operate program dedicated to beginners that will

understand and learn to use PIC micro controllers, on one hand, and to

high-end software developers with an experience that will find a low-cost

tools for their job.

We hope that PS will become a part of your every-day utilities and that you

will enjoy it in the same way as its creators.

General description

Usually, development of a software dedicated for any type of micro

controller starts with development of a source code - file with a commands

of a micro controller you are writing for. Then, source code must be

compiled using some (dedicated) compiler, to produce machine code that

micro controller understand. Next step is to download the machine code

(object code) into ROM of a micro controller (internal or external) with

a hope that you didn't make a lot of mistakes (called bugs) and try it.

So far, ever everything passed easily. But, if your program has some bugs,

it is pretty complicated to find exactly where is the point where your

program is mistaking. This is rather easy to discover if you have an

In-Circuit Emulator system, or if you have a lot, lot of a experience.

But, sometimes it is not enough.

PS is a program interpreter that is using your source code to simulate

functioning of a PIC micro controller. It will show you step by step

program execution without touching your hardware prototype, it will warn

you for every mistake you made (watch dog time-out, for example), it will

show you how input and output pins are changing during program execution.

Statistics are showing that using PS, software development and debug

procedure can be reduced up to 70% for the users that doesn't have real

emulaton system, and 30% for those that has it.

User interface

PS has a friendly user interface: completely menu-driven, with almost all

command shortcuts (hot keys), and graphics user interface for a digital I/O

simulation. You don't need more then one day to learn complete usage of PS.

NOTE: You MUST have a VGA color monitor to be able to use PS.

PAGE 2

Restrictions

PS is a software simulation of a micro controller. This automatically means

that it is NOT a real time analyzer. It has NO hardware input/output ports

to be able to interface with your hardware. You will use it to find out

where your program is mistaking, to determine exact length of a signals

of your output pins, to establish the program execution time etc.

This version does NOT support any in-line input simulation - you must place

a breakpoints at any place where you want to change status of your input

pins. This version does NOT support RTCC input. Also, it is not possible

to change your source code directly in PS. Every mistake you find, you will

have to change directly in your source file, to recompile it and then to

reload into PS. Those features are planed for next versions of a PS.

Hardware requirements

To run PS you will need a 286 (at least) DOS computer with 640 KB of RAM,

Hard disk with 1 MB of free space, VGA color monitor and graphics adapter

and a mouse. It is recommended to use 486/SX/DX computer with fast hard

disc for optimum results.

Software requirements

PS is using LIST file produced with MPALC, PICALC or similar compiler.

So, you must pass your program (source code) through a compiler to produce

a LIST (.LST) file, and then to load it into PS. It is planned to expand PS

for a direct full screen editing and program changing.

You must have at least DOS 3.31 or higher to run PS. You can work without,

but we recommend to have mouse driver installed. We don't recommend to run

PS from the Windows or some other graphics application.

How it works?

Simple! You write your program, compile it, and when you are sure that there

is no syntax mistakes, load it into PS. Press ENTER to start the execution

and follow your program flow. Place a breakpoints where you want, capture

output status when you want, edit register values, add watch points and you

will discover how easy it is to write a assembler program using PS.

Installation and running

Place your install disk into floppy and type INSTALL. Installation program

will copy all necessary files into your hard disc, directly in directory you

choose.

After the installation you will have to make ANY "LST" file to be load in

PS (you can use PSDEMO.LST that will be installed during installation).

When you have your file ready, type:

PSDEMO

PAGE 3

Command Line Options

PS must be called from DOS prompt using following command

PS <list_file_name> [/S]

<list_file_name> is the name of the .LST file created after program

compiling.

[/S] is an optional parameter that will tell PS to load SET file

automatically on the start. See following chapters for SET file information.

List File Description

List file is a product of compiling. Compiler will load your source file

(usually .ASM), and produce .HEX (object code file, can be .OBJ), SYM file

(PS don't need it) and LST file. The last one is exactly your source file

with machine code instructions. This file will be used as source file for PS.

PS is created to understand and recognise structure of the list file

produced by PICALC compiler. We cannot take a responsibility in the case

that you are using some other PIC compiler.

How to write your software

We recommend to use custom names for file registers rather then register

numbers. For example, it is more easy to understand the program writing

following line:

KEY_TABLE       ADDWF     PC,Same            ; PC = PC + W

where KEY_TABLE is a label name, PC is a name for register 2 (program

counter), and Same is the value 1 that tells that W will be added to PC

and result will be placed in PC.

The same command can be written in this way:

LAB_101            ADDWF      2,1

In this way the result is the same, but it is not clear for anybody except

yourself what this command is doing.

Labels can be placed in the same line where commands are. They can be placed

before the row where command is also. But, as PS is reading ONLY the line

where command is, we recommend to place the labels in the same line where

command is. In this way it will be more easy to understand program flow.

Comments will not be loaded in PS. This means that PS will not see the

comments, and NOT that we say not to write them. Comments are very

important, and useful. Write as more as you can, because it is the best

documentation you can give with your software.

PAGE 4

Main Screen

PS Screen is devided in three important parts: Main menu, ROM (program)

window and RAM (memory) window. Main menu is used as access to program

functions. Each menu option name starts with a different letter. Pressing

ALT key with the starting letter of the option you need, the sub menu

dedicated for that option will be activated. To select other sub menues,

press cursor keys LEFT and RIGHT. Selection of the program function is done

by highlighting the option (LIGHT BAR) and pressing ENTER.

Almost all program functions have their shortcut keys: for example, when

you want to add or remove (toggle) a breakpoint, simply press CTRL + A.

ROM Program window is actually a list of commands you created writing a

source code. You will use it to control program flow.

RAM window is a list of file registers assigned in your source code. It is

important to add all file registers you want to control (watch points).

What to prepare before to run the program

You must do some preparing before to start to examine your program. See all

command description in the paragraphs that follows.

First, set the PIC type. Then, set the oscillator frequency - the real one

you will use. Enable or disable the watch dog exactly as it will be in your

real PIC application. Add all file registers you want to analyse. Set

breakpoints at the lines you consider important. Save your settings.

How program is executed

At the beginning, PS will load your program into computer memory, and

"reset" PIC. By setting the PIC type, PS will automatically decide the

value of the program counter (for PIC 16C54 is 1FF, for PIC 16C57 is 7FF).

All file registers will have their original values as they would have

inside the real chip: almost all value FF (except SWR...).

PS is presuming that all input pins have "pull up" resistors, so when you

set some port pin as input, it will have value 1.

Timer and Cycle counter will have value 0, and RTCC and WD counters will

have their default values. In this way, the same conditions are created as

they will be in the PIC after reset.

In the program window only the first row will have its command (usually it

is "GOTO <<somewhere>>") - it is the first command to be executed. You can

see program list using cursor keys. In this way you can point to the

commands you want to assign as breakpoints, start or stop capturing.

When you start the program (single step or run), PS will execute first

command and read next one. One by one, PS will read commands from the

"ROM" and execute them - exactly as it will be inside the real PIC.

Only difference is that it will not be in REAL TIME, and there are no

physical input - output pins to command. This action you must emulate.

When you start your program by pressing ENTER, PS will search the address

you assigned as "somewhere". In the case that it will find a non-existing

command at the address, it will rapport an error.

PAGE 5

Program window will change, and you will see light bar placed at the next

command to be executed. By pressing ENTER once again, you will execute that

command. Cycle counter will increment for exact number of program cycles as

it should be inside the PIC, and time counter will show you how much time

has passed.

Program can be executed also in automatic mode, where PS will execute

commands one by one, updating contents of the file registers, and showing

it in RAM window. This method is faster then single step, but you will not

be able to follow program flow. You can use this method to see if program

is "looping" without end somewhere - following program counter.

Execution of the program can be done very fast without displaying results

of the execution. This method shell be used when you will capture data for

later analyse.

Load a file

You can load a file into PS in two ways: by calling it from the DOS command

prompt as it was explained in the paragraph before, or after, from the

program using menu option File -> Load (pressing CTRL+L).

After the file is loaded PS will automatically reset the "micro controller".

Reset the processor

The option "Reset" is used to restart the processor. All file registers will

get their default reset values, and program counter will point to reset

vector (1FF for PIC 16C54, 7FF for PIC 16C57).

View ROM contents - program list

Once you load the program, the last memory location command will be

displayed in the first row of the program window. You can use cursor

keys to view program list: Up/Down to move one row up or down,

PageUp/PageDown to move one page up and down, or Home/End to go to the

top and the bottom of the program list. Moving the light bar you will NOT

change the contents of the program counter - you will only be able to view

the program lines not on the screen. How to change contents of the PC see

paragraphs below.

Find current command

Once you have moved light bar from the current PC (program counter)

location, you can return to it pressing SPACE bar or selecting

Run -> Current Command option from the main menu. In this way you will

NOT execute current command, but ONLY place the light bar on the current

command to be executed.

PAGE 6

View file register

Each file register you assigned in your source code can be shown on the

screen - its value in decimal and binary format. On other hand, you don't

need to place all file registers that you have created in your source code

in the RAM window - you should display only most important ones. RAM

window can keep up to 14 added file registers, apart from program counter

(PC) and I/O ports A,B and C. Those file registers are default and cannot

be removed.

From the main menu select Watch -> Add register or just press CTRL+F.

Enter register name exactly as you have name it in your source code. Note

that PS is case-sensitive program - you must type it as you did in your

source code.

File register will appear in the RAM window. To remove the file register,

do the same thing: press CTRL+F and type the name of the register. It will

be removed from the RAM window.

Edit file register

This function will allow you to change the contents of the file register.

Usually you will place a breakpoint on the line where you need to change

the value of the file register, and during the execution program will stop

at the breakpoint. You can change the value entering in binary or decimal

format.

Select Watch -> Edit file register from the main menu, or press CTRL+E as

a shortcut. Enter the name of the register, and then change the value.

Execute the program

Program can be executed in three different modes: single step, execution

with screen supervise and totally automatic without possibility to see

exact program execution.

Single step execution - this mode is used to trace program flow and/or to

change contents of the program counter. One command will be executed by

pressing ENTER key - the command where light bar is placed. In this way

you can see exactly flow of the program, and all file register changes

during each step. Use this option when you need to force start of the

program from the address (command) that is not a logically next program

command - in this way you will change contents of the program counter.

Pressing ENTER PS will read current command address and change contents

of the program counter. This mode does NOT recognise breakpoints.

Running the program with watch points - This mode is more fast then single

step, because PS will start from the CURRENT PROGRAM COUNTER address and

execute every next command automatically, updating contents of the file

registers on the screen. In this way you can see how values are changed.

Use this method when you need to see if the program is "looping" without

exit somewhere. PS will stop the execution after key press, or on

breakpoint, if the option IGNORE BREAKPOINTS is not activated. To run the

program in this way press CTRL+G or select Run -> Go from the main menu.

PAGE 6

Running and ignoring the display - This mode is fastest mode to execute

the program. Use it when you want to capture I/O output, or when you place

breakpoints to arrive to next breakpoint. You will NOT see the program flow

or display update. After a key press or on breakpoint PS will stop the

program execution and update RAM window. To start the program in this way,

press CTRL+Y or select Run -> Ignore display and Go from the main menu.

Trace the program

PS gives a possibility to make a program trace - flow of the command

execution. You can make a trace of the whole program, or only of a routines

you need to test. To do this, first place the light bar on the first command

of the routine you want to supervise. Then, press ENTER to change the PC

value. Next, place light bar on the last command you want to execute, and

there place a breakpoint. Press SPACE to return to origin command and from

there you can continue in Single Step mode or with "GO". Program will

stop automatically on breakpoint. See next paragraph for more details.

Breakpoints

Breakpoints are ROM memory locations where PS will stop the program

execution WITHOUT EXECUTING THE COMMAND signed with a breakpoint.

Breakpoints are very useful when you want to control the program flow, or

when you need to capture I/O ports. To place the breakpoint, press CTRL+A

or select Breakpoint -> Toggle breakpoint from the main menu. The command

currently selected with the light bar will be selected as breakpoint

command, and a triangle will appear on the right side of the program window.

Use the same key (option) to remove current breakpoint.

Pressing CTRL+I you will activate option Breakpoint -> Ignore breakpoints.

In this way program will execute without stopping on breakpoint.

To delete all breakpoints, press CTRL+D, or select Breakpoint -> Delete

breakpoints from the main menu.

NOTE: breakpoint positions will be saved during SAVE CONFIGURATION function.

Timer and Local timer

PS is equipped with a program execution counter that is counting exact

number of program cycles. From this value PS is calculating program

execution time using the value of the crystal. To clear the timer press

CTRL+T or select Settings -> Timer Reset from the main menu.

Local timer is another counter, that can be started at any point of the

program and is used to calculate the time of the execution of the small

routines. Local timer can count up to 32767 uS. To start local timer,

press CTRL+H or select Analyse -> Start local timer from the main menu.

To stop local timer, press CTRL+N or select Analyse -> Stop local timer

from the main menu. Local timer will be automatically reseted by restarting

it.

PAGE 7

Oscillator frequency

PS needs this value (given in Hz) to be able to calculate exact time elapsed

from the program start. Use this option to "set" oscillator frequency.

Calling from the main menu Settings -> Oscillator Frequency following you

can change default value (4000000 Hz).

Allowed values are from 10 to 99999999 Hz (apart that PIC can support

crystals up to 25 MHz).

Watch dog timer

PS has build-in watch dog timer as real micro controller has. When you

start PS first time, watch dog timer is disabled. To enable it, press

CTRL+W of select Settings ->> Watch Dog option. From that moment, PS will

update watch dog timer as it will happen in the real micro controller.

In the case that in your software you didn't enable watch dog, PS will

give you a warning message. During program execution, when WD time out

appears, PS will give you a message that WD is out of time, and it will

reset the processor. This is very useful to make a debug of your source

code. On SLEEP command, PS will go into "SLEEP" mode, and restart from

the beginning execution without resetting time counter.

Analyse

One of the most powerful features of the PS is possibility to analyse

program flow and I/O port status. This is important for all applications

where exact timing must be respected on output ports (serial communication,

for example).

During program execution, you can capture one port at the time (port A, B

or C in the case of 16C55/57).

NOTE: Capturing time cannot be longer than 32000 program cycles.

Analyse target

Before to start to capture data, you must configure which port to capture.

Default port is port B (register F6). Select option Analyse -> Target from

the main menu or press CTRL+O.

Select port to be captured, and press ENTER.

Start and Stop the capture

Start and Stop capture are two visible flags that will appear in the row

where you want to start or to stop to capture data. Imagine that you have

a routine that must send bits on one output port (Port B), using bit 0

(PB.0) pin as output. Your routine starts on label SERIAL_TX and finishes

with RETLW xxx where you return to the main routine. Locate your SERIAL_TX

routine moving light bar in the program window. Then select Analyse ->

Start analyse from the main menu or press CTRL+Z. "OK" sign will appear at

the right side of the program window. You have just selected starting point

of the capture option. From that command, PS will follow changing of the

port B until it fills all capturing buffer (32000 program cycles), or until

it finds STOP CAPTURE flag.

PAGE 8

Stop capture flag is "ASTERSC" sign. Find your "RETLW xxx" command at the

end of the SERIAL_TX routine and press CTRL+X or select Analyse -> End

analyse from the main menu. We recommend to put a breakpoint also where

you place stop capture sign, and in that way you can see immediately result

of the capture.

Now, return to your START CAPTURE sign moving the light bar, and press

ENTER to make a single step program running. You will see a "CAPTURING"

message at the bottom left corner of the screen. This is showing you that

program is capturing data. Now, you can continue in the single step mode,

or in automatic mode by pressing CTRL+Y. When PS finds Stop capture flag,

it will close the capturing process and if you placed a breakpoint, it will

stop the program execution also.

You have captured your data on currently active port.

View analyse

To see what you captured, press CTRL+V or select Analyse -> View from the

main menu. You DON'T need to stop the analyse to be able to see what you

have captured. You can select VIEW at any point of the program.

We recommend to use mouse for VIEW option.

At the centar of the screen you have working area where result of the

capturing is displayed.

Below you have 6 push bottons reserved for various propose: to move the

window left-right, make a zoom in-out, to select active bits and to return

to the main menu.

Set active bits

At the beginning, PS has captured all bits of a selected port. But, as

capturing time can be very long (up to 32000 program cycles), a lot of

capturing results has been captured. To make your view process more fast,

you can select only bits you need to see, and to make refresh of the

display more fast.

Press SET button or from the keyboard CTRL+S.

Make a click inside the rectangle you want to make active, or press on the

keyboard numbers 0 through 7 to select bits you want to see.

Press ENTER or select OK when finished. Only bits you selected will appear

inside working window.

Time measurement

You can see exact time that has passed from the one point you select up to

next point. At the left side of the working window you can see two red

cursors. Pick one of them and move it to the right. Then, pick up other

one and move it to the point you want to select as starting point. Pick

again first one and move it to the ending point. Below the working window

you will see how much time has been passed between two selected points.

You can move cursor lines with a keyboard. Use cursor keys (LEFT/RIGHT)

to move one of the cursors, and CTRL+(LEFT/RIGHT) to move other one.

PAGE 9

Zoom

Zoom is used to show part of the working screen between cursors at the

complete working screen. Using zoom you can see better small "spikes"

between two cursor points. There are 128 levels of the zoom.

Zoom is also useful to be sure that time between two cursor lines is

exactly the one you wanted: in the case when you capture large number of

program cycles (20000), you will not be able to see if your signal was

10 or 20 microseconds. Zooming it, you can see only one strobe you need.

Zoom in can be done with ENTER key, and zoom out with CTRL+ENTER.

It is more easy to use push bottons Zoom + and Zoom -.

Move the window

Once you made a Zoom, only part of the signal you placed between two cursors

can be seen in the working window. You can move (shift) your working screen

left and right to see other parts of the screen. With a keyboard, use

Page Up / Page Down keys. The numbers in the lower left and right corners

of the working screen will tell you which part of the window you are

examine.

Exit the analyse

To exit the analyse mode, press ESC or "RETURN" push-button. You can go as

many times as you like from main screen to view screen and back.

Delete analyse

Delete analyse is used to remove start and stop analyse flags from the

program list and erase contents of the analyse (capture) buffer.

You don't need to delete analyse when you misplace your start or stop

analyse flag. Just place cursor at new command where you want to place

flags and press buttons for start/stop. Flags will be removed from

previous position and placed to new one.