PWM.DOC

PWM Motor Control

High power DC motors (>12V, >2A) has usually very high start peak current

(8-25 A). This can (and it will) destroy any type of motor controller if

the energy will be gieven immidiately. Modern solution to this problem is

PWM motor control (Pulse Width Modulation) - contionus motor supply with

DC current that changes polarity very fast 10-200 KHz. In this way motor

is "stopped" when duty cycle of the controller is 50%, and is running

when duty cycle is almost 100% (1% or 99%). Controller for PWM is usually

MOS-FET bridge: 2 P channel and 2 N channel MOS-FET transistors with

2-10 A sink current. Motor is connected in the middle of the bridge

and it is using always only two transistors as power supply, while other

two are blocked by harware driver logic.

PIC 16C64, 16C74 and 17C42 have already implemented PWM output, so they

can be used directly without any software development to drive DC motor.

This document is describing low level of PWM motor controller with PIC

16C56:

Low level becouse it has been used PIC 16C56 on 20 MHz and maximum PWM

frequency is arround 20 KHz. Acceleration for start and stop is arround

100 mS and duty cycle has 24 passes to arrive to 2:98 duty cycle (2% : 98%).

Low level becouse it is not possible to reprogram acceleration time or

different PWM frequency - you can change it only by changing oscillator

frequency.

Positive part of this project is that we used low cost device (16C56-HS),

and with only two input pins (STOP and DIRECTON) you can control high

power DC motor.

We tested this project with a motor that burned out MOS FET bridge builded

with 50 A transistors in conventional power control, using 5 A MOS FET with

PWM.

If you have our PS (PIC SIMULATOR), you can load LST file and test PWM.ASM

directly to see how PWM output is changing when motor is accelerating (and

if you don't have, do you thing that it is the right time to order it?).

Scope of this project is NOT to give detailed schematic diagram of MOS FET

bridge - anyone will create it for its needs. Important is to show that

with a little software support is possible to have good PWM controller.

If you will have any questions about this (or some other) projects, please

contact us.

We hope that you will find this project usefull.

Sam Lowry

You can find me on (+39)-336-40-80-90 (voice) or contact us:

CS (Gordan Rancic)

Via Circonvallazione 11

20060 MASATE (MI)

ITALY