Precision Timing Loops for PIC MicroControllers

Copyright (c) 1993, Parallax Inc.

INTRODUCTION

Due to the RISC nature of the PIC, the application domain is almost

exclusively that of real time control. With this in mind, it is easy

to see how important it is for a programmer to generate precise

delays between events. Unfortunately, the PIC's relativly simple

hardware is only marginally helpful in helping us generate precise

timing intervals. The are two major methods for generating precision

intervals.

USING THE RTCC FOR PRECISION TIMING

The natural inclination of the programmer is to use hardware for

timing purposes, and often this is a good selection. In the PIC,

however, this is usually a poor option. Since an external timer would

defeat the PIC's attraction as a single chip solution (not to mention

using precision I/O pins), such second chip solution are generally

out of the question. Thus the programmer turns to the internal timer

of the PIC, the Real Time Clock/Counter or RTCC.

The RTCC is an eight bit counter which increments on either an

external signal (via the RTCC input pin) or the internal clock

(incremented once every 4 clocks which is equivalent to once every

instruction). This count can be further extended by using the

prescaler. The prescaler is a binary divider which can be programmed

in multiples of 2 from 2 to 256. Thus, the effective counting range can

be extended to 65536. Sounds like all the precision you could need,

right?

Unfortunately, three major problems exist with the RTCC. First, since

no overflow flag or interrupt is available when the RTCC rolls over,

the PIC must wait by watching the actual value of the register. While

this only sometimes creates problems, it is more a nuisance than a

prohibition.

Secondly, the tighest timing loop that can be done to watch for rollover

might appear as follows :

jnb	RTCC.7,$	; 2/3	Wait for MSB = 1

jb	RTCC.7,$	; 2/3	Wait for Rollover

;	Execution Resumes Here on Rollover

The second statement of this pair waits for the actual rollover, with

execution resuming after it occurs. Note, however, that for every

interation that rollover doesn't occurs, the branch is executed. Thus

the total cycle time from check to check is 3 cycles/12 clocks. Since the

timeout could occur the cycle after the check is done, it possible

for up to a 2 cycle/8 clock latency to develop. This random latency,

plus the 2 cycles used when the rollover is detected tranlates to a

latency anywhere from 2 cycles/8 clock to 4 cycles/16 clocks. This 2

cycle variance can represent a large error when no prescaler is used

(0.8% for a full RTCC 256 cycle count - even greater for lesser

delays)!!! Obviously, this problem can be somewhat alleviated using

the prescaler. While this will reduce the percentage error by

expanding the operating range, the 2 cycle variance will always

exist. Ultimatly, any timing system which uses busy waiting to

sychronize with hardware will NEVER be more accurate that the

minimum time between signal checks.

The third potential problem with the RTCC is granularity. While it is

true that the RTCC has excellect granularity without the prescaler

(1 uSec/Count @ 4 Mhz), it is also extremely limited in range (upto

256 uSec). This can be extended by software with only minor problem,

but is most often expanded with the prescaler. While the range can be

increased significantly, the minimum change in time that can be measured

(or granularity) also increases significantly. For example, a PIC

running at 4 Mhz and using the prescaler as divide by 256 can time

delays up to 65.5 mSec, but only in 256 uSec increments. Thus,

someone wishing to timeut 30 mSec exactly must settle for either

29.95 uSec (count = 139) or 30.2 uSec (count = 138). While these

small errors might be acceptable for some applications, they are often

unacceptable for the time critical nature of many embedded systems.

Before preceeding to the discussion of software timing loops, it

should be noted that the RTCC does have some practical application as

a timer. Most notably, system designers are always attempting to get

the most out of hardware. This often requires the controller to

handle multiple tasks. Unfortunately, the PIC is rather poor at

handling too many things at once. The solution to handling multiple

asynchronous timed events is to allow the RTCC to time the slowest

(and hopefully least critical) event. In such a scenario, the latency

and granularity problems of the RTCC can be eclipsed by the delay

generated by the necessity of handling more critical tasks first.

If such delay won't affect the correctness of your design, the RTCC

is actually an excellent timing choice and can even simply the

software a great deal.

USING SOFTWARE LOOPS FOR PRECISION TIMING

For precision timing loops on the PIC, nothing beats the good ole'

software busy loop. While these loops are often impractical in other

environments due to various uncontrollable factors (such as

interrupts and unpredicitable bus wait states for DMA or dynamic

refresh), the PIC suffers from none of these hidden timing monsters.

Provided you write and document your code carefully, the execution

time of a particular section of code is always determinant. In this

section will be shown the most common types of delays and method for

calculating precisely how many cycles each delays.

Notes on Loops

--------------

All calculations are done in system cycles. Each system (or

instruction) cycle takes 4 clocks to execute. Thus, to determine the

time delayed, use the relationship

Cycles * 4

Time (Sec) = ----------------

Clock Speed (hz)

Conversely,

Time (Sec) * Clock Speed (hz)

Cycles = -----------------------------

4

Also note that each instruction in a loop is commented with its

execution time. This allows us to document and easily calculate our

results. In the event that an instruction can optionally branch, the

non branch time is listed first and separated from the branching time

by a '/'.

Some higher level math functions are used in some of the computations.

While they are simple and commonly used, the are explained here for

clarity.

Floor(x) evaluates to the nearest integer less than x.

Floor (1.9999) = 1

Floor (0.0001) = 0

Floor (-0.999) = -1

Simple Single Loop Delay

------------------------

The most common delay is the single loop. It takes the form :

mov	R0,#count	; 2	Load Register R0 with Delay

djnz	R0,$		; 2/3	Loop Here Until Delay Done

The delay on this is calculated as follows :

Cycles = 2 + 3 * count - 1

= 3 * count - 1

The first lines shows each component of delay. The 2 is the time to

load the delay constant. The 3 * count represents the number of 3

cycles branches which must be executed to decrement delay to 0. And

the -1 compensates for the fact that the last loop does not branch.

The second line is merely an algrebraic simplification. Considering

that delay has an effective range of 1 to 256 (delay = 0), the single

loop can provide delays from 2 to 767 cycles in 3 cycle increments.

This 3 cycle granularity can be eliminated by padding the code with

additional instructions. If no useful work can be done with these

instructions, consider the following cycle wasters :

nop			; 1	Waste 1 Cycle, Use 1 Word

jmp	$+1		; 2	Waste 2 Cycles, Use 1 Word

Including padding to eliminate granularity, the whole cycle

calculation is :

Cycles = 3 * count + padding - 1

The big question is how to calculate count from the desired delay in

cycles. A little algebra reveals that :

Cycles - padding + 1

count = --------------------

3

This creates a small problem since our unknown (count) is defined in

terms of another unknown (padding). This can be solved, however, by

recognizing that padding ranges from 0 to 2. Thus, we can relate

count and padding to Cycles via :

/ Cycles + 1 \

count = Floor |  ---------- |

\      3     /

padding = Cycles - 3 * count + 1

Extended Single Loop Delay

--------------------------

The natural way to extended the delay of the single loop is to merely

make the body of the loop take more time to execute. For example,

mov	R0,#count	; 2	Load Register R0 with Delay

:loop	nop			; 1	Waste Addition Cycle in Loop

djnz	R0,:loop	; 2/3	Loop Here Until Delay Done

This has the effect of increasing the loop time from roughly 3 cycles

to 4 cycles per loop. All the calculations from the previous section

still apply, but a 4 must be used in place of a 3. In general, for a

delay body where N cycles is executed for each count, the following

hold true :

Cycles = N * count + padding - 1

/ Cycles + N - 2 \

count = Floor |  -------------- |

\        N       /

padding = Cycles - N * count + 1

Notice that for any given N, up to N-1 padding cycles may be needed

for precise timing. This is granularity rearing its ugly head

once again. For small values of N or well chosen combinations of N

and desired delay, padding can be minimal or even non existant. In

other situations, a nested loop is often a better choice for delays.

Simple Nested Loop

------------------

Simple nested loop delays take the form :

mov	R0,#hi		; 2	Set MSB of Delay Count

mov	R1,#lo		; 2	Set LSB of Delay Count

:loop	djnz	R1,:loop	; 2/3	Inner Loop

djnz	R0,:loop	; 2/3	Outer Loop

Before calculating the delay, one potential hazards should be noted.

R1 is assigned the value #lo for the first loop only. After that, the

first DJNZ assures that all future values of R1 are 0, which is 256

loops. Keeping this in mind, the delay is :

Cycles = 4				; Setup

+ 3 * lo - 1			; One Pass R0 = lo

+ (767 + 3) * (hi - 1)		; hi-1 Passes R0 = 256

+ 2				; Failure for --R1 = 0

Cycles = 770 * hi + 3 * lo - 765

The 3 * lo term leading us to suspect that there is a granularity of

3 clocks. There may, however, be a larger "grain" which exists

between the minimum step size of hi and the maximum step size of lo.

By checking, however, we find that 256 * 3 (maximum lo step size) is

768, within 2 of 770 (minimum hi step size). Thus the granularity is

3, assuring that for padding from 0 to 2 :

Cycles = 770 * hi + 3 * lo + padding - 765

can cover any delay from 8 to 197,128 cycles.

Now, of course, we must determine hi, lo, and padding for a given

delay. Thus :

/ Cycles + 765 \

hi = Floor | ------------ |

\      770     /

/ Cycles - 770 * hi + 765 \

lo = Floor | ----------------------- |

\            3            /

padding = Cycles - 770 * hi - 3 * lo + 765

Extended Nested Loop

--------------------

Just as we extended the delay for the single loop by bulking up the

body, the nested loop can be extended. Using the form

mov	R0,#hi		; 2	Set MSB of Delay Count

mov	R1,#lo		; 2	Set LSB of Delay Count

:loop	nops			; M	Wasted Cycles

djnz	R1,:loop	; 2/3	Inner Loop

djnz	R0,:loop	; 2/3	Outer Loop

we'll let N = M + 3 be our basis for computation. Thus,

Cycles = 4				; Setup

+ N * lo - 1			; One Pass R0 = lo

+ (256 * N + 3) * (hi - 1)	; hi-1 Passes R0 = 256

+ 2				; Failure for --R1 = 0

Cycles = (256 * N + 3) * hi		; MSB Contribution

+ N * lo				; LSB Contribution

- (256 * N + 8)			; Overhead & Offsets

+ padding			; Padding (0..N-1)

/ Cycles - N + (256 * N + 8) \

hi = Floor | -------------------------- |

\        256 * N + 3         /

/ Cycles - (256 * N + 3) * hi + (256 * N + 8) \

lo = Floor |-------------------------------------------- |

\                      N                      /

padding = Cycles - (256 * N + 3) * hi - N * lo + (256 * N + 8)

Larger Nested Loop

------------------

While it would be possible to continue this nesting of loops ad

infinitum, it would be ridiculous. Few, if any, real time

applications need a 3-deep timing loop capable of up to 49 seconds of

delay on a 4 Mhz PIC. If you need this, just continue the analysis as

done for the first two loop types. Anyone developing the general

solution for a 3-deep loop should post it on this system (if only

just for kicks).

Using Precision Delays

----------------------

When attempting to generate precision delays, keep a few facts in

mind. First, timing loops aren't the only things which use CPU time.

Every instruction takes time. For example, if you're using the PIC to

generate some periodic wave form, don't forget to include both the

timing delay and the wave form calculation in determining the

expected frequency of the output.

Another thing to keep in mind is that PIC instructions output near

the end of the 4 clock cycle but sample input near the beginning of a

cycle. This problems can be further amplified by some of the Parallax

instructions (such as MOVB) where the I/O occurs mid instruction or,

worse yet, at some indeterminant point in the instruction. It is

therefore sometimes necessary to make an adjustment to timing loops

which relate input to output or vice versa.

EXAMPLES

Example 1

---------

Suppose you need a 50 uSec delay on a 4 Mhz PIC. First, you determine

the number of cycles that must be delayed by :

0.000050 Sec * 4000000 hz

50 Cycles = -------------------------

4

Since a simple loop will delay from 4 to 769 cycles, we'll use it. We

need to calculate the count and padding for 50 cycles :

/ 50 + 1 \

17 = Floor | ------ |

\    3   /

0 = 50 - 3 * 17 + 1

With the count (17) and the padding (0) computed, we only need to write

our code :

mov	R0,#17	; 2	Setup Delay

djnz	R0,$	; 2/3	Delay

This routine will use one byte of register (here labeled R0) and will

delay exactly 50uSec on a 4Mhz PIC.

Example 2

---------

A more practical example is an attempt to generate a 32,768hz square

wave on a PIC pin. Each half wave of this train would last 15.26

uSec. The basic program flow for this would be as follows :

loop	setb	PIN	; 1	Output High	High Cycle = 15.26 uSec

;	Delay

clrb	PIN	; 1	Output Low	Low Cycle  = 15.26 uSec

;	Delay

jmp	loop	; 2	Continue

At 8Mhz, 15.26 uSec is 30.52 clocks. It would be tempting to round

such a figure to 30 or 31. It should be noted that if we are willing

to tolerate a slight asymmetry to the output, having one half wave of

30 and the other of 31 would keep us vary near the target frequency

of 32.768 Khz.

Using the 30/31 cycle delays and noting the skelton shown above,

we'll make the high cycle delay 30 and the low cycle delay 31. Since

a cycle must also include the other instructions (like setb, clrb,

and jmp), the actual delays needed are 29 and 28, respectively. Our

new skeleton looks like :

loop	setb	PIN	; 1	Output High	30 Cycles

; 29	Delay

clrb	PIN	; 1	Output Low	31 Cycles

; 28	Delay

jmp	loop	; 2	Continue

Now compute the count and padding for each of the single loop delays

as before :

/ 29 + 1 \

10 = Floor | ------ |		29 Cycle Delay

\    3   /

0 = 29 - 3 * 10 + 1

/ 28 + 1 \

9 = Floor | ------ |		29 Cycle Delay

\    3   /

2 = 28 - 3 * 9 + 1

With the count and padding now computed, we may complete the skeleton

for our program :

loop	setb	PIN	; 1	Output High	30 Cycles

mov	R0,#10	; 2	Setup Delay

djnz	R0,$	; 2/3

clrb	PIN	; 1	Output Low	31 Cycles

mov	R0,#10	; 2	Setup Delay

djnz	R0,$	; 2/3

jmp	$+1	; 2	Padding for Delay

jmp	loop	; 2	Continue

This program should approximate a 32768hz square wave on one PIC pin.

In fact, the actual frequency is 32258hz and the duty cycle is 49.1%

(not quite square). For more accuracy, either use a higher frequency

(to decrease granularity) or a clock frequency which is some even

multiple of the target frequency.